Beelink GTR9 Pro

UEFI BIOS Setup Reference

Comprehensive Configuration Settings Catalog

AMD Strix Halo  ·  Ryzen™ AI Max+ Platform

AMI Aptio V Firmware

Image & Extraction Summary

| Firmware image | GTRPR05.rom (33,554,432 bytes / 32 MB) |
| --- | --- |
| Flash package | GTRPR05_WinFlash (AMI AFUWIN / AfuEfi) |
| BIOS vendor | American Megatrends International (AMI Aptio V) |
| SoC platform | AMD Strix Halo (Ryzen AI Max series APU) |
| Setup form-sets | 7 (Aptio Setup, AMD CBS, AMD PBS, AMD Overclocking, AMD PMF, DASH/ASF, RAIDXpert2) |
| Total forms / pages | 186 |
| Configurable settings | 1,045 |
| BIOS Version / Build Date | Runtime-populated (shown on Main page at boot) |

# Contents

# About This Document

This catalog enumerates every configuration option exposed by the Beelink GTR9 Pro UEFI firmware, extracted directly from the BIOS image. Settings are decoded from the firmware’s Internal Forms Representation (IFR) and presented in the exact order they appear on the BIOS Setup screens, organized by the firmware form-set that owns each menu.

## How the document is organized

Each chapter corresponds to one firmware form-set. Within a chapter, every form (a single BIOS Setup page) appears as a sub-section. Read-only information fields and links to child pages are summarized at the top of each form; the configurable options follow in a table.

## Reading the settings tables

- Setting — the option label as shown on screen.

- Type — Selection (one-of list), Numeric (value within a range), Checkbox, Text, Password, or Ordered List.

- Options / Values / Range — for selections, each choice with its stored value in [brackets]; for numerics, the Min/Max/Step and default. The factory default is marked ◀ default.

- ◀ performance — a recommended setting for maximizing performance on this APU, shown beneath the option list with a one-line rationale. Each marker opens with a color-coded tag: CHANGE = change away from default for a clear gain; TUNE = performance-relevant but workload-specific or expert-only (read the note); KEEP = the default already favors performance, leave it; NEUTRAL = no performance impact (security, connectivity, debug, or factory-trained values), leave at default.

## How the performance recommendations were derived

The Performance markers reflect published guidance for the AMD Strix Halo / Ryzen AI Max+ 395 platform and long-standing AMD CBS / Precision Boost Overdrive tuning practice, cross-referenced against community testing on this exact mini-PC. Key sources: AMD’s Variable Graphics Memory guidance and community Strix Halo LLM/BIOS setup guides (UMA Frame Buffer and IOMMU behavior, including the ~6% memory-read gain from disabling IOMMU and the finding that a small UMA carve-out costs nothing under Vulkan/GTT); AMD and 1usmus high-performance CBS recommendations (Global C-state, DF C-states, Power Supply Idle Control, PPC/Fixed-SOC P-state); Tom’s Hardware and other PBO + Curve Optimizer tuning references (PBO Limits = Motherboard, negative all-core Curve Optimizer, +200 MHz boost override); and the Beelink GTR9 Pro “Performance” power-mode reports that unlock the ~140 W sustained budget. Treat every TUNE entry as a starting point to validate on your unit, not a guaranteed-stable value: silicon, cooling, and firmware revision vary, and the soldered LPDDR5X memory is already trained to spec, so memory sub-timings and rail voltages are marked accordingly. Record originals before changing low-level CBS, Overclocking, or PMF values.

## Important notes

- Visibility: Many options are conditionally shown (suppress-if / grayout-if logic tied to other settings or detected hardware). A setting listed here may be hidden on screen unless its prerequisites are met. Some pages — notably Trusted Computing — appear more than once because the firmware presents a different variant per detected TPM/security device.

- Defaults: Values shown are the firmware’s compiled defaults (Standard Defaults / DefaultId 0). A unit’s current values may differ if previously changed or if the OEM ships an alternate profile.

- Stored values: Bracketed values are the raw values written to NVRAM, useful for scripted configuration (e.g. AMISCE/SCEWIN) but not normally seen in the GUI.

- Advanced tuning caution: The AMD CBS, AMD Overclocking, and AMD PMF chapters expose low-level processor, memory, and power-policy controls. Incorrect values can cause instability or prevent boot. Change only what you understand, and record originals first.

## Scope and coverage

This catalog covers every BIOS-owned Setup form-set in the image: the Aptio Setup tree and the AMD CBS, PBS, Overclocking, PMF, DASH/ASF, and RAIDXpert2 form-sets that hang off it. Together these account for 186 forms and 1,045 configurable options.

Deliberately excluded are the generic UEFI network-stack driver forms, which are not board-specific BIOS settings and appear only when a matching network controller or driver is present at runtime. For completeness these are inventoried in the appendix.

# Aptio Setup (Main Menu Tree)

The primary Aptio Setup interface — the menu tree presented on entry (Main, Advanced, Chipset, Security, Boot, Save & Exit) together with every standard submenu. This is the form-set most users interact with directly.

Form-set GUID: 7B59104A-C00D-4158-87FF-F04D6396A915     ·     Forms: 35     ·     Configurable settings: 184

### Setup   (Form 0x2710)

Submenus: Main  ‣  Advanced  ‣  Chipset  ‣  Security  ‣  Boot  ‣  Save & Exit

### Main   (Form 0x2711)

Information fields: BIOS Vendor, Core Version, Compliancy, BIOS Version, Build Date and Time, Access Level, Serial Number, Total Memory

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| System Language | Selection | (populated at runtime) [0x0] (populated at runtime) [0x1] ◀ performance NEUTRAL UI language — no performance impact. |
| System Date | Date | Date field (MM/DD/YYYY) ◀ performance NEUTRAL Clock setting — no performance impact. |
| System Time | Time | Time field (HH:MM:SS) ◀ performance NEUTRAL Clock setting — no performance impact. |

### Advanced   (Form 0x2712)

Submenus: Trusted Computing  ‣  Trusted Computing  ‣  Trusted Computing  ‣  AMD fTPM configuration  ‣  ACPI Settings  ‣  AMD CBS  ‣  AMD PBS  ‣  AMD PMF Feature  ‣  Dash Function  ‣  Super IO Configuration  ‣  Hardware Monitor  ‣  Watch Dog Configuration  ‣  S5 RTC Wake Settings  ‣  UEFI Variables Protection  ‣  CPU Configuration  ‣  AmiAgesaChipsetWrapper Configuration  ‣  AMI Graphic Output Protocol Policy  ‣  Debug Port Table Configuration  ‣  SIO Common Setting  ‣  PCI Subsystem Settings  ‣  USB Configuration  ‣  Network Stack Configuration  ‣  NVMe Configuration  ‣  Demo Board

### Trusted Computing   (Form 0x2719)

Information fields: NO Security Device Found

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| Security Device Support | Selection | Disabled [0x0] Enabled [0x1] ◀ default ◀ performance NEUTRAL Security/attestation feature — no performance impact. Set per your security policy (defaults are fine for performance). |

### Trusted Computing   (Form 0x271A)

Information fields: Firmware Version:, Vendor:, TPM Enabled Status:, TPM Active Status:, TPM Owner Status:, TCM Enabled Status:, TCM Active Status:, TCM Owner Status:

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| Security Device Support | Selection | Disabled [0x0] Enabled [0x1] ◀ default ◀ performance NEUTRAL Security/attestation feature — no performance impact. Set per your security policy (defaults are fine for performance). |
| TPM State | Selection | Disabled [0x0] Enabled [0x1] ◀ default ◀ performance NEUTRAL Security/attestation feature — no performance impact. Set per your security policy (defaults are fine for performance). |
| Pending operation | Selection | None [0x0] ◀ default TPM Clear [0x5] ◀ performance NEUTRAL TPM/Pluton security control — no performance impact; leave default. |
| Security Device Support | Selection | Disabled [0x0] Enabled [0x1] ◀ default ◀ performance NEUTRAL Security/attestation feature — no performance impact. Set per your security policy (defaults are fine for performance). |
| TCM State | Selection | Disabled [0x0] Enabled [0x1] ◀ default ◀ performance NEUTRAL Security/attestation feature — no performance impact. Set per your security policy (defaults are fine for performance). |
| Pending operation | Selection | None [0x0] ◀ default TPM Clear [0x5] ◀ performance NEUTRAL TPM/Pluton security control — no performance impact; leave default. |
| Device Select | Selection | TPM 1.2 [0x0] TPM 2.0 [0x1] Auto [0x2] ◀ default ◀ performance NEUTRAL TPM/Pluton security control — no performance impact; leave default. |

### Trusted Computing   (Form 0x271B)

Information fields: Firmware Version:, Vendor:, Active PCR banks, Available PCR banks

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| Security Device Support | Selection | Disabled [0x0] Enabled [0x1] ◀ default ◀ performance NEUTRAL Security/attestation feature — no performance impact. Set per your security policy (defaults are fine for performance). |
| SHA-1 PCR Bank | Selection | Disabled [0x0] Enabled [0x1] ◀ performance NEUTRAL Security/attestation feature — no performance impact. Set per your security policy (defaults are fine for performance). |
| SHA256 PCR Bank | Selection | Disabled [0x0] Enabled [0x2] ◀ performance NEUTRAL Security/attestation feature — no performance impact. Set per your security policy (defaults are fine for performance). |
| SHA384 PCR Bank | Selection | Disabled [0x0] Enabled [0x4] ◀ performance NEUTRAL Security/attestation feature — no performance impact. Set per your security policy (defaults are fine for performance). |
| SHA512 PCR Bank | Selection | Disabled [0x0] Enabled [0x8] ◀ performance NEUTRAL Security/attestation feature — no performance impact. Set per your security policy (defaults are fine for performance). |
| SM3_256 PCR Bank | Selection | Disabled [0x0] Enabled [0x1] ◀ performance NEUTRAL Security/attestation feature — no performance impact. Set per your security policy (defaults are fine for performance). |
| Pending operation | Selection | None [0x0] ◀ default TPM Clear [0x1] ◀ performance NEUTRAL TPM/Pluton security control — no performance impact; leave default. |
| Platform Hierarchy | Selection | Disabled [0x0] Enabled [0x1] ◀ default ◀ performance NEUTRAL Security/attestation feature — no performance impact. Set per your security policy (defaults are fine for performance). |
| Storage Hierarchy | Selection | Disabled [0x0] Enabled [0x1] ◀ default ◀ performance NEUTRAL Security/attestation feature — no performance impact. Set per your security policy (defaults are fine for performance). |
| Endorsement Hierarchy | Selection | Disabled [0x0] Enabled [0x1] ◀ default ◀ performance NEUTRAL Security/attestation feature — no performance impact. Set per your security policy (defaults are fine for performance). |
| Physical Presence Spec Version | Selection | 1.2 [0x0] 1.3 [0x1] ◀ default ◀ performance NEUTRAL Security/attestation feature — no performance impact. Set per your security policy (defaults are fine for performance). |
| TPM 2.0 UEFI Spec Version | Selection | TCG_1_2 [0x1] TCG_2 [0x2] ◀ default ◀ performance NEUTRAL Security/attestation feature — no performance impact. Set per your security policy (defaults are fine for performance). |
| TPM 2.0 InterfaceType | Selection | CRB [0x0] ◀ default TIS [0x1] ◀ performance NEUTRAL Security/attestation feature — no performance impact. Set per your security policy (defaults are fine for performance). |
| Device Select | Selection | TPM 1.2 [0x0] TPM 2.0 [0x1] Auto [0x2] ◀ default ◀ performance NEUTRAL TPM/Pluton security control — no performance impact; leave default. |

### AMD fTPM configuration   (Form 0x271C)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| Erase fTPM NV for factory reset | Selection | Disabled [0x0] Enabled [0x1] ◀ default ◀ performance NEUTRAL Security/attestation feature — no performance impact. Set per your security policy (defaults are fine for performance). |

### ACPI Settings   (Form 0x271D)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| ACPI Sleep State | Selection | Suspend Disabled [0x0] S3 (Suspend to RAM) [0x2] ◀ default ◀ performance NEUTRAL ACPI sleep/button policy — no runtime performance impact. S3 sleep is fine; set per preference. |

### Hardware Monitor   (Form 0x2735)

Information fields: CPU temperature, System temperature, CPU Fan1 Speed, CPU Fan2 Speed, VCore, V_SM, 3.3V, 5V

Submenus: Smart Fan Function

### Smart Fan Function   (Form 0x2737)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| CPU Smart Fan1 Mode | Selection | Manual Mode [0x0] Automatic Mode [0x1] ◀ default Full on Mode [0x2] ◀ performance TUNE Automatic (curve) for balance; Full-On maximizes cooling/boost headroom at the cost of noise. Manual lets you set an aggressive fixed curve. |
| Manual PWM Setting | Numeric | Min 0x0 · Max 0xFF · Step 0x1 · Default 0x7F ◀ default ◀ performance TUNE Fan-curve point. For sustained performance, bias the curve toward more airflow (lower start temp / higher PWM) to gain thermal headroom; trade against noise. |
| Fan off temperature limit | Numeric | Min 0x0 · Max 0x7F · Step 0x1 · Default 0x1E ◀ default ◀ performance TUNE Fan-curve point. For sustained performance, bias the curve toward more airflow (lower start temp / higher PWM) to gain thermal headroom; trade against noise. |
| Fan start temperature limit | Numeric | Min 0x0 · Max 0x7F · Step 0x1 · Default 0x23 ◀ default ◀ performance TUNE Fan-curve point. For sustained performance, bias the curve toward more airflow (lower start temp / higher PWM) to gain thermal headroom; trade against noise. |
| Fan full speed temperature limit | Numeric | Min 0x0 · Max 0xFF · Step 0x1 · Default 0x5B ◀ default ◀ performance TUNE Fan-curve point. For sustained performance, bias the curve toward more airflow (lower start temp / higher PWM) to gain thermal headroom; trade against noise. |
| Fan start PWM | Numeric | Min 0x0 · Max 0xFF · Step 0x1 · Default 0x46 ◀ default ◀ performance TUNE Fan-curve point. For sustained performance, bias the curve toward more airflow (lower start temp / higher PWM) to gain thermal headroom; trade against noise. |
| PWM SLOPE SETTING | Selection | 0.125 PWM [0x0] 0.25 PWM [0x1] 0.5 PWM [0x2] 1 PWM [0x3] ◀ default 2 PWM [0x4] 4 PWM [0x5] 8 PWM [0x6] 15.875 PWM [0x7] 4.5 PWM [0x8] ◀ performance TUNE Fan-curve point. For sustained performance, bias the curve toward more airflow (lower start temp / higher PWM) to gain thermal headroom; trade against noise. |
| Fan off temperature limit | Numeric | Min 0x0 · Max 0x7F · Step 0x1 · Default 0x1E ◀ default ◀ performance TUNE Fan-curve point. For sustained performance, bias the curve toward more airflow (lower start temp / higher PWM) to gain thermal headroom; trade against noise. |
| Fan start temperature limit | Numeric | Min 0x0 · Max 0x7F · Step 0x1 · Default 0x23 ◀ default ◀ performance TUNE Fan-curve point. For sustained performance, bias the curve toward more airflow (lower start temp / higher PWM) to gain thermal headroom; trade against noise. |
| Fan full speed temperature limit | Numeric | Min 0x0 · Max 0xFF · Step 0x1 · Default 0x5B ◀ default ◀ performance TUNE Fan-curve point. For sustained performance, bias the curve toward more airflow (lower start temp / higher PWM) to gain thermal headroom; trade against noise. |
| Fan start PWM | Numeric | Min 0x0 · Max 0xFF · Step 0x1 · Default 0x64 ◀ default ◀ performance TUNE Fan-curve point. For sustained performance, bias the curve toward more airflow (lower start temp / higher PWM) to gain thermal headroom; trade against noise. |
| PWM SLOPE SETTING | Selection | 0.125 PWM [0x0] 0.25 PWM [0x1] 0.5 PWM [0x2] 1 PWM [0x3] ◀ default 2 PWM [0x4] 4 PWM [0x5] 8 PWM [0x6] 15.875 PWM [0x7] 4.5 PWM [0x8] ◀ performance TUNE Fan-curve point. For sustained performance, bias the curve toward more airflow (lower start temp / higher PWM) to gain thermal headroom; trade against noise. |
| CPU Smart Fan2 Mode | Selection | Manual Mode [0x0] Automatic Mode [0x1] ◀ default Full on Mode [0x2] ◀ performance TUNE Automatic for balance, Full-On for maximum thermal headroom (louder). Match to Fan1. |
| Manual PWM Setting | Numeric | Min 0x0 · Max 0xFF · Step 0x1 · Default 0x7F ◀ default ◀ performance TUNE Fan-curve point. For sustained performance, bias the curve toward more airflow (lower start temp / higher PWM) to gain thermal headroom; trade against noise. |
| Fan off temperature limit | Numeric | Min 0x0 · Max 0x7F · Step 0x1 · Default 0x1E ◀ default ◀ performance TUNE Fan-curve point. For sustained performance, bias the curve toward more airflow (lower start temp / higher PWM) to gain thermal headroom; trade against noise. |
| Fan start temperature limit | Numeric | Min 0x0 · Max 0x7F · Step 0x1 · Default 0x23 ◀ default ◀ performance TUNE Fan-curve point. For sustained performance, bias the curve toward more airflow (lower start temp / higher PWM) to gain thermal headroom; trade against noise. |
| Fan full speed temperature limit | Numeric | Min 0x0 · Max 0xFF · Step 0x1 · Default 0x5B ◀ default ◀ performance TUNE Fan-curve point. For sustained performance, bias the curve toward more airflow (lower start temp / higher PWM) to gain thermal headroom; trade against noise. |
| Fan start PWM | Numeric | Min 0x0 · Max 0xFF · Step 0x1 · Default 0x46 ◀ default ◀ performance TUNE Fan-curve point. For sustained performance, bias the curve toward more airflow (lower start temp / higher PWM) to gain thermal headroom; trade against noise. |
| PWM SLOPE SETTING | Selection | 0.125 PWM [0x0] 0.25 PWM [0x1] 0.5 PWM [0x2] 1 PWM [0x3] ◀ default 2 PWM [0x4] 4 PWM [0x5] 8 PWM [0x6] 15.875 PWM [0x7] ◀ performance TUNE Fan-curve point. For sustained performance, bias the curve toward more airflow (lower start temp / higher PWM) to gain thermal headroom; trade against noise. |
| Fan off temperature limit | Numeric | Min 0x0 · Max 0x7F · Step 0x1 · Default 0x1E ◀ default ◀ performance TUNE Fan-curve point. For sustained performance, bias the curve toward more airflow (lower start temp / higher PWM) to gain thermal headroom; trade against noise. |
| Fan start temperature limit | Numeric | Min 0x0 · Max 0x7F · Step 0x1 · Default 0x23 ◀ default ◀ performance TUNE Fan-curve point. For sustained performance, bias the curve toward more airflow (lower start temp / higher PWM) to gain thermal headroom; trade against noise. |
| Fan full speed temperature limit | Numeric | Min 0x0 · Max 0xFF · Step 0x1 · Default 0x5B ◀ default ◀ performance TUNE Fan-curve point. For sustained performance, bias the curve toward more airflow (lower start temp / higher PWM) to gain thermal headroom; trade against noise. |
| Fan start PWM | Numeric | Min 0x0 · Max 0xFF · Step 0x1 · Default 0x64 ◀ default ◀ performance TUNE Fan-curve point. For sustained performance, bias the curve toward more airflow (lower start temp / higher PWM) to gain thermal headroom; trade against noise. |
| PWM SLOPE SETTING | Selection | 0.125 PWM [0x0] 0.25 PWM [0x1] 0.5 PWM [0x2] 1 PWM [0x3] ◀ default 2 PWM [0x4] 4 PWM [0x5] 8 PWM [0x6] 15.875 PWM [0x7] ◀ performance TUNE Fan-curve point. For sustained performance, bias the curve toward more airflow (lower start temp / higher PWM) to gain thermal headroom; trade against noise. |
| Smart Fan 3 Mode | Selection | Manual Mode [0x0] ◀ default Automatic Mode [0x1] ◀ performance TUNE Automatic for balance; aggressive/Full-On if this fan cools a perf-critical zone. |
| Manual PWM Setting | Numeric | Min 0x0 · Max 0xFF · Step 0x1 · Default 0x7F ◀ default ◀ performance TUNE Fan-curve point. For sustained performance, bias the curve toward more airflow (lower start temp / higher PWM) to gain thermal headroom; trade against noise. |
| Fan off temperature limit | Numeric | Min 0x0 · Max 0x7F · Step 0x1 · Default 0x10 ◀ default ◀ performance TUNE Fan-curve point. For sustained performance, bias the curve toward more airflow (lower start temp / higher PWM) to gain thermal headroom; trade against noise. |
| Fan start temperature limit | Numeric | Min 0x0 · Max 0x7F · Step 0x1 · Default 0x20 ◀ default ◀ performance TUNE Fan-curve point. For sustained performance, bias the curve toward more airflow (lower start temp / higher PWM) to gain thermal headroom; trade against noise. |
| Fan full speed temperature limit | Numeric | Min 0x0 · Max 0xFF · Step 0x1 · Default 0x7F ◀ default ◀ performance TUNE Fan-curve point. For sustained performance, bias the curve toward more airflow (lower start temp / higher PWM) to gain thermal headroom; trade against noise. |
| Fan start PWM | Numeric | Min 0x0 · Max 0xFF · Step 0x1 · Default 0x7F ◀ default ◀ performance TUNE Fan-curve point. For sustained performance, bias the curve toward more airflow (lower start temp / higher PWM) to gain thermal headroom; trade against noise. |
| PWM SLOPE SETTING | Selection | 0.125 PWM [0x0] 0.25 PWM [0x1] 0.5 PWM [0x2] ◀ default 1 PWM [0x3] 2 PWM [0x4] 4 PWM [0x5] 8 PWM [0x6] 15.875 PWM [0x7] ◀ performance TUNE Fan-curve point. For sustained performance, bias the curve toward more airflow (lower start temp / higher PWM) to gain thermal headroom; trade against noise. |
| Smart Fan 4 Mode | Selection | Manual Mode [0x0] ◀ default Automatic Mode [0x1] ◀ performance TUNE Automatic for balance; aggressive curve to hold temps down under sustained load. |
| Manual PWM Setting | Numeric | Min 0x0 · Max 0xFF · Step 0x1 · Default 0x7F ◀ default ◀ performance TUNE Fan-curve point. For sustained performance, bias the curve toward more airflow (lower start temp / higher PWM) to gain thermal headroom; trade against noise. |
| Fan off temperature limit | Numeric | Min 0x0 · Max 0x7F · Step 0x1 · Default 0x10 ◀ default ◀ performance TUNE Fan-curve point. For sustained performance, bias the curve toward more airflow (lower start temp / higher PWM) to gain thermal headroom; trade against noise. |
| Fan start temperature limit | Numeric | Min 0x0 · Max 0x7F · Step 0x1 · Default 0x20 ◀ default ◀ performance TUNE Fan-curve point. For sustained performance, bias the curve toward more airflow (lower start temp / higher PWM) to gain thermal headroom; trade against noise. |
| Fan full speed temperature limit | Numeric | Min 0x0 · Max 0xFF · Step 0x1 · Default 0x7F ◀ default ◀ performance TUNE Fan-curve point. For sustained performance, bias the curve toward more airflow (lower start temp / higher PWM) to gain thermal headroom; trade against noise. |
| Fan start PWM | Numeric | Min 0x0 · Max 0xFF · Step 0x1 · Default 0x7F ◀ default ◀ performance TUNE Fan-curve point. For sustained performance, bias the curve toward more airflow (lower start temp / higher PWM) to gain thermal headroom; trade against noise. |
| PWM SLOPE SETTING | Selection | 0.125 PWM [0x0] 0.25 PWM [0x1] 0.5 PWM [0x2] ◀ default 1 PWM [0x3] 2 PWM [0x4] 4 PWM [0x5] 8 PWM [0x6] 15.875 PWM [0x7] ◀ performance TUNE Fan-curve point. For sustained performance, bias the curve toward more airflow (lower start temp / higher PWM) to gain thermal headroom; trade against noise. |

### Watch Dog Configuration   (Form 0x2738)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| WDT Timeout Mode | Selection | Disabled [0x0] ◀ default Minute [0x1] Second [0x2] ◀ performance NEUTRAL Watchdog timer — reliability feature, no performance impact. Leave default. |
| WDT Timeout Value | Numeric | Min 0x1 · Max 0xFF · Step 0x1 · Default 0x1E ◀ default ◀ performance NEUTRAL Watchdog timer — reliability feature, no performance impact. Leave default. |

### S5 RTC Wake Settings   (Form 0x2739)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| Wake system from S5 | Selection | Disabled [0x0] ◀ default Fixed Time [0x1] Dynamic Time [0x2] ◀ performance NEUTRAL Wake-source/RTC option — no performance impact; set per convenience. |
| Wake up hour | Numeric | Min 0x0 · Max 0x17 · Step 0x1 · Default 0x0 ◀ default ◀ performance NEUTRAL Wake-source/RTC option — no performance impact; set per convenience. |
| Wake up minute | Numeric | Min 0x0 · Max 0x3B · Step 0x1 · Default 0x0 ◀ default ◀ performance NEUTRAL Wake-source/RTC option — no performance impact; set per convenience. |
| Wake up second | Numeric | Min 0x0 · Max 0x3B · Step 0x1 · Default 0x0 ◀ default ◀ performance NEUTRAL Wake-source/RTC option — no performance impact; set per convenience. |
| Wake up minute increase | Numeric | Min 0x1 · Max 0x5 · Step 0x1 · Default 0x1 ◀ default ◀ performance NEUTRAL Wake-source/RTC option — no performance impact; set per convenience. |

### UEFI Variables Protection   (Form 0x273A)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| Password protection of Runtime Variables | Selection | Enabled [0x1] Disabled [0x0] ◀ performance NEUTRAL Access-control credential — no performance impact. |

### CPU Configuration   (Form 0x273C)

Submenus: Node 0 Information

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| PSS Support | Selection | Disabled [0x0] Enabled [0x1] ◀ default ◀ performance KEEP Enabled. Exposes ACPI P-states (_PSS) so the OS governor can scale frequency; required for normal turbo behavior. |
| PPC Adjustment | Selection | PState 0 [0x0] ◀ default ◀ performance TUNE PState 0. Pinning the OS to P0 (with Fixed SOC Pstate / APBDIS) holds max fabric/SoC clock for lowest latency and consistent boost; costs idle power. Leave Auto for efficiency. |
| PPC Adjustment | Selection | PState 0 [0x0] ◀ default PState 1 [0x1] ◀ performance TUNE PState 0. Pinning the OS to P0 (with Fixed SOC Pstate / APBDIS) holds max fabric/SoC clock for lowest latency and consistent boost; costs idle power. Leave Auto for efficiency. |
| PPC Adjustment | Selection | PState 0 [0x0] ◀ default PState 1 [0x1] PState 2 [0x2] ◀ performance TUNE PState 0. Pinning the OS to P0 (with Fixed SOC Pstate / APBDIS) holds max fabric/SoC clock for lowest latency and consistent boost; costs idle power. Leave Auto for efficiency. |
| PPC Adjustment | Selection | PState 0 [0x0] ◀ default PState 1 [0x1] PState 2 [0x2] PState 3 [0x3] ◀ performance TUNE PState 0. Pinning the OS to P0 (with Fixed SOC Pstate / APBDIS) holds max fabric/SoC clock for lowest latency and consistent boost; costs idle power. Leave Auto for efficiency. |
| PPC Adjustment | Selection | PState 0 [0x0] ◀ default PState 1 [0x1] PState 2 [0x2] PState 3 [0x3] PState 4 [0x4] ◀ performance TUNE PState 0. Pinning the OS to P0 (with Fixed SOC Pstate / APBDIS) holds max fabric/SoC clock for lowest latency and consistent boost; costs idle power. Leave Auto for efficiency. |
| PPC Adjustment | Selection | PState 0 [0x0] ◀ default PState 1 [0x1] PState 2 [0x2] PState 3 [0x3] PState 4 [0x4] PState 5 [0x5] ◀ performance TUNE PState 0. Pinning the OS to P0 (with Fixed SOC Pstate / APBDIS) holds max fabric/SoC clock for lowest latency and consistent boost; costs idle power. Leave Auto for efficiency. |
| PPC Adjustment | Selection | PState 0 [0x0] ◀ default PState 1 [0x1] PState 2 [0x2] PState 3 [0x3] PState 4 [0x4] PState 5 [0x5] PState 6 [0x6] ◀ performance TUNE PState 0. Pinning the OS to P0 (with Fixed SOC Pstate / APBDIS) holds max fabric/SoC clock for lowest latency and consistent boost; costs idle power. Leave Auto for efficiency. |
| PPC Adjustment | Selection | PState 0 [0x0] ◀ default PState 1 [0x1] PState 2 [0x2] PState 3 [0x3] PState 4 [0x4] PState 5 [0x5] PState 6 [0x6] PState 7 [0x7] ◀ performance TUNE PState 0. Pinning the OS to P0 (with Fixed SOC Pstate / APBDIS) holds max fabric/SoC clock for lowest latency and consistent boost; costs idle power. Leave Auto for efficiency. |
| NX Mode | Selection | Disabled [0x0] Enabled [0x1] ◀ default ◀ performance KEEP Enabled. No-Execute page protection; negligible perf cost and required by modern OSes. |

### AmiAgesaChipsetWrapper Configuration   (Form 0x2741)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| PCR7 WA Support | Selection | Disabled [0x0] Enabled [0x1] Auto [0x2] ◀ default ◀ performance NEUTRAL Security/attestation feature — no performance impact. Set per your security policy (defaults are fine for performance). |
| Enable Dual Vga Controllers | Selection | Disabled [0x0] ◀ default Enabled [0x1] ◀ performance NEUTRAL Only relevant with a second GPU — no performance impact for the iGPU-only default. |

### AMI Graphic Output Protocol Policy   (Form 0x274B)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| Output Select | Selection | Unknown Device [0x0] ◀ default ◀ performance NEUTRAL Serial/debug interface — leave Disabled/default. Debug output can slightly slow boot; no runtime performance benefit. |
| Output Select | Selection | Unknown Device [0x0] ◀ default Unknown Device [0x1] ◀ performance NEUTRAL Serial/debug interface — leave Disabled/default. Debug output can slightly slow boot; no runtime performance benefit. |
| Output Select | Selection | Unknown Device [0x0] ◀ default Unknown Device [0x1] Unknown Device [0x2] ◀ performance NEUTRAL Serial/debug interface — leave Disabled/default. Debug output can slightly slow boot; no runtime performance benefit. |
| Output Select | Selection | Unknown Device [0x0] ◀ default Unknown Device [0x1] Unknown Device [0x2] Unknown Device [0x3] ◀ performance NEUTRAL Serial/debug interface — leave Disabled/default. Debug output can slightly slow boot; no runtime performance benefit. |
| Output Select | Selection | Unknown Device [0x0] ◀ default Unknown Device [0x1] Unknown Device [0x2] Unknown Device [0x3] Unknown Device [0x4] ◀ performance NEUTRAL Serial/debug interface — leave Disabled/default. Debug output can slightly slow boot; no runtime performance benefit. |
| Output Select | Selection | Unknown Device [0x0] ◀ default Unknown Device [0x1] Unknown Device [0x2] Unknown Device [0x3] Unknown Device [0x4] Unknown Device [0x5] ◀ performance NEUTRAL Serial/debug interface — leave Disabled/default. Debug output can slightly slow boot; no runtime performance benefit. |
| Output Select | Selection | Unknown Device [0x0] ◀ default Unknown Device [0x1] Unknown Device [0x2] Unknown Device [0x3] Unknown Device [0x4] Unknown Device [0x5] Unknown Device [0x6] ◀ performance NEUTRAL Serial/debug interface — leave Disabled/default. Debug output can slightly slow boot; no runtime performance benefit. |
| Output Select | Selection | Unknown Device [0x0] ◀ default Unknown Device [0x1] Unknown Device [0x2] Unknown Device [0x3] Unknown Device [0x4] Unknown Device [0x5] Unknown Device [0x6] Unknown Device [0x7] ◀ performance NEUTRAL Serial/debug interface — leave Disabled/default. Debug output can slightly slow boot; no runtime performance benefit. |

### Debug Port Table   (Form 0x274D)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| Debug Port Table | Selection | Disabled [0x0] ◀ default Enabled [0x1] ◀ performance NEUTRAL Serial/debug interface — leave Disabled/default. Debug output can slightly slow boot; no runtime performance benefit. |
| Debug Port Table 2 | Selection | Disabled [0x0] ◀ default Enabled [0x1] ◀ performance NEUTRAL Serial/debug interface — leave Disabled/default. Debug output can slightly slow boot; no runtime performance benefit. |

### USB Configuration   (Form 0x2750)

Information fields: USB Module Version, USB Controllers:, USB Devices:, USB hardware delays and time-outs:, Mass Storage Devices:

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| USB Support | Selection | Disabled [0x0] Enabled [0x1] ◀ default ◀ performance NEUTRAL USB port/controller enable — functional toggle, no performance impact. Leave default unless disabling unused ports for security/power. |
| Legacy USB Support | Selection | Enabled [0x0] Disabled [0x1] ◀ default Auto [0x2] ◀ performance NEUTRAL USB port/controller enable — functional toggle, no performance impact. Leave default unless disabling unused ports for security/power. |
| XHCI Legacy Support | Selection | Enabled [0x1] ◀ default Disabled [0x0] ◀ performance NEUTRAL USB port/controller enable — functional toggle, no performance impact. Leave default unless disabling unused ports for security/power. |
| XHCI Hand-off | Selection | Enabled [0x1] Disabled [0x0] ◀ performance NEUTRAL USB port/controller enable — functional toggle, no performance impact. Leave default unless disabling unused ports for security/power. |
| EHCI Hand-off | Selection | Disabled [0x0] ◀ default Enabled [0x1] ◀ performance NEUTRAL USB port/controller enable — functional toggle, no performance impact. Leave default unless disabling unused ports for security/power. |
| USB Mass Storage Driver Support | Selection | Disabled [0x0] Enabled [0x1] ◀ default ◀ performance NEUTRAL USB port/controller enable — functional toggle, no performance impact. Leave default unless disabling unused ports for security/power. |
| USB transfer time-out | Selection | 1 sec [0x1] 5 sec [0x5] 10 sec [0xA] 20 sec [0x14] ◀ default ◀ performance NEUTRAL USB port/controller enable — functional toggle, no performance impact. Leave default unless disabling unused ports for security/power. |
| Device reset time-out | Selection | 10 sec [0x0] 20 sec [0x1] ◀ default 30 sec [0x2] 40 sec [0x3] ◀ performance NEUTRAL USB port/controller enable — functional toggle, no performance impact. Leave default unless disabling unused ports for security/power. |
| Device power-up delay | Selection | Auto [0x0] ◀ default Manual [0x1] ◀ performance NEUTRAL USB port/controller enable — functional toggle, no performance impact. Leave default unless disabling unused ports for security/power. |
| Device power-up delay in seconds | Numeric | Min 0x1 · Max 0x28 · Step 0x1 · Default 0x5 ◀ default ◀ performance NEUTRAL USB port/controller enable — functional toggle, no performance impact. Leave default unless disabling unused ports for security/power. |
| Mass Storage Device Emulation Type | Selection | Per-device emulation override, repeated once per detected USB mass-storage device. Options (identical for each): Auto [0x0] ◀ default Floppy [0x1] Forced FDD [0x2] Hard Disk [0x3] CD-ROM [0x4]. Collapsed from 32 identical runtime rows. ◀ performance NEUTRAL Informational / non-configurable field — no performance action. |

### Network Stack Configuration   (Form 0x2752)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| Network Stack | Selection | Disabled [0x0] ◀ default Enabled [0x1] ◀ performance NEUTRAL Network-boot stack — leave Disabled if not PXE-booting (trims boot time); no runtime performance impact. |
| IPv4 PXE Support | Selection | Disabled [0x0] ◀ default Enabled [0x1] ◀ performance NEUTRAL Network-boot stack — leave Disabled if not PXE-booting (trims boot time); no runtime performance impact. |
| IPv4 HTTP Support | Selection | Disabled [0x0] ◀ default Enabled [0x1] ◀ performance NEUTRAL Network-boot stack — leave Disabled if not PXE-booting (trims boot time); no runtime performance impact. |
| IPv6 PXE Support | Selection | Disabled [0x0] ◀ default Enabled [0x1] ◀ performance NEUTRAL Network-boot stack — leave Disabled if not PXE-booting (trims boot time); no runtime performance impact. |
| IPv6 HTTP Support | Selection | Disabled [0x0] ◀ default Enabled [0x1] ◀ performance NEUTRAL Network-boot stack — leave Disabled if not PXE-booting (trims boot time); no runtime performance impact. |
| PXE boot wait time | Numeric | Min 0x0 · Max 0x5 · Step 0x1 · Default 0x0 ◀ default ◀ performance NEUTRAL Network-boot stack — leave Disabled if not PXE-booting (trims boot time); no runtime performance impact. |
| Media detect count | Numeric | Min 0x1 · Max 0x32 · Step 0x1 · Default 0x1 ◀ default ◀ performance NEUTRAL Network-boot stack — leave Disabled if not PXE-booting (trims boot time); no runtime performance impact. |

### Demo Board   (Form 0x2964)

Submenus: PCI-E Port

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| Onboard PCIE LAN PXE ROM | Selection | Disabled [0x0] Enabled [0x1] ◀ default ◀ performance NEUTRAL Network-boot stack — leave Disabled if not PXE-booting (trims boot time); no runtime performance impact. |

### PCI-E Port   (Form 0x2966)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| Pcie Port Control | Selection | Disabled [0x0] Enabled [0x1] ◀ default ◀ performance TUNE PCIe link power management. Disabled = full link speed and lowest latency (best performance, higher idle power); Enabled (L1) = power saving with wake latency. Performance: Disabled. |
| Device0 | Selection | Disabled [0x0] Enabled [0x1] Auto [0xFF] ◀ default ◀ performance NEUTRAL Per-port PCIe device enable — functional toggle, no performance impact. Leave default (enabled for ports you use). |
| ASPM Mode Control(Device0) | Selection | Disabled [0x0] L0s Entry [0x1] L1 Entry [0x2] L0s And L1 Entry [0x3] Auto [0xFF] ◀ default ◀ performance TUNE PCIe link power management. Disabled = full link speed and lowest latency (best performance, higher idle power); Enabled (L1) = power saving with wake latency. Performance: Disabled. |
| Hotplug Mode Control(Device0) | Selection | Disabled [0x0] Hotplug Basic [0x1] Hotplug Server [0x2] Hotplug Enhanced [0x3] Hotplug Inboard [0x4] Auto [0xFF] ◀ default ◀ performance NEUTRAL PCIe hot-plug capability for the port — no throughput impact; enable only if you hot-swap devices on that slot. |
| Device1 | Selection | Disabled [0x0] Enabled [0x1] Auto [0xFF] ◀ default ◀ performance NEUTRAL Per-port PCIe device enable — functional toggle, no performance impact. Leave default (enabled for ports you use). |
| ASPM Mode Control(Device1) | Selection | Disabled [0x0] L0s Entry [0x1] L1 Entry [0x2] L0s And L1 Entry [0x3] Auto [0xFF] ◀ default ◀ performance TUNE PCIe link power management. Disabled = full link speed and lowest latency (best performance, higher idle power); Enabled (L1) = power saving with wake latency. Performance: Disabled. |
| Hotplug Mode Control(Device1) | Selection | Disabled [0x0] Hotplug Basic [0x1] Hotplug Server [0x2] Hotplug Enhanced [0x3] Hotplug Inboard [0x4] Auto [0xFF] ◀ default ◀ performance NEUTRAL PCIe hot-plug capability for the port — no throughput impact; enable only if you hot-swap devices on that slot. |
| Device2 | Selection | Disabled [0x0] Enabled [0x1] Auto [0xFF] ◀ default ◀ performance NEUTRAL Per-port PCIe device enable — functional toggle, no performance impact. Leave default (enabled for ports you use). |
| ASPM Mode Control(Device2) | Selection | Disabled [0x0] L0s Entry [0x1] L1 Entry [0x2] L0s And L1 Entry [0x3] Auto [0xFF] ◀ default ◀ performance TUNE PCIe link power management. Disabled = full link speed and lowest latency (best performance, higher idle power); Enabled (L1) = power saving with wake latency. Performance: Disabled. |
| Hotplug Mode Control(Device2) | Selection | Disabled [0x0] Hotplug Basic [0x1] Hotplug Server [0x2] Hotplug Enhanced [0x3] Hotplug Inboard [0x4] Auto [0xFF] ◀ default ◀ performance NEUTRAL PCIe hot-plug capability for the port — no throughput impact; enable only if you hot-swap devices on that slot. |
| Device3 | Selection | Disabled [0x0] Enabled [0x1] Auto [0xFF] ◀ default ◀ performance NEUTRAL Per-port PCIe device enable — functional toggle, no performance impact. Leave default (enabled for ports you use). |
| ASPM Mode Control(Device3) | Selection | Disabled [0x0] L0s Entry [0x1] L1 Entry [0x2] L0s And L1 Entry [0x3] Auto [0xFF] ◀ default ◀ performance TUNE PCIe link power management. Disabled = full link speed and lowest latency (best performance, higher idle power); Enabled (L1) = power saving with wake latency. Performance: Disabled. |
| Hotplug Mode Control(Device3) | Selection | Disabled [0x0] Hotplug Basic [0x1] Hotplug Server [0x2] Hotplug Enhanced [0x3] Hotplug Inboard [0x4] Auto [0xFF] ◀ default ◀ performance NEUTRAL PCIe hot-plug capability for the port — no throughput impact; enable only if you hot-swap devices on that slot. |
| Device4 | Selection | Disabled [0x0] Enabled [0x1] Auto [0xFF] ◀ default ◀ performance NEUTRAL Per-port PCIe device enable — functional toggle, no performance impact. Leave default (enabled for ports you use). |
| ASPM Mode Control(Device4) | Selection | Disabled [0x0] L0s Entry [0x1] L1 Entry [0x2] L0s And L1 Entry [0x3] Auto [0xFF] ◀ default ◀ performance TUNE PCIe link power management. Disabled = full link speed and lowest latency (best performance, higher idle power); Enabled (L1) = power saving with wake latency. Performance: Disabled. |
| Hotplug Mode Control(Device4) | Selection | Disabled [0x0] Hotplug Basic [0x1] Hotplug Server [0x2] Hotplug Enhanced [0x3] Hotplug Inboard [0x4] Auto [0xFF] ◀ default ◀ performance NEUTRAL PCIe hot-plug capability for the port — no throughput impact; enable only if you hot-swap devices on that slot. |
| Device5 | Selection | Disabled [0x0] Enabled [0x1] Auto [0xFF] ◀ default ◀ performance NEUTRAL Per-port PCIe device enable — functional toggle, no performance impact. Leave default (enabled for ports you use). |
| ASPM Mode Control(Device5) | Selection | Disabled [0x0] L0s Entry [0x1] L1 Entry [0x2] L0s And L1 Entry [0x3] Auto [0xFF] ◀ default ◀ performance TUNE PCIe link power management. Disabled = full link speed and lowest latency (best performance, higher idle power); Enabled (L1) = power saving with wake latency. Performance: Disabled. |
| Hotplug Mode Control(Device5) | Selection | Disabled [0x0] Hotplug Basic [0x1] Hotplug Server [0x2] Hotplug Enhanced [0x3] Hotplug Inboard [0x4] Auto [0xFF] ◀ default ◀ performance NEUTRAL PCIe hot-plug capability for the port — no throughput impact; enable only if you hot-swap devices on that slot. |
| Device6 | Selection | Disabled [0x0] Enabled [0x1] Auto [0xFF] ◀ default ◀ performance NEUTRAL Per-port PCIe device enable — functional toggle, no performance impact. Leave default (enabled for ports you use). |
| ASPM Mode Control(Device6) | Selection | Disabled [0x0] L0s Entry [0x1] L1 Entry [0x2] L0s And L1 Entry [0x3] Auto [0xFF] ◀ default ◀ performance TUNE PCIe link power management. Disabled = full link speed and lowest latency (best performance, higher idle power); Enabled (L1) = power saving with wake latency. Performance: Disabled. |
| Hotplug Mode Control(Device6) | Selection | Disabled [0x0] Hotplug Basic [0x1] Hotplug Server [0x2] Hotplug Enhanced [0x3] Hotplug Inboard [0x4] Auto [0xFF] ◀ default ◀ performance NEUTRAL PCIe hot-plug capability for the port — no throughput impact; enable only if you hot-swap devices on that slot. |
| Device7 | Selection | Disabled [0x0] Enabled [0x1] Auto [0xFF] ◀ default ◀ performance NEUTRAL Per-port PCIe device enable — functional toggle, no performance impact. Leave default (enabled for ports you use). |
| ASPM Mode Control(Device7) | Selection | Disabled [0x0] L0s Entry [0x1] L1 Entry [0x2] L0s And L1 Entry [0x3] Auto [0xFF] ◀ default ◀ performance TUNE PCIe link power management. Disabled = full link speed and lowest latency (best performance, higher idle power); Enabled (L1) = power saving with wake latency. Performance: Disabled. |
| Hotplug Mode Control(Device7) | Selection | Disabled [0x0] Hotplug Basic [0x1] Hotplug Server [0x2] Hotplug Enhanced [0x3] Hotplug Inboard [0x4] Auto [0xFF] ◀ default ◀ performance NEUTRAL PCIe hot-plug capability for the port — no throughput impact; enable only if you hot-swap devices on that slot. |

### Chipset   (Form 0x2713)

Submenus: South Bridge  ‣  GFX Configuration  ‣  North Bridge

### South Bridge   (Form 0x2725)

Submenus: SB USB Configuration  ‣  SB MISC DEBUG Configuration

### SB USB Configuration   (Form 0x2726)

Submenus: USB0 Ports  ‣  USB1 Ports  ‣  USB3 Ports  ‣  USB4 Ports

### USB0 Ports   (Form 0x2727)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| XHCI0 Port 0 | Selection | Disabled [0x0] Enabled [0x1] ◀ default ◀ performance NEUTRAL USB port/controller enable — functional toggle, no performance impact. Leave default unless disabling unused ports for security/power. |
| XHCI0 Port 1 | Selection | Disabled [0x0] Enabled [0x1] ◀ default ◀ performance NEUTRAL USB port/controller enable — functional toggle, no performance impact. Leave default unless disabling unused ports for security/power. |
| XHCI0 2.0 Port 0 | Selection | Disabled [0x0] Enabled [0x1] ◀ default ◀ performance NEUTRAL USB port/controller enable — functional toggle, no performance impact. Leave default unless disabling unused ports for security/power. |
| XHCI0 2.0 Port 1 | Selection | Disabled [0x0] Enabled [0x1] ◀ default ◀ performance NEUTRAL USB port/controller enable — functional toggle, no performance impact. Leave default unless disabling unused ports for security/power. |
| XHCI0 2.0 Port 2 | Selection | Disabled [0x0] Enabled [0x1] ◀ default ◀ performance NEUTRAL USB port/controller enable — functional toggle, no performance impact. Leave default unless disabling unused ports for security/power. |
| XHCI0 2.0 Port 3 | Selection | Disabled [0x0] Enabled [0x1] ◀ default ◀ performance NEUTRAL USB port/controller enable — functional toggle, no performance impact. Leave default unless disabling unused ports for security/power. |
| XHCI0 2.0 Port 4 | Selection | Disabled [0x0] Enabled [0x1] ◀ default ◀ performance NEUTRAL USB port/controller enable — functional toggle, no performance impact. Leave default unless disabling unused ports for security/power. |

### USB1 Ports   (Form 0x2728)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| XHCI1 Port 0 | Selection | Disabled [0x0] Enabled [0x1] ◀ default ◀ performance NEUTRAL USB port/controller enable — functional toggle, no performance impact. Leave default unless disabling unused ports for security/power. |
| XHCI1 2.0 Port 0 | Selection | Disabled [0x0] Enabled [0x1] ◀ default ◀ performance NEUTRAL USB port/controller enable — functional toggle, no performance impact. Leave default unless disabling unused ports for security/power. |

### USB3 Ports   (Form 0x2729)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| XHCI3 Port 0 | Selection | Disabled [0x0] Enabled [0x1] ◀ default ◀ performance NEUTRAL USB port/controller enable — functional toggle, no performance impact. Leave default unless disabling unused ports for security/power. |
| XHCI3 2.0 Port 0 | Selection | Disabled [0x0] Enabled [0x1] ◀ default ◀ performance NEUTRAL USB port/controller enable — functional toggle, no performance impact. Leave default unless disabling unused ports for security/power. |

### USB4 Ports   (Form 0x272A)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| XHCI4 Port 0 | Selection | Disabled [0x0] Enabled [0x1] ◀ default ◀ performance NEUTRAL USB port/controller enable — functional toggle, no performance impact. Leave default unless disabling unused ports for security/power. |
| XHCI4 2.0 Port 0 | Selection | Disabled [0x0] Enabled [0x1] ◀ default ◀ performance NEUTRAL USB port/controller enable — functional toggle, no performance impact. Leave default unless disabling unused ports for security/power. |

### SB MISC DEBUG Configuration   (Form 0x272B)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| HPET ACPI table In SB | Selection | Disabled [0x0] Enabled [0x1] Auto [0xFF] ◀ default ◀ performance TUNE HPET: leave Enabled (Auto). Modern AMD platforms use invariant TSC; forcing HPET off rarely helps and can break timing-sensitive software. |

### North Bridge   (Form 0x273F)

Submenus: Socket 0 Information

### Security   (Form 0x2714)

Information fields: Minimum length, Maximum length

Submenus: Secure Boot  ‣  Secure Flash Update

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| Administrator Password | Password | Password entry ◀ performance NEUTRAL Access-control credential — no performance impact. |
| User Password | Password | Password entry ◀ performance NEUTRAL Access-control credential — no performance impact. |

### Secure Boot   (Form 0x2815)

Information fields: System Mode

Submenus: Restore Factory Keys  ‣  Reset To Setup Mode  ‣  Expert Key Management

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| Secure Boot | Selection | Disabled [0x0] Enabled [0x1] ◀ performance NEUTRAL Security/attestation feature — no performance impact. Set per your security policy (defaults are fine for performance). |
| Secure Boot Mode | Selection | Standard [0x0] Custom [0x1] ◀ performance NEUTRAL Security/attestation feature — no performance impact. Set per your security policy (defaults are fine for performance). |

### Expert Key Management   (Form 0x281F)

Information fields: Vendor Keys

Submenus: Restore Factory Keys  ‣  Reset To Setup Mode  ‣  Enroll Efi Image  ‣  Export Secure Boot variables

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| Factory Key Provision | Selection | Disabled [0x0] ◀ default Enabled [0x1] ◀ performance NEUTRAL Security/attestation feature — no performance impact. Set per your security policy (defaults are fine for performance). |

### Secure Flash Update   (Form 0x282E)

Information fields: Signed FW Update, FW Key File, FW Key Type, FW Update Methods, FW Revision Rollback, Flash Write Protect

### Boot   (Form 0x2715)

Submenus: UEFI NVME Drive BBS Priorities  ‣  UEFI Hard Disk Drive BBS Priorities  ‣  UEFI CDROM/DVD Drive BBS Priorities  ‣  UEFI SD Drive BBS Priorities  ‣  UEFI USB Drive BBS Priorities  ‣  UEFI NETWORK Drive BBS Priorities  ‣  UEFI Application Boot Priorities

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| Setup Prompt Timeout | Numeric | Min 0x1 · Max 0xFFFF · Step 0x1 · Default 0x1 ◀ default ◀ performance NEUTRAL POST key-wait duration — cosmetic; lower trims a couple seconds off boot. |
| Bootup NumLock State | Selection | On [0x1] ◀ default Off [0x0] ◀ performance NEUTRAL Cosmetic boot state — no performance impact. |
| Fast Boot | Selection | Disabled [0x0] ◀ default Enabled [0x1] ◀ performance TUNE Enabled for faster POST (skips some device init). Disable temporarily if you need full enumeration or to enter Setup reliably. |
| SATA Support | Selection | Last Boot SATA Devices Only [0x0] All SATA Devices [0x1] ◀ performance KEEP Keep enabled for the storage/devices you use. Enabling NVMe support is required to boot/perform from NVMe; no downside. |
| NVMe Support | Selection | Disabled [0x0] Enabled [0x1] ◀ default ◀ performance KEEP Keep enabled for the storage/devices you use. Enabling NVMe support is required to boot/perform from NVMe; no downside. |
| UFS Support | Selection | Disabled [0x0] Enabled [0x1] ◀ default ◀ performance KEEP Keep enabled for the storage/devices you use. Enabling NVMe support is required to boot/perform from NVMe; no downside. |
| VGA Support | Selection | Auto [0x0] ◀ default EFI Driver [0x1] ◀ performance KEEP Keep enabled for the storage/devices you use. Enabling NVMe support is required to boot/perform from NVMe; no downside. |
| USB Support | Selection | Disabled [0x0] Full Initial [0x1] ◀ default Partial Initial [0x2] ◀ performance NEUTRAL USB port/controller enable — functional toggle, no performance impact. Leave default unless disabling unused ports for security/power. |
| PS2 Devices Support | Selection | Disabled [0x0] Enabled [0x1] ◀ default ◀ performance KEEP Keep enabled for the storage/devices you use. Enabling NVMe support is required to boot/perform from NVMe; no downside. |
| Network Stack Driver Support | Selection | Disabled [0x0] ◀ default Enabled [0x1] ◀ performance NEUTRAL Network-boot stack — leave Disabled if not PXE-booting (trims boot time); no runtime performance impact. |
| Redirection Support | Selection | Disabled [0x0] ◀ default Enabled [0x1] ◀ performance NEUTRAL Serial/debug interface — leave Disabled/default. Debug output can slightly slow boot; no runtime performance benefit. |
| Driver Option #%d | Selection | (populated at runtime) [0x0] ◀ default (populated at runtime) [0x1] ◀ performance NEUTRAL Boot/driver ordering — no performance impact; arrange to boot your primary OS first. |
| Boot mode select | Selection | LEGACY [0x0] UEFI [0x1] ◀ default ◀ performance KEEP UEFI (not Legacy/CSM). UEFI is required for Resizable BAR and modern features; CSM blocks Above-4G/ReBAR. |
| Boot Option #1 | Selection | NVME [0x0] ◀ default Hard Disk [0x1] CD/DVD [0x2] SD [0x3] USB Device [0x4] Network [0x5] UEFI AP [0x6] Disabled [0x7] ◀ performance NEUTRAL Boot/driver ordering — no performance impact; arrange to boot your primary OS first. |
| Boot Option #2 | Selection | NVME [0x0] Hard Disk [0x1] ◀ default CD/DVD [0x2] SD [0x3] USB Device [0x4] Network [0x5] UEFI AP [0x6] Disabled [0x7] ◀ performance NEUTRAL Boot/driver ordering — no performance impact; arrange to boot your primary OS first. |
| Boot Option #3 | Selection | NVME [0x0] Hard Disk [0x1] CD/DVD [0x2] ◀ default SD [0x3] USB Device [0x4] Network [0x5] UEFI AP [0x6] Disabled [0x7] ◀ performance NEUTRAL Boot/driver ordering — no performance impact; arrange to boot your primary OS first. |
| Boot Option #4 | Selection | NVME [0x0] Hard Disk [0x1] CD/DVD [0x2] SD [0x3] ◀ default USB Device [0x4] Network [0x5] UEFI AP [0x6] Disabled [0x7] ◀ performance NEUTRAL Boot/driver ordering — no performance impact; arrange to boot your primary OS first. |
| Boot Option #5 | Selection | NVME [0x0] Hard Disk [0x1] CD/DVD [0x2] SD [0x3] USB Device [0x4] ◀ default Network [0x5] UEFI AP [0x6] Disabled [0x7] ◀ performance NEUTRAL Boot/driver ordering — no performance impact; arrange to boot your primary OS first. |
| Boot Option #6 | Selection | NVME [0x0] Hard Disk [0x1] CD/DVD [0x2] SD [0x3] USB Device [0x4] Network [0x5] ◀ default UEFI AP [0x6] Disabled [0x7] ◀ performance NEUTRAL Boot/driver ordering — no performance impact; arrange to boot your primary OS first. |
| Boot Option #7 | Selection | NVME [0x0] Hard Disk [0x1] CD/DVD [0x2] SD [0x3] USB Device [0x4] Network [0x5] UEFI AP [0x6] ◀ default Disabled [0x7] ◀ performance NEUTRAL Boot/driver ordering — no performance impact; arrange to boot your primary OS first. |
| Boot Option #1 | Selection | Hard Disk [0x0] ◀ default NVME [0x1] CD/DVD [0x2] SD [0x3] USB Device [0x4] Network [0x5] Disabled [0x6] ◀ performance NEUTRAL Boot/driver ordering — no performance impact; arrange to boot your primary OS first. |
| Boot Option #2 | Selection | Hard Disk [0x0] NVME [0x1] ◀ default CD/DVD [0x2] SD [0x3] USB Device [0x4] Network [0x5] Disabled [0x6] ◀ performance NEUTRAL Boot/driver ordering — no performance impact; arrange to boot your primary OS first. |
| Boot Option #3 | Selection | Hard Disk [0x0] NVME [0x1] CD/DVD [0x2] ◀ default SD [0x3] USB Device [0x4] Network [0x5] Disabled [0x6] ◀ performance NEUTRAL Boot/driver ordering — no performance impact; arrange to boot your primary OS first. |
| Boot Option #4 | Selection | Hard Disk [0x0] NVME [0x1] CD/DVD [0x2] SD [0x3] ◀ default USB Device [0x4] Network [0x5] Disabled [0x6] ◀ performance NEUTRAL Boot/driver ordering — no performance impact; arrange to boot your primary OS first. |
| Boot Option #5 | Selection | Hard Disk [0x0] NVME [0x1] CD/DVD [0x2] SD [0x3] USB Device [0x4] ◀ default Network [0x5] Disabled [0x6] ◀ performance NEUTRAL Boot/driver ordering — no performance impact; arrange to boot your primary OS first. |
| Boot Option #6 | Selection | Hard Disk [0x0] NVME [0x1] CD/DVD [0x2] SD [0x3] USB Device [0x4] Network [0x5] ◀ default Disabled [0x6] ◀ performance NEUTRAL Boot/driver ordering — no performance impact; arrange to boot your primary OS first. |

# AMD CBS (Common BIOS Settings)

AMD Common BIOS Settings — the AGESA-provided processor and fabric tuning tree, reached via Advanced ‣ AMD CBS. Governs CPU, Data Fabric (DF), Unified Memory Controller (UMC/DDR), NBIO/PCIe, FCH, and SoC/APU miscellaneous options.

Form-set GUID: B04535E3-3004-4946-9EB7-149428983053     ·     Forms: 51     ·     Configurable settings: 318

### AMD CBS   (Form 0x7000)

Information fields: AMD CBS Revision Number

Submenus: CPU Common Options  ‣  DF Common Options  ‣  UMC Common Options  ‣  NBIO Common Options  ‣  FCH Common Options  ‣  SMU Common Options  ‣  SOC Miscellaneous Control

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| Combo CBS | Numeric | Min 0x0 · Max 0xFF · Default 0xFE ◀ default ◀ performance NEUTRAL Firmware wrapper/info field — no user performance impact; leave default. |

### CPU Common Options   (Form 0x7001)

Submenus: Performance  ‣  Prefetcher settings  ‣  Core Watchdog

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| REP-MOV/STOS Streaming | Selection | Disabled [0x0] Enabled [0x1] ◀ default ◀ performance KEEP Enabled (Auto). Streaming string ops improve large memcpy/memset bandwidth. |
| RedirectForReturnDis | Selection | Auto [0xFF] ◀ default 1 [0x1] 0 [0x0] ◀ performance NEUTRAL Errata/compat workaround — leave Auto; no performance benefit to changing. |
| Platform First Error Handling | Selection | Enabled [0x1] Disabled [0x0] Auto [0x3] ◀ default ◀ performance NEUTRAL RAS/error-handling control — reliability feature, no performance impact. Leave Auto/default. |
| Core Performance Boost | Selection | Disabled [0x0] Auto [0x1] ◀ default ◀ performance KEEP Enabled. Allows cores to boost above base clock; the single biggest stock throughput lever. Leave on. |
| Global C-state Control | Selection | Disabled [0x0] ◀ default Enabled [0x1] Auto [0x3] ◀ performance TUNE Enabled for daily/efficiency. Disable only to chase last-percent latency consistency (raises idle power/heat, reduces boost headroom). |
| Opcache grayout flag | Selection | 0 [0x0] ◀ default 1 [0x1] Display [0x2] ◀ performance NEUTRAL Internal grayout control — no performance impact; leave default. |
| Opcache Control | Selection | Disabled [0x1] Enabled [0x0] Auto [0xFF] ◀ default ◀ performance KEEP Enabled (Auto). The decoded-op cache improves front-end throughput and efficiency; leave on. |
| SEV ASID Count | Selection | 253 ASIDs [0x0] 509 ASIDs [0x1] Auto [0x3] ◀ default ◀ performance NEUTRAL Confidential-compute/encryption control — no performance benefit for general use; small cost when enabled. Leave default. |
| SEV-ES ASID Space Limit Control | Selection | Auto [0x0] ◀ default Manual [0x1] ◀ performance NEUTRAL Confidential-compute/encryption control — no performance benefit for general use; small cost when enabled. Leave default. |
| SEV-ES ASID Space Limit | Numeric | Min 0x1 · Max 0x1FE · Default 0x1 ◀ default ◀ performance NEUTRAL Confidential-compute/encryption control — no performance benefit for general use; small cost when enabled. Leave default. |
| Streaming Stores Control | Selection | Disabled [0x1] Enabled [0x0] Auto [0xFF] ◀ default ◀ performance KEEP Enabled (Auto). Non-temporal stores help bandwidth-bound workloads. |
| Local APIC Mode | Selection | Compatibility [0x0] xAPIC [0x1] x2APIC [0x2] Auto [0xFF] ◀ default ◀ performance KEEP Auto/x2APIC. x2APIC improves interrupt scaling on high-core-count parts; Auto selects correctly. |
| ACPI _CST C1 Declaration | Selection | Disabled [0x0] Enabled [0x1] Auto [0x3] ◀ default ◀ performance KEEP Enabled (Auto). Lets the OS use C1 for efficiency without measurable throughput loss. |
| MCA error thresh enable | Selection | False [0x0] True [0x1] Auto [0xFF] ◀ default ◀ performance NEUTRAL RAS/error-handling control — reliability feature, no performance impact. Leave Auto/default. |
| MCA error thresh count | Numeric | Min 0x1 · Max 0xFFF · Default 0xFF5 ◀ default ◀ performance NEUTRAL RAS/error-handling control — reliability feature, no performance impact. Leave Auto/default. |
| SMU and PSP Debug Mode | Selection | Disabled [0x0] Enabled [0x1] Auto [0x3] ◀ default ◀ performance NEUTRAL RAS/error-handling control — reliability feature, no performance impact. Leave Auto/default. |
| Enhanced REP MOVSB/STOSB (ERSM) | Selection | Disabled [0x0] Enabled [0x1] Auto [0xFF] ◀ default ◀ performance KEEP Enabled (Auto). Faster string-copy microcode path. |
| Log Transparent Errors | Selection | Auto [0x3] ◀ default Disabled [0x0] Enabled [0x1] ◀ performance NEUTRAL RAS/error-handling control — reliability feature, no performance impact. Leave Auto/default. |
| AVX512 | Selection | Disabled [0x0] Enabled [0x1] Auto [0xFF] ◀ default ◀ performance KEEP Enabled (Auto). AVX-512 accelerates vectorized/AI workloads on Zen 5; only disable for thermal-limited edge cases. |
| MONITOR and MWAIT disable | Selection | Enabled [0x1] Disabled [0x0] Auto [0xFF] ◀ default ◀ performance KEEP Keep MWAIT enabled (option Disabled = false). MWAIT-based idle is more efficient and lower-latency than polling. |
| SVM Lock | Selection | Enabled [0x1] Disabled [0x0] Auto [0xFF] ◀ default ◀ performance NEUTRAL Security lock for SVM state — no performance impact; set per security policy. |
| SVM Enable | Selection | Enabled [0x1] Disabled [0x0] Auto [0xFF] ◀ default ◀ performance TUNE Enabled if you run VMs/containers/WSL2 (AMD-V); a few-percent overhead otherwise. Most users: Enabled. |
| Corrector Branch Predictor | Selection | Disable [0x0] ◀ default Enable [0x1] ◀ performance KEEP Enabled (Auto). Improves branch prediction accuracy; leave on. |
| PAUSE Delay | Selection | Auto [0xFF] ◀ default Disable [0x0] 16 cycles [0x1] 32 cycles [0x2] 64 cycles [0x3] 128 cycles [0x4] ◀ performance NEUTRAL Spin-loop hint timing — leave Auto; no general performance benefit. |

### Performance   (Form 0x7008)

Submenus: Custom Core Pstates

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| OC Mode | Selection | Normal Operation [0x0] OC1 [0x1] OC2 [0x2] OC3 [0x3] Customized [0x5] ◀ default ◀ performance TUNE Leave Auto/Default unless you have a deliberate tuning plan. ‘Custom’ unlocks manual P-states (expert-only). |

### Custom Core Pstates   (Form 0x700F)

Information fields: WARNING — DAMAGE CAUSED BY USE OF YOUR AMD PROCESSOR OUTSIDE OF SPECIFICATION OR IN EXCESS OF FACTORY SETTINGS ARE NOT COVERED UNDER YOUR AMD PRODUCT WARRANTY AND MAY NOT BE COVERED BY YOUR SYSTEM MANUFACTURER'S WARRANTY., Operating your AMD processor outside of specification or in excess of factory settings

Submenus: Decline  ‣  Accept

### Accept   (Form 0x7011)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| Custom Pstate0 | Selection | Custom [0x1] Auto [0x2] ◀ default ◀ performance TUNE Expert-only manual P0 definition; leave default unless you know your stable freq/VID pair. |
| Pstate0 Freq (MHz) | Numeric | Min 0x0 · Max 0xFFFFFFFF · Default 0x0 ◀ default ◀ performance TUNE Manual P0 frequency — only set with a validated freq/VID; PBO+CO is safer for gains. |
| Pstate0 VID | Numeric | Min 0x0 · Max 0x1FF · Default 0x0 ◀ default ◀ performance TUNE Manual P0 voltage ID — pair carefully with the frequency; wrong values prevent boot. |

### Prefetcher settings   (Form 0x7009)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| L1 Stream HW Prefetcher | Selection | Disable [0x0] Enable [0x1] Auto [0x3] ◀ default ◀ performance KEEP Enabled (Auto). Hardware prefetch hides memory latency; broadly positive. Disable only if profiling a specific regression. |
| L2 Stream HW Prefetcher | Selection | Disable [0x0] Enable [0x1] Auto [0x3] ◀ default ◀ performance KEEP Enabled (Auto). Improves memory-bound throughput; leave on. |
| L2 Up/Down Prefetcher | Selection | Disable [0x0] Enable [0x1] Auto [0x3] ◀ default ◀ performance KEEP Enabled (Auto). Bidirectional stride prefetch; generally beneficial. |

### Core Watchdog   (Form 0x700A)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| Core Watchdog Timer Enable | Selection | Disabled [0x0] Enabled [0x1] Auto [0x3] ◀ default ◀ performance NEUTRAL Watchdog timer — reliability feature, no performance impact. Leave default. |
| Core Watchdog Timer Interval | Selection | 21.461s [0x900] 10.730s [0x800] 5.364s [0x0] 2.681s [0x100] 1.340s [0x200] 669.41ms [0x300] 334.05ms [0x400] 166.37ms [0x500] 82.53ms [0x600] 40.61ms [0x700] 20.970ms [0x901] 10.484ms [0x801] 5.241ms [0x1] 2.620ms [0x101] 1.309ms [0x201] 654.08us [0x301] 326.4us [0x401] 162.56us [0x501] 80.64us [0x601] 39.68us [0x701] Auto [0xFFFF] ◀ default ◀ performance NEUTRAL Watchdog timer — reliability feature, no performance impact. Leave default. |
| Core Watchdog Timer Severity | Selection | No Error [0x0] Transparent [0x1] Corrected [0x2] Deferred [0x3] Uncorrected [0x4] Fatal [0x5] Auto [0xFF] ◀ default ◀ performance NEUTRAL Watchdog timer — reliability feature, no performance impact. Leave default. |

### DF Common Options   (Form 0x7002)

Submenus: Memory Addressing

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| DF Watchdog Timer Interval | Selection | Auto [0xFF] ◀ default 41 ms [0x0] 166 ms [0x1] 334 ms [0x2] 669 ms [0x3] 1.34 seconds [0x4] 2.68 seconds [0x5] 5.36 seconds [0x6] ◀ performance NEUTRAL Watchdog timer — reliability feature, no performance impact. Leave default. |
| Disable DF to external downstream IP SyncFloodPropagation | Selection | Sync flood disabled [0x1] Sync flood enabled [0x0] Auto [0xFF] ◀ default ◀ performance NEUTRAL RAS/error-handling control — reliability feature, no performance impact. Leave Auto/default. |
| Sync Flood Propagation to DF Components | Selection | Sync flood disabled [0x1] Sync flood enabled [0x0] Auto [0xFF] ◀ default ◀ performance NEUTRAL RAS/error-handling control — reliability feature, no performance impact. Leave Auto/default. |
| Freeze DF module queues on error | Selection | Disabled [0x0] Enabled [0x1] Auto [0x3] ◀ default ◀ performance NEUTRAL RAS/error-handling control — reliability feature, no performance impact. Leave Auto/default. |
| CC6 memory region encryption | Selection | Disabled [0x0] Enabled [0x1] Auto [0x3] ◀ default ◀ performance NEUTRAL Encrypts the CC6 save region (security) — negligible perf impact; leave Auto. |
| System probe filter | Selection | Disabled [0x0] Enabled [0x1] Auto [0x3] ◀ default ◀ performance KEEP Auto/Enabled. Probe filter reduces coherency traffic — helps multi-core memory performance. |
| Disable DF sync flood propagation | Selection | Sync flood disabled [0x1] Sync flood enabled [0x0] Auto [0x3] ◀ default ◀ performance NEUTRAL RAS/error-handling control — reliability feature, no performance impact. Leave Auto/default. |
| DF Cstates | Selection | Disabled [0x0] Enabled [0x1] Auto [0xFF] ◀ default ◀ performance TUNE Disabled for lowest-latency / sustained all-core work (keeps Infinity Fabric out of low-power state); Enabled (Auto) for best idle efficiency. |

### Memory Addressing   (Form 0x7015)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| Memory interleaving | Selection | Disabled [0x0] Auto [0x7] ◀ default ◀ performance KEEP Auto/Enabled. Channel interleaving maximizes memory bandwidth — leave on. |
| Memory interleaving size | Selection | 256 Bytes [0x0] 512 Bytes [0x1] 1 KB [0x2] 2 KB [0x3] Auto [0x7] ◀ default ◀ performance KEEP Auto. Firmware picks the interleave granularity that suits the soldered config. |
| DRAM map inversion | Selection | Disabled [0x0] Enabled [0x1] Auto [0x3] ◀ default ◀ performance NEUTRAL Address-map internal option — leave Auto; no user performance impact. |
| Location of private memory regions | Selection | Distributed [0x0] Consolidated [0x1] Consolidated to 1st DRAM pair [0x2] Auto [0xFF] ◀ default ◀ performance NEUTRAL Reserved-region placement — leave Auto; no performance impact. |
| NUMA Nodes | Selection | NPS1 [0x1] ◀ default NPS2 [0x2] NPS4 [0x4] ◀ performance KEEP Auto (NPS1 / single node). A single unified node is best for this monolithic APU’s latency and simplest for the scheduler. |

### UMC Common Options   (Form 0x7003)

Submenus: LPDDR Options

### LPDDR Options   (Form 0x7019)

Submenus: LPDDR Timing Configuration  ‣  LPDDR Controller Configuration  ‣  LPDDR Bus Configuration  ‣  LPDDR RAS  ‣  LPDDR Security  ‣  LPDDR Addressing Options  ‣  LPDDR Training Options  ‣  LPDDR Memory MBIST

### LPDDR Timing Configuration   (Form 0x701A)

Information fields: WARNING — DAMAGE CAUSED BY USE OF YOUR AMD PROCESSOR OUTSIDE OF SPECIFICATION OR IN EXCESS OF FACTORY SETTINGS ARE NOT COVERED UNDER YOUR AMD PRODUCT WARRANTY AND MAY NOT BE COVERED BY YOUR SYSTEM MANUFACTURER'S WARRANTY., Operating your AMD processor outside of specification or in excess of factory settings

Submenus: Decline  ‣  Accept

### Accept   (Form 0x7023)

Submenus: LPDDR SPD Timing  ‣  LPDDR Non-SPD Timing

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| Active Memory Timing Settings | Selection | Auto [0xFF] ◀ default Enabled [0x1] ◀ performance NEUTRAL Memory test/training diagnostic — not a runtime performance setting. Leave default (Disabled). |
| Maximum Memory Data Clock Speed | Selection | Auto [0xFFFF] ◀ default 1600MT/s [0x640] 3200MT/s [0xC80] 4267MT/s [0x10AB] 5500MT/s [0x157C] 6400MT/s [0x1900] 7500MT/S [0x1D4C] 8000MT/S [0x1F40] ◀ performance TUNE Leave at the SoC default (LPDDR5X-8000 is soldered and pre-tuned). Forcing higher than the validated rate risks training failure with no headroom on soldered memory. |

### LPDDR SPD Timing   (Form 0x7024)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| Trcpage Ctrl | Selection | Auto [0x0] ◀ default Manual [0x1] ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| Trcpage | Numeric | Min 0x0 · Max 0xFFF · Default 0x0 ◀ default ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| Tcwl Ctrl | Selection | Auto [0x0] ◀ default Manual [0x1] ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| Tcwl | Numeric | Min 0x9 · Max 0x16 · Default 0x9 ◀ default ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| Tcl Ctrl | Selection | Auto [0x0] ◀ default Manual [0x1] ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| Tcl | Numeric | Min 0x8 · Max 0x21 · Default 0x8 ◀ default ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| Trcdrd Ctrl | Selection | Auto [0x0] ◀ default Manual [0x1] ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| Trcdrd | Numeric | Min 0x8 · Max 0x1B · Default 0x8 ◀ default ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| Trcdwr Ctrl | Selection | Auto [0x0] ◀ default Manual [0x1] ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| Trcdwr | Numeric | Min 0x8 · Max 0x1B · Default 0x8 ◀ default ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| Trp Ctrl | Selection | Auto [0x0] ◀ default Manual [0x1] ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| Trp | Numeric | Min 0x8 · Max 0x1B · Default 0x8 ◀ default ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| TrpPb Ctrl | Selection | Auto [0x0] ◀ default Manual [0x1] ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| TrpPb | Numeric | Min 0x8 · Max 0x1B · Default 0x8 ◀ default ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| Trfc Ctrl | Selection | Auto [0x0] ◀ default Manual [0x1] ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| Trfc | Numeric | Min 0x3C · Max 0x3DE · Default 0x138 ◀ default ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| TrfcPb Ctrl | Selection | Auto [0x0] ◀ default Manual [0x1] ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| TrfcPb | Numeric | Min 0x3C · Max 0x3DE · Default 0x138 ◀ default ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |

### LPDDR Non-SPD Timing   (Form 0x7025)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| Tras Ctrl | Selection | Auto [0x0] ◀ default Manual [0x1] ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| Tras | Numeric | Min 0x15 · Max 0x3A · Default 0x15 ◀ default ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| Trc Ctrl | Selection | Auto [0x0] ◀ default Manual [0x1] ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| Trc | Numeric | Min 0x1D · Max 0x87 · Default 0x39 ◀ default ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| TrcPb Ctrl | Selection | Auto [0x0] ◀ default Manual [0x1] ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| TrcPb | Numeric | Min 0x1D · Max 0x87 · Default 0x39 ◀ default ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| TrrdS Ctrl | Selection | Auto [0x0] ◀ default Manual [0x1] ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| TrrdS | Numeric | Min 0x4 · Max 0xC · Default 0x4 ◀ default ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| TrrdL Ctrl | Selection | Auto [0x0] ◀ default Manual [0x1] ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| TrrdL | Numeric | Min 0x4 · Max 0xC · Default 0x4 ◀ default ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| Tfaw Ctrl | Selection | Auto [0x0] ◀ default Manual [0x1] ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| Tfaw | Numeric | Min 0x6 · Max 0x36 · Default 0x1A ◀ default ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| TwtrS Ctrl | Selection | Auto [0x0] ◀ default Manual [0x1] ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| TwtrS | Numeric | Min 0x2 · Max 0xE · Default 0x2 ◀ default ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| TwtrL Ctrl | Selection | Auto [0x0] ◀ default Manual [0x1] ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| TwtrL | Numeric | Min 0x2 · Max 0xE · Default 0x2 ◀ default ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| Twr Ctrl | Selection | Auto [0x0] ◀ default Manual [0x1] ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| Twr | Numeric | Min 0xA · Max 0x51 · Default 0x12 ◀ default ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| Trtp Ctrl | Selection | Auto [0x0] ◀ default Manual [0x1] ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| Trtp | Numeric | Min 0x5 · Max 0xE · Default 0x5 ◀ default ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| Tcke Ctrl | Selection | Auto [0x0] ◀ default Manual [0x1] ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| Tcke | Numeric | Min 0x1 · Max 0x1F · Default 0x1 ◀ default ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| TrdrdScL Ctrl | Selection | Auto [0x0] ◀ default Manual [0x1] ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| TrdrdScL | Numeric | Min 0x1 · Max 0xF · Default 0x3 ◀ default ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| TrdrdSc Ctrl | Selection | Auto [0x0] ◀ default Manual [0x1] ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| TrdrdSc | Numeric | Min 0x1 · Max 0xF · Default 0x1 ◀ default ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| TrdrdSd Ctrl | Selection | Auto [0x0] ◀ default Manual [0x1] ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| TrdrdSd | Numeric | Min 0x1 · Max 0xF · Default 0x1 ◀ default ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| TrdrdDd Ctrl | Selection | Auto [0x0] ◀ default Manual [0x1] ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| TrdrdDd | Numeric | Min 0x1 · Max 0xF · Default 0x1 ◀ default ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| TwrwrScL Ctrl | Selection | Auto [0x0] ◀ default Manual [0x1] ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| TwrwrScL | Numeric | Min 0x1 · Max 0x3F · Default 0x3 ◀ default ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| TwrwrSc Ctrl | Selection | Auto [0x0] ◀ default Manual [0x1] ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| TwrwrSc | Numeric | Min 0x1 · Max 0xF · Default 0x1 ◀ default ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| TwrwrSd Ctrl | Selection | Auto [0x0] ◀ default Manual [0x1] ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| TwrwrSd | Numeric | Min 0x1 · Max 0xF · Default 0x1 ◀ default ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| TwrwrDd Ctrl | Selection | Auto [0x0] ◀ default Manual [0x1] ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| TwrwrDd | Numeric | Min 0x1 · Max 0xF · Default 0x1 ◀ default ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| Twrrd Ctrl | Selection | Auto [0x0] ◀ default Manual [0x1] ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| Twrrd | Numeric | Min 0x1 · Max 0xF · Default 0x1 ◀ default ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| Trdwr Ctrl | Selection | Auto [0x0] ◀ default Manual [0x1] ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| Trdwr | Numeric | Min 0x1 · Max 0x1F · Default 0x1 ◀ default ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |

### LPDDR Controller Configuration   (Form 0x701B)

Submenus: LPDDR Power Options

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| LPDDR Refresh Mode | Selection | Auto [0xFF] ◀ default All Banks [0x1] Per Bank [0x2] ◀ performance KEEP Auto. Refresh policy is tuned for the soldered LPDDR5X; leave default. |
| LPDDR RFM | Selection | Disabled [0x0] Enabled [0x1] Auto [0xFF] ◀ default ◀ performance KEEP Auto. Refresh-management feature — leave to firmware. |
| WCK Always On | Selection | Disabled [0x0] Enabled [0x1] Auto [0xFF] ◀ default ◀ performance NEUTRAL Keeping the write clock always on trades idle power for marginal latency; leave Auto. |
| DVFSC Mode | Selection | Disabled [0x0] Enabled [0x1] Auto [0xFF] ◀ default ◀ performance KEEP Auto. Dynamic memory voltage/freq scaling — leave to firmware. |
| RRW Memory Test Control | Selection | Auto [0xFF] ◀ default Enabled [0x1] Disabled [0x0] ◀ performance NEUTRAL Memory test/training diagnostic — not a runtime performance setting. Leave default (Disabled). |

### LPDDR Power Options   (Form 0x7026)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| Power Down Enable | Selection | Disabled [0x0] Enabled [0x1] Auto [0xFF] ◀ default ◀ performance TUNE Disabled can trim memory latency slightly under load; Enabled (Auto) saves idle power. Default is fine for most. |
| Phy Low Power Disable | Selection | Auto [0xFF] ◀ default 0 [0x0] 1 [0x1] ◀ performance NEUTRAL Memory test/training diagnostic — not a runtime performance setting. Leave default (Disabled). |

### LPDDR Bus Configuration   (Form 0x701C)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| CA drive strengths | Selection | Auto [0xFF] ◀ default 120.0 Ohm [0x78] 60.0 Ohm [0x3C] 40.0 Ohm [0x28] ◀ performance NEUTRAL Memory signal-integrity (drive strength / ODT) — auto-trained for the soldered LPDDR5X. Manual changes are expert-only and risk training failure; leave Auto. |
| CK drive strengths | Selection | Auto [0xFF] ◀ default 120.0 Ohm [0x78] 60.0 Ohm [0x3C] 40.0 Ohm [0x28] ◀ performance NEUTRAL Memory signal-integrity (drive strength / ODT) — auto-trained for the soldered LPDDR5X. Manual changes are expert-only and risk training failure; leave Auto. |
| DQ drive strengths | Selection | Auto [0xFF] ◀ default 120.0 Ohm [0x78] 60.0 Ohm [0x3C] 40.0 Ohm [0x28] ◀ performance NEUTRAL Memory signal-integrity (drive strength / ODT) — auto-trained for the soldered LPDDR5X. Manual changes are expert-only and risk training failure; leave Auto. |
| DQS drive strengths | Selection | Auto [0xFF] ◀ default 120.0 Ohm [0x78] 60.0 Ohm [0x3C] 40.0 Ohm [0x28] ◀ performance NEUTRAL Memory signal-integrity (drive strength / ODT) — auto-trained for the soldered LPDDR5X. Manual changes are expert-only and risk training failure; leave Auto. |
| WCK drive strengths | Selection | Auto [0xFF] ◀ default 120.0 Ohm [0x78] 60.0 Ohm [0x3C] 40.0 Ohm [0x28] ◀ performance NEUTRAL Memory signal-integrity (drive strength / ODT) — auto-trained for the soldered LPDDR5X. Manual changes are expert-only and risk training failure; leave Auto. |
| Processor CA ODT impedance | Selection | Auto [0xFF] ◀ default 120.0 Ohm [0x78] 60.0 Ohm [0x3C] 40.0 Ohm [0x28] Disable [0x0] ◀ performance NEUTRAL Memory signal-integrity (drive strength / ODT) — auto-trained for the soldered LPDDR5X. Manual changes are expert-only and risk training failure; leave Auto. |
| Processor CK ODT impedance | Selection | Auto [0xFF] ◀ default 120.0 Ohm [0x78] 60.0 Ohm [0x3C] 40.0 Ohm [0x28] Disable [0x0] ◀ performance NEUTRAL Memory signal-integrity (drive strength / ODT) — auto-trained for the soldered LPDDR5X. Manual changes are expert-only and risk training failure; leave Auto. |
| Processor CS ODT impedance | Selection | Auto [0xFF] ◀ default 120.0 Ohm [0x78] 60.0 Ohm [0x3C] 40.0 Ohm [0x28] Disable [0x0] ◀ performance NEUTRAL Memory signal-integrity (drive strength / ODT) — auto-trained for the soldered LPDDR5X. Manual changes are expert-only and risk training failure; leave Auto. |
| Processor DQ ODT impedance | Selection | Auto [0xFF] ◀ default 120.0 Ohm [0x78] 60.0 Ohm [0x3C] 40.0 Ohm [0x28] Disable [0x0] ◀ performance NEUTRAL Memory signal-integrity (drive strength / ODT) — auto-trained for the soldered LPDDR5X. Manual changes are expert-only and risk training failure; leave Auto. |
| Processor DQS ODT impedance | Selection | Auto [0xFF] ◀ default 120.0 Ohm [0x78] 60.0 Ohm [0x3C] 40.0 Ohm [0x28] Disable [0x0] ◀ performance NEUTRAL Memory signal-integrity (drive strength / ODT) — auto-trained for the soldered LPDDR5X. Manual changes are expert-only and risk training failure; leave Auto. |
| Processor WCK ODT impedance | Selection | Auto [0xFF] ◀ default 120.0 Ohm [0x78] 60.0 Ohm [0x3C] 40.0 Ohm [0x28] Disable [0x0] ◀ performance NEUTRAL Memory signal-integrity (drive strength / ODT) — auto-trained for the soldered LPDDR5X. Manual changes are expert-only and risk training failure; leave Auto. |
| Dram CA ODT impedance | Selection | Auto [0xFF] ◀ default Disable [0x0] RZQ/1 [0x1] RZQ/2 [0x2] RZQ/3 [0x3] RZQ/4 [0x4] RZQ/5 [0x5] RZQ/6 [0x6] ◀ performance NEUTRAL Memory signal-integrity (drive strength / ODT) — auto-trained for the soldered LPDDR5X. Manual changes are expert-only and risk training failure; leave Auto. |
| Dram CS ODT impedance | Selection | Auto [0xFF] ◀ default Disable [0x0] RZQ/1 [0x1] RZQ/2 [0x2] RZQ/3 [0x3] ◀ performance NEUTRAL Memory signal-integrity (drive strength / ODT) — auto-trained for the soldered LPDDR5X. Manual changes are expert-only and risk training failure; leave Auto. |
| Dram DQ ODT impedance | Selection | Auto [0xFF] ◀ default Disable [0x0] RZQ/1 [0x1] RZQ/2 [0x2] RZQ/3 [0x3] RZQ/4 [0x4] RZQ/5 [0x5] RZQ/6 [0x6] ◀ performance NEUTRAL Memory signal-integrity (drive strength / ODT) — auto-trained for the soldered LPDDR5X. Manual changes are expert-only and risk training failure; leave Auto. |
| Dram WCK ODT impedance | Selection | Auto [0xFF] ◀ default Disable [0x0] RZQ/1 [0x1] RZQ/2 [0x2] RZQ/3 [0x3] RZQ/4 [0x4] RZQ/5 [0x5] RZQ/6 [0x6] ◀ performance NEUTRAL Memory signal-integrity (drive strength / ODT) — auto-trained for the soldered LPDDR5X. Manual changes are expert-only and risk training failure; leave Auto. |
| Dram Non-Target ODT impedance | Selection | Auto [0xFF] ◀ default Disable [0x0] RZQ/1 [0x1] RZQ/2 [0x2] RZQ/3 [0x3] RZQ/4 [0x4] RZQ/5 [0x5] RZQ/6 [0x6] ◀ performance NEUTRAL Memory signal-integrity (drive strength / ODT) — auto-trained for the soldered LPDDR5X. Manual changes are expert-only and risk training failure; leave Auto. |
| Dram Pull-Down drive strengths | Selection | Auto [0xFF] ◀ default Disable [0x0] RZQ/1 [0x1] RZQ/2 [0x2] RZQ/3 [0x3] RZQ/4 [0x4] RZQ/5 [0x5] RZQ/6 [0x6] ◀ performance NEUTRAL Memory signal-integrity (drive strength / ODT) — auto-trained for the soldered LPDDR5X. Manual changes are expert-only and risk training failure; leave Auto. |

### LPDDR RAS   (Form 0x701D)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| DRAM Read Link ECC Enable | Selection | Disabled [0x0] Enabled [0x1] Auto [0xFF] ◀ default ◀ performance TUNE Link ECC adds reliability with a small bandwidth cost. Disabled for maximum memory throughput; Enabled for data integrity. Default is fine. |
| DRAM Write Link ECC Enable | Selection | Disabled [0x0] Enabled [0x1] Auto [0xFF] ◀ default ◀ performance TUNE Link ECC adds reliability with a small bandwidth cost. Disabled for maximum memory throughput; Enabled for data integrity. Default is fine. |
| Disable Memory Error Injection | Selection | False [0x0] True [0x1] Auto [0xFF] ◀ default ◀ performance NEUTRAL Memory test/training diagnostic — not a runtime performance setting. Leave default (Disabled). |

### LPDDR Security   (Form 0x701E)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| TSME | Selection | Auto [0xFF] ◀ default Enabled [0x1] Disabled [0x0] ◀ performance TUNE Transparent SME (full-memory encryption) adds a small bandwidth/latency cost. Disabled for maximum memory performance; Enabled for at-rest RAM encryption. |
| Data Scramble | Selection | Enabled [0x1] Disabled [0x0] Auto [0xFF] ◀ default ◀ performance NEUTRAL Signal-integrity scrambling — leave Auto; no measurable performance effect. |

### LPDDR Addressing Options   (Form 0x701F)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| Chip Select Interleaving | Selection | Disabled [0x0] Auto [0xFF] ◀ default ◀ performance KEEP Auto/Enabled. Interleaving across chip selects aids bandwidth. |
| Bank Swap | Selection | Disabled [0x0] Enabled [0x1] Auto [0xFF] ◀ default ◀ performance KEEP Auto. Bank remapping for access spreading — leave to firmware. |
| BankGroup Swap | Selection | Disabled [0x0] Enabled [0x2] Auto [0xFF] ◀ default ◀ performance KEEP Auto. Bank-group swap can improve access parallelism; leave default. |
| Address Hash Bank | Selection | Disabled [0x0] Enabled [0x1] Auto [0xFF] ◀ default ◀ performance KEEP Auto/Enabled. Address hashing reduces bank conflicts — beneficial for bandwidth. |
| Address Hash CS | Selection | Auto [0xFF] ◀ default Enabled [0x1] Disabled [0x0] ◀ performance KEEP Auto/Enabled. Chip-select hashing spreads accesses — leave on. |

### LPDDR Training Options   (Form 0x7020)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| DFE Read Training | Selection | Auto [0xFF] ◀ default Enable [0x1] Disable [0x0] ◀ performance NEUTRAL Memory test/training diagnostic — not a runtime performance setting. Leave default (Disabled). |
| DFE Write Training | Selection | Auto [0xFF] ◀ default Enable [0x1] Disable [0x0] ◀ performance NEUTRAL Memory test/training diagnostic — not a runtime performance setting. Leave default (Disabled). |

### LPDDR Memory MBIST   (Form 0x7021)

Submenus: LPDDR Data Eye

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| MBIST Enable | Selection | Disabled [0x0] Enabled [0x1] Auto [0xFF] ◀ default ◀ performance NEUTRAL Memory test/training diagnostic — not a runtime performance setting. Leave default (Disabled). |
| MBIST Test Mode | Selection | Interface Mode [0x0] Data Eye Mode [0x1] Both [0x2] Auto [0xFF] ◀ default ◀ performance NEUTRAL Memory test/training diagnostic — not a runtime performance setting. Leave default (Disabled). |
| MBIST Aggressors | Selection | Disabled [0x0] Enabled [0x1] Auto [0xFF] ◀ default ◀ performance NEUTRAL Memory test/training diagnostic — not a runtime performance setting. Leave default (Disabled). |

### LPDDR Data Eye   (Form 0x7029)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| Pattern Select | Selection | Auto [0xFF] ◀ default PRBS [0x0] SSO [0x1] Both [0x2] ◀ performance NEUTRAL Memory test/training diagnostic — not a runtime performance setting. Leave default (Disabled). |
| Pattern Length Select | Selection | Auto [0xFF] ◀ default Manual [0x1] ◀ performance NEUTRAL Memory test/training diagnostic — not a runtime performance setting. Leave default (Disabled). |
| Pattern Length | Numeric | Min 0x3 · Max 0xC · Default 0x3 ◀ default ◀ performance NEUTRAL Memory test/training diagnostic — not a runtime performance setting. Leave default (Disabled). |
| Aggressor Channel | Selection | Auto [0xFF] ◀ default Disabled [0x0] 1 Aggressor Channel [0x1] 3 Aggressor Channels [0x3] 7 Aggressor Channels [0x7] ◀ performance NEUTRAL Memory test/training diagnostic — not a runtime performance setting. Leave default (Disabled). |
| Read Voltage Sweep Step Size | Selection | Auto [0xFF] ◀ default 1 [0x0] 2 [0x1] 4 [0x4] ◀ performance NEUTRAL Memory test/training diagnostic — not a runtime performance setting. Leave default (Disabled). |
| Read Timing Sweep Step Size | Selection | Auto [0xFF] ◀ default 1 [0x1] 2 [0x2] 4 [0x4] ◀ performance NEUTRAL Memory test/training diagnostic — not a runtime performance setting. Leave default (Disabled). |
| Write Voltage Sweep Step Size | Selection | Auto [0xFF] ◀ default 1 [0x0] 2 [0x1] 4 [0x4] ◀ performance NEUTRAL Memory test/training diagnostic — not a runtime performance setting. Leave default (Disabled). |
| Write Timing Sweep Step Size | Selection | Auto [0xFF] ◀ default 1 [0x1] 2 [0x2] 4 [0x4] ◀ performance NEUTRAL Memory test/training diagnostic — not a runtime performance setting. Leave default (Disabled). |

### NBIO Common Options   (Form 0x7004)

Submenus: GFX Configuration  ‣  Audio Configuration

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| IOMMU | Selection | Disabled [0x0] Enabled [0x1] Auto [0xF] ◀ default ◀ performance TUNE Disabled for max iGPU memory-read bandwidth (~6% better on Strix Halo) and lowest latency; Enabled only for VFIO/GPU-passthrough or device isolation. |
| DMAr Support | Selection | Auto [0xF] ◀ default Enabled [0x1] Disabled [0x0] ◀ performance NEUTRAL DMA remapping (tied to IOMMU) — leave default unless using passthrough; slight perf cost when on. |
| DMA Protection | Selection | Auto [0xF] ◀ default Enabled [0x1] Disabled [0x0] ◀ performance NEUTRAL Kernel DMA protection (security) — no throughput benefit; set per security needs. |
| SCPC attribute control | Selection | 0 [0x0] 1 [0x1] 2 [0x2] 3 [0x3] Customized [0xFF] ◀ default ◀ performance NEUTRAL Internal PCIe attribute — leave Auto; no user performance impact. |
| PCIe ARI Support | Selection | Disabled [0x0] Enabled [0x1] Auto [0xF] ◀ default ◀ performance KEEP Enabled (Auto). Alternative Routing-ID improves multi-function PCIe device support; leave on. |
| PCIe loopback Mode | Selection | Auto [0xF] ◀ default Disabled [0x0] Enabled [0x1] ◀ performance NEUTRAL Diagnostic loopback — keep Disabled; not a performance setting. |
| PSPP Policy | Selection | Auto [0xF] ◀ default Disabled [0x0] Balanced [0x2] ◀ performance TUNE Balanced (Auto) for daily; set to Disabled/Performance to keep PCIe links at full speed (lower latency, higher idle power). |

### GFX Configuration   (Form 0x702C)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| iGPU Configuration | Selection | Auto [0xF] iGPU Disabled [0x0] UMA_SPECIFIED [0x1] ◀ default UMA_AUTO [0x2] UMA_GAME_OPTIMIZED [0x3] UMA_WORKSTATION_OPTIMIZED [0x4] ◀ performance KEEP Enabled (Auto/UMA). The Radeon 8060S iGPU is the only graphics on this APU — must stay enabled. |
| iGPU Mem Bar Configuration | Selection | Auto [0xF] ◀ default LegacyGfxBAR [0x1] LargeBAR [0x2] ResizableBAR [0x3] ◀ performance KEEP Leave Auto. Large iGPU BAR pairs with Above-4G/ReBAR for full-aperture access. |
| UMA Version | Selection | Legacy [0x0] Non-Legacy [0x1] Auto [0xF] ◀ default ◀ performance NEUTRAL UMA descriptor version — informational; no performance choice. |
| UMA Frame buffer Size | Selection | 512M [0x200] 4G [0x1000] 8G [0x2000] 16G [0x4000] 32G [0x8000] 64G [0x10000] 96G [0x18000] ◀ default ◀ performance TUNE Workload-dependent. Windows gaming/AI: large carve-out (e.g. 64-96 GB) for guaranteed VRAM. Linux LLM (Vulkan/GTT) or general use: minimum (512 MB) so the OS sees full RAM — GTT still gives the GPU the whole pool with no speed loss. |
| GPU Host Translation Cache | Selection | Disabled [0x0] Enabled [0x1] Auto [0xF] ◀ default ◀ performance KEEP Enabled (Auto). Caches GPU address translations — helps iGPU memory-access latency. |
| TCON INSTANT ON LOGO | Selection | Disabled [0x0] Enabled [0x1] Auto [0xF] ◀ default ◀ performance NEUTRAL Display/panel/LED configuration — no compute-performance impact. |
| Dedicated Graphics Memory Auto Level | Selection | UmaSizeMinimum [0x0] ◀ default UmaSizeMedium [0x1] UmaSizeHigh [0x2] ◀ performance NEUTRAL Auto-sizing helper for the UMA carve-out — tune via UMA Frame Buffer Size instead. |
| Remaining System Memory | Selection | RemainGB [0x0] ◀ default ◀ performance NEUTRAL Read-only readout of RAM left after the GPU carve-out — informational. |

### Audio Configuration   (Form 0x702D)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| NB Azalia | Selection | Disabled [0x0] Enabled [0x1] Auto [0xF] ◀ default ◀ performance NEUTRAL Onboard HD-audio controller enable — no compute-performance impact; set per need. |
| Audio IOs | Selection | Auto [0xFF] ◀ default HDA(3SDI) [0x1] HDA(1SDI) + SW0(1MDATA) [0x2] SW0(4MDATA) + SW1(1MDATA) [0x3] HDA(3SDI) + PDM(2CH) [0x4] HDA(1SDI) + PDM(6CH) [0x5] HDA(1SDI) + SW0(1MDATA) + PDM(2CH) [0x6] SW0(4MDATA) + PDM(6CH) [0x7] SW0(4MDATA) + SW1(1MDATA) + PDM(2CH) [0x8] 3I2S + 1 REFCLK + 1 INTR [0x9] HDA(3SDI) + PDM(6CH) +I2S [0xA] HDA(3SDI) + PDM(8CH) [0xB] HDA(1SDI) + SW0(1MDATA) + PDM(6CH) + I2S [0xC] SW0(4MDATA)+ SW1(1MDATA) +PDM(6CH) + I2S [0xD] SW0(4MDATA) + SW1(1MDATA)+ PDM(8CH) [0xE] LPFLL Clock out [0xF] SW0(3MDATA) + SW1(2MDATA) [0x10] SW0(3MDATA) + SW1(2MDATA) + PDM(2CH) [0x11] SW0(3MDATA) + SW1(2MDATA) + PDM(8CH) [0x12] SW0(3MDATA) + SW1(2MDATA) + PDM(6CH) + I2S [0x13] 2I2S + 1 INTR + PDM(4CH) [0x14] ◀ performance NEUTRAL Audio pin config — no performance impact. |

### FCH Common Options   (Form 0x7005)

Submenus: I3C/I2C Configuration Options  ‣  USB Configuration Options  ‣  Ac Power Loss Options  ‣  Uart Configuration Options  ‣  USB4 Configuration Options  ‣  SPI Configuration Options

### I3C/I2C Configuration Options   (Form 0x7037)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| I3C/I2C 0 Enable | Selection | Both Disabled [0x3] I3C Enabled [0x0] I2C Enabled [0x1] Auto [0xF] ◀ default ◀ performance NEUTRAL Peripheral/sensor enable — no compute-performance impact; set per hardware need. |
| I3C 0 Mode | Selection | I3C [0x0] I2C [0x1] Auto [0xF] ◀ default ◀ performance NEUTRAL Peripheral/sensor enable — no compute-performance impact; set per hardware need. |
| I3C/I2C 1 Enable | Selection | Both Disabled [0x3] I3C Enabled [0x0] I2C Enabled [0x1] Auto [0xF] ◀ default ◀ performance NEUTRAL Peripheral/sensor enable — no compute-performance impact; set per hardware need. |
| I3C 1 Mode | Selection | I3C [0x0] I2C [0x1] Auto [0xF] ◀ default ◀ performance NEUTRAL Peripheral/sensor enable — no compute-performance impact; set per hardware need. |
| I3C/I2C 2 Enable | Selection | Both Disabled [0x3] I3C Enabled [0x0] I2C Enabled [0x1] Auto [0xF] ◀ default ◀ performance NEUTRAL Peripheral/sensor enable — no compute-performance impact; set per hardware need. |
| I3C 2 Mode | Selection | I3C [0x0] I2C [0x1] Auto [0xF] ◀ default ◀ performance NEUTRAL Peripheral/sensor enable — no compute-performance impact; set per hardware need. |
| I3C/I2C 3 Enable | Selection | Both Disabled [0x3] I3C Enabled [0x0] I2C Enabled [0x1] Auto [0xF] ◀ default ◀ performance NEUTRAL Peripheral/sensor enable — no compute-performance impact; set per hardware need. |
| I3C 3 Mode | Selection | I3C [0x0] I2C [0x1] Auto [0xF] ◀ default ◀ performance NEUTRAL Peripheral/sensor enable — no compute-performance impact; set per hardware need. |

### USB Configuration Options   (Form 0x7038)

Submenus: USB0 2.0 port enable  ‣  USB1 2.0 port enable  ‣  USB2 2.0 port enable  ‣  USB3 2.0 port enable  ‣  USB0 3.1 port enable  ‣  USB1 3.1 port enable  ‣  USB2 3.1 port enable  ‣  USB3 3.1 port enable

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| USB0 controller enable | Selection | Enabled [0x1] Disabled [0x0] Auto [0xF] ◀ default ◀ performance NEUTRAL USB port/controller enable — functional toggle, no performance impact. Leave default unless disabling unused ports for security/power. |
| USB1 controller enable | Selection | Enabled [0x1] Disabled [0x0] Auto [0xF] ◀ default ◀ performance NEUTRAL USB port/controller enable — functional toggle, no performance impact. Leave default unless disabling unused ports for security/power. |

### USB0 2.0 port enable   (Form 0x703D)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| USB0 2.0 Port 0 | Selection | Disabled [0x0] Enabled [0x1] Auto [0xF] ◀ default ◀ performance NEUTRAL USB port/controller enable — functional toggle, no performance impact. Leave default unless disabling unused ports for security/power. |
| USB0 2.0 Port 1 | Selection | Disabled [0x0] Enabled [0x1] Auto [0xF] ◀ default ◀ performance NEUTRAL USB port/controller enable — functional toggle, no performance impact. Leave default unless disabling unused ports for security/power. |
| USB0 2.0 Port 2 | Selection | Disabled [0x0] Enabled [0x1] Auto [0xF] ◀ default ◀ performance NEUTRAL USB port/controller enable — functional toggle, no performance impact. Leave default unless disabling unused ports for security/power. |
| USB0 2.0 Port 3 | Selection | Disabled [0x0] Enabled [0x1] Auto [0xF] ◀ default ◀ performance NEUTRAL USB port/controller enable — functional toggle, no performance impact. Leave default unless disabling unused ports for security/power. |
| USB0 2.0 Port 4 | Selection | Disabled [0x0] Enabled [0x1] Auto [0xF] ◀ default ◀ performance NEUTRAL USB port/controller enable — functional toggle, no performance impact. Leave default unless disabling unused ports for security/power. |

### USB1 2.0 port enable   (Form 0x703E)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| USB1 2.0 Port 0 | Selection | Disabled [0x0] Enabled [0x1] Auto [0xF] ◀ default ◀ performance NEUTRAL USB port/controller enable — functional toggle, no performance impact. Leave default unless disabling unused ports for security/power. |

### USB2 2.0 port enable   (Form 0x703F)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| USB2 2.0 Port 0 | Selection | Disabled [0x0] Enabled [0x1] Auto [0xF] ◀ default ◀ performance NEUTRAL USB port/controller enable — functional toggle, no performance impact. Leave default unless disabling unused ports for security/power. |

### USB3 2.0 port enable   (Form 0x7040)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| USB3 2.0 Port 0 | Selection | Disabled [0x0] Enabled [0x1] Auto [0xF] ◀ default ◀ performance NEUTRAL USB port/controller enable — functional toggle, no performance impact. Leave default unless disabling unused ports for security/power. |

### USB0 3.1 port enable   (Form 0x7041)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| USB0 3.1 Port 0 | Selection | Disabled [0x0] Enabled [0x1] Auto [0xF] ◀ default ◀ performance NEUTRAL USB port/controller enable — functional toggle, no performance impact. Leave default unless disabling unused ports for security/power. |
| USB0 3.1 Port 1 | Selection | Disabled [0x0] Enabled [0x1] Auto [0xF] ◀ default ◀ performance NEUTRAL USB port/controller enable — functional toggle, no performance impact. Leave default unless disabling unused ports for security/power. |

### USB1 3.1 port enable   (Form 0x7042)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| USB1 3.1 Port 0 | Selection | Disabled [0x0] Enabled [0x1] Auto [0xF] ◀ default ◀ performance NEUTRAL USB port/controller enable — functional toggle, no performance impact. Leave default unless disabling unused ports for security/power. |

### USB2 3.1 port enable   (Form 0x7043)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| USB2 3.1 Port 0 | Selection | Disabled [0x0] Enabled [0x1] Auto [0xF] ◀ default ◀ performance NEUTRAL USB port/controller enable — functional toggle, no performance impact. Leave default unless disabling unused ports for security/power. |

### USB3 3.1 port enable   (Form 0x7044)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| USB3 3.1 Port 0 | Selection | Disabled [0x0] Enabled [0x1] Auto [0xF] ◀ default ◀ performance NEUTRAL USB port/controller enable — functional toggle, no performance impact. Leave default unless disabling unused ports for security/power. |

### Ac Power Loss Options   (Form 0x7039)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| Ac Loss Control | Selection | Always Off [0x0] ◀ default Always On [0x1] Last State [0x3] ◀ performance NEUTRAL Power-on-after-AC-loss behavior — operational preference; no performance impact. |

### Uart Configuration Options   (Form 0x703A)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| Uart 0 Enable | Selection | Disabled [0x0] Enabled [0x1] Auto [0xF] ◀ default ◀ performance NEUTRAL Serial/debug interface — leave Disabled/default. Debug output can slightly slow boot; no runtime performance benefit. |
| Uart 0 Legacy Options | Selection | Disabled [0x0] 0x2E8 [0x1] 0x2F8 [0x2] 0x3E8 [0x3] 0x3F8 [0x4] Auto [0xF] ◀ default ◀ performance NEUTRAL Serial/debug interface — leave Disabled/default. Debug output can slightly slow boot; no runtime performance benefit. |
| Uart 1 Enable (no HW FC) | Selection | Disabled [0x0] Enabled [0x1] Auto [0xF] ◀ default ◀ performance NEUTRAL Serial/debug interface — leave Disabled/default. Debug output can slightly slow boot; no runtime performance benefit. |
| Uart 1 Legacy Options | Selection | Disabled [0x0] 0x2E8 [0x1] 0x2F8 [0x2] 0x3E8 [0x3] 0x3F8 [0x4] Auto [0xF] ◀ default ◀ performance NEUTRAL Serial/debug interface — leave Disabled/default. Debug output can slightly slow boot; no runtime performance benefit. |
| Uart 2 Enable | Selection | Disabled [0x0] Enabled [0x1] Auto [0xF] ◀ default ◀ performance NEUTRAL Serial/debug interface — leave Disabled/default. Debug output can slightly slow boot; no runtime performance benefit. |
| Uart 2 Legacy Options | Selection | Disabled [0x0] 0x2E8 [0x1] 0x2F8 [0x2] 0x3E8 [0x3] 0x3F8 [0x4] Auto [0xF] ◀ default ◀ performance NEUTRAL Serial/debug interface — leave Disabled/default. Debug output can slightly slow boot; no runtime performance benefit. |
| Uart 3 Enable (no HW FC) | Selection | Disabled [0x0] Enabled [0x1] Auto [0xF] ◀ default ◀ performance NEUTRAL Serial/debug interface — leave Disabled/default. Debug output can slightly slow boot; no runtime performance benefit. |
| Uart 3 Legacy Options | Selection | Disabled [0x0] 0x2E8 [0x1] 0x2F8 [0x2] 0x3E8 [0x3] 0x3F8 [0x4] Auto [0xF] ◀ default ◀ performance NEUTRAL Serial/debug interface — leave Disabled/default. Debug output can slightly slow boot; no runtime performance benefit. |
| Uart 4 Enable | Selection | Disabled [0x0] Enabled [0x1] Auto [0xF] ◀ default ◀ performance NEUTRAL Serial/debug interface — leave Disabled/default. Debug output can slightly slow boot; no runtime performance benefit. |

### USB4 Configuration Options   (Form 0x703B)

Submenus: USB4 RT0  ‣  USB4 RT1

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| USB4 pre-CM Enable | Selection | Disabled [0x0] Enabled [0x1] Auto [0xF] ◀ default ◀ performance NEUTRAL USB port/controller enable — functional toggle, no performance impact. Leave default unless disabling unused ports for security/power. |

### USB4 RT0   (Form 0x7045)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| RT0 Router Enable | Selection | Disabled [0x0] Enabled [0x1] Auto [0xF] ◀ default ◀ performance NEUTRAL USB4/Thunderbolt resource reservation — leave default unless a specific TB device needs more PCIe window; no general perf gain. |
| RT0 PCIe Tunnel | Selection | Disabled [0x0] Enabled [0x1] Auto [0xF] ◀ default ◀ performance NEUTRAL USB4/Thunderbolt resource reservation — leave default unless a specific TB device needs more PCIe window; no general perf gain. |

### USB4 RT1   (Form 0x7046)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| RT1 Router Enable | Selection | Disabled [0x0] Enabled [0x1] Auto [0xF] ◀ default ◀ performance NEUTRAL USB4/Thunderbolt resource reservation — leave default unless a specific TB device needs more PCIe window; no general perf gain. |
| RT1 PCIe Tunnel | Selection | Disabled [0x0] Enabled [0x1] Auto [0xF] ◀ default ◀ performance NEUTRAL USB4/Thunderbolt resource reservation — leave default unless a specific TB device needs more PCIe window; no general perf gain. |

### SPI Configuration Options   (Form 0x703C)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| HFP Enable | Selection | Disabled [0x0] Enabled [0x1] Auto [0xF] ◀ default ◀ performance NEUTRAL Embedded HID/SPI interface — no general compute-performance impact; leave default. |
| HID Enable | Selection | Disabled [0x0] Enabled [0x1] Auto [0xF] ◀ default ◀ performance NEUTRAL Embedded HID/SPI interface — no general compute-performance impact; leave default. |
| HID2 Enable | Selection | Disabled [0x0] Enabled [0x1] Auto [0xF] ◀ default ◀ performance NEUTRAL Embedded HID/SPI interface — no general compute-performance impact; leave default. |
| HID Spi Read Mode | Selection | Quad IO 1-4-4 [0x5] ◀ default Dual IO 1-2-2 [0x4] Fast Read 1-1-1 [0x7] ◀ performance NEUTRAL Embedded HID/SPI interface — no general compute-performance impact; leave default. |
| HID Spi Speed | Selection | (1)33HMz [0x1] ◀ default (2)22NMz [0x2] (3)17.6Mhz [0x3] (5)800Khz [0x5] spi_spd6 [0x6] spi_spd7 [0x7] ◀ performance NEUTRAL Embedded HID/SPI interface — no general compute-performance impact; leave default. |
| HID Spi Speed6 N | Numeric | Min 0x4 · Max 0x3F · Default 0x4 ◀ default ◀ performance NEUTRAL Embedded HID/SPI interface — no general compute-performance impact; leave default. |
| HID Spi Speed7 N | Numeric | Min 0x4 · Max 0x3F · Default 0x32 ◀ default ◀ performance NEUTRAL Embedded HID/SPI interface — no general compute-performance impact; leave default. |
| HID2 Spi Read Mode | Selection | Quad IO 1-4-4 [0x5] ◀ default Dual IO 1-2-2 [0x4] Fast Read 1-1-1 [0x7] ◀ performance NEUTRAL Embedded HID/SPI interface — no general compute-performance impact; leave default. |
| HID2 Spi Speed | Selection | (1)33HMz [0x1] ◀ default (2)22NMz [0x2] (3)17.6Mhz [0x3] (5)800Khz [0x5] spi_spd6 [0x6] spi_spd7 [0x7] ◀ performance NEUTRAL Embedded HID/SPI interface — no general compute-performance impact; leave default. |
| HID2 Spi Speed6 N | Numeric | Min 0x4 · Max 0x3F · Default 0x4 ◀ default ◀ performance NEUTRAL Embedded HID/SPI interface — no general compute-performance impact; leave default. |
| HID2 Spi Speed7 N | Numeric | Min 0x4 · Max 0x3F · Default 0x32 ◀ default ◀ performance NEUTRAL Embedded HID/SPI interface — no general compute-performance impact; leave default. |

### SMU Common Options   (Form 0x7006)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| System Configuration | Selection | 15W [0xE] 28W [0xF] 30W [0x6] 35W [0x7] 45W [0x4] 54W [0x9] 65W [0xA] Auto [0xF] 55W [0x1] 85W [0x2] 120W [0x3] ◀ default 140W [0x5] ◀ performance TUNE Select the highest power profile available (e.g. ‘Performance’/140 W) for max sustained clocks; lower profiles cap power for quiet/cool operation. |
| ECO Mode | Selection | Auto [0xF] ◀ default Enabled [0x1] Disabled [0x0] ◀ performance TUNE Disabled for maximum performance. ECO caps APU power for efficiency/quiet at the cost of sustained clocks. |
| SPL Control | Selection | Manual [0x1] Auto [0x0] ◀ default ◀ performance TUNE Manual to raise the Sustained Power Limit; higher SPL = more sustained all-core/iGPU clock within thermal limits. |
| Sustained Power Limit | Numeric | Min 0x0 · Max 0xFFFFFFFF · Default 0x0 ◀ default ◀ performance TUNE Raise toward the GTR9 Pro ceiling (~140 W) for best sustained throughput; bounded by cooling and skin-temp limits. |
| PPT Control | Selection | Manual [0x1] Auto [0x0] ◀ default ◀ performance TUNE Manual to lift Fast/Slow PPT limits for more boost headroom (cooling permitting). |
| Fast PPT Limit | Numeric | Min 0x0 · Max 0xFFFFFFFF · Default 0x0 ◀ default ◀ performance TUNE Higher Fast PPT allows larger short-term power spikes for snappier boost; keep within thermal headroom. |
| Slow PPT Limit | Numeric | Min 0x0 · Max 0xFFFFFFFF · Default 0x0 ◀ default ◀ performance TUNE Higher Slow PPT raises the sustained power ceiling for long workloads. |
| Slow PPT Time Constant | Numeric | Min 0x0 · Max 0xFFFFFFFF · Default 0x0 ◀ default ◀ performance NEUTRAL Tau for the slow power filter — leave default unless deliberately shaping boost behavior. |
| STAPM Control | Selection | Manual [0x1] Auto [0x0] ◀ default ◀ performance TUNE Manual to relax STAPM (skin-temp-aware power) for more sustained performance; default protects chassis temperature. |
| System Temperature Tracking | Selection | Auto [0xF] ◀ default 1 [0x1] 0 [0x0] ◀ performance TUNE Disabling skin-temp tracking removes a sustained-power cap (more performance, hotter chassis); keep enabled for safe thermals. |
| STAPM Boost Override | Selection | Auto [0xF] ◀ default 0 [0x1] 1 [0x0] ◀ performance TUNE Enable to extend boost duration before STAPM clamps; raises temps. |
| STAPM Boost | Selection | Auto [0xF] ◀ default 1 [0x1] 0 [0x0] ◀ performance TUNE Higher STAPM boost prolongs high-power boost; bounded by chassis thermals. |
| Tskin Time Constant (STAPM) | Numeric | Min 0x0 · Max 0xFFFFFFFF · Default 0x0 ◀ default ◀ performance NEUTRAL Skin-temp filter constant — leave default. |
| Thermal Control | Selection | Manual [0x1] ◀ default Auto [0x0] ◀ performance TUNE Manual to raise TjMax-driven limits (only with strong cooling); default keeps the die at safe temps. |
| TjMax | Numeric | Min 0x0 · Max 0xFFFFFFFF · Default 0x5A ◀ default ◀ performance TUNE Default (typically 100 C). Lowering TjMax reduces boost; raising is not advisable. Leave at the AMD-validated junction limit. |
| TDC Control | Selection | Manual [0x1] Auto [0x0] ◀ default ◀ performance TUNE Manual to raise sustained current limits in step with PPT for all-core boost. |
| TDC_VDDCR_VDD | Numeric | Min 0x0 · Max 0xFFFFFFFF · Default 0x0 ◀ default ◀ performance TUNE Core-rail sustained current cap — raise with PPT for sustained all-core clocks. |
| TDC_VDDCR_SOC | Numeric | Min 0x0 · Max 0xFFFFFFFF · Default 0x0 ◀ default ◀ performance NEUTRAL SoC-rail sustained current — leave Auto; not a core-throughput lever. |
| TDC_VDDCR_SR | Numeric | Min 0x0 · Max 0xFFFFFFFF · Default 0x0 ◀ default ◀ performance NEUTRAL Secondary-rail sustained current — leave Auto. |
| EDC Control | Selection | Manual [0x1] Auto [0x0] ◀ default ◀ performance TUNE Manual to raise peak current limits for burst boost. |
| EDC_VDDCR_VDD | Numeric | Min 0x0 · Max 0xFFFFFFFF · Default 0x0 ◀ default ◀ performance TUNE Core-rail peak current cap — raise for burst boost headroom. |
| EDC_VDDCR_SOC | Numeric | Min 0x0 · Max 0xFFFFFFFF · Default 0x0 ◀ default ◀ performance NEUTRAL SoC-rail peak current — leave Auto. |
| EDC_VDDCR_SR | Numeric | Min 0x0 · Max 0xFFFFFFFF · Default 0x0 ◀ default ◀ performance NEUTRAL Secondary-rail peak current — leave Auto. |
| PSI3 Control | Selection | Manual [0x1] Auto [0x0] ◀ default ◀ performance NEUTRAL Power-state interface control (VRM efficiency) — leave Auto; no throughput impact. |
| PSI3_VDDCR_VDD | Numeric | Min 0x0 · Max 0xFFFFFFFF · Default 0x0 ◀ default ◀ performance NEUTRAL VRM PSI threshold — leave Auto. |
| PROCHOT Control | Selection | Manual [0x1] Auto [0x0] ◀ default ◀ performance TUNE Leave Auto. PROCHOT is the thermal-safety throttle; disabling it risks the silicon and is not a sanctioned performance gain. |
| PROCHOT Deassertion Ramp Time | Numeric | Min 0x0 · Max 0xFFFFFFFF · Default 0x0 ◀ default ◀ performance NEUTRAL Throttle-recovery ramp — leave default. |
| STT Control | Selection | Manual [0x1] Auto [0x0] ◀ default ◀ performance TUNE Skin-Temperature Tracking master — Manual to raise skin-temp limits for more sustained power (hotter chassis). |
| STT_PCB_SENSOR_COUNT | Numeric | Min 0x0 · Max 0xF · Default 0x0 ◀ default ◀ performance NEUTRAL Skin-Temperature-Tracking model coefficient — part of the OEM-tuned thermal model. Don’t hand-edit individual coefficients; adjust the skin-temp limit instead. Leave default. |
| STT_MIN_POWER_LIMIT | Numeric | Min 0x0 · Max 0xFFFF · Default 0x0 ◀ default ◀ performance NEUTRAL Floor for STT power — leave default. |
| STT_M1 | Numeric | Min 0x0 · Max 0xFFFF · Default 0x0 ◀ default ◀ performance NEUTRAL Skin-Temperature-Tracking model coefficient — part of the OEM-tuned thermal model. Don’t hand-edit individual coefficients; adjust the skin-temp limit instead. Leave default. |
| STT_M2 | Numeric | Min 0x0 · Max 0xFFFF · Default 0x0 ◀ default ◀ performance NEUTRAL Skin-Temperature-Tracking model coefficient — part of the OEM-tuned thermal model. Don’t hand-edit individual coefficients; adjust the skin-temp limit instead. Leave default. |
| STT_M3 | Numeric | Min 0x0 · Max 0xFFFF · Default 0x0 ◀ default ◀ performance NEUTRAL Skin-Temperature-Tracking model coefficient — part of the OEM-tuned thermal model. Don’t hand-edit individual coefficients; adjust the skin-temp limit instead. Leave default. |
| STT_M4 | Numeric | Min 0x0 · Max 0xFFFF · Default 0x0 ◀ default ◀ performance NEUTRAL Skin-Temperature-Tracking model coefficient — part of the OEM-tuned thermal model. Don’t hand-edit individual coefficients; adjust the skin-temp limit instead. Leave default. |
| STT_M5 | Numeric | Min 0x0 · Max 0xFFFF · Default 0x0 ◀ default ◀ performance NEUTRAL Skin-Temperature-Tracking model coefficient — part of the OEM-tuned thermal model. Don’t hand-edit individual coefficients; adjust the skin-temp limit instead. Leave default. |
| STT_M6 | Numeric | Min 0x0 · Max 0xFFFF · Default 0x0 ◀ default ◀ performance NEUTRAL Skin-Temperature-Tracking model coefficient — part of the OEM-tuned thermal model. Don’t hand-edit individual coefficients; adjust the skin-temp limit instead. Leave default. |
| STT_C_APU | Numeric | Min 0x0 · Max 0xFFFF · Default 0x0 ◀ default ◀ performance NEUTRAL Skin-Temperature-Tracking model coefficient — part of the OEM-tuned thermal model. Don’t hand-edit individual coefficients; adjust the skin-temp limit instead. Leave default. |
| STT_C_HS2 | Numeric | Min 0x0 · Max 0xFFFF · Default 0x0 ◀ default ◀ performance NEUTRAL Skin-Temperature-Tracking model coefficient — part of the OEM-tuned thermal model. Don’t hand-edit individual coefficients; adjust the skin-temp limit instead. Leave default. |
| STT_ALPHA_APU | Numeric | Min 0x0 · Max 0xFFFF · Default 0x0 ◀ default ◀ performance NEUTRAL Skin-Temperature-Tracking model coefficient — part of the OEM-tuned thermal model. Don’t hand-edit individual coefficients; adjust the skin-temp limit instead. Leave default. |
| STT_ALPHA_HS2 | Numeric | Min 0x0 · Max 0xFFFF · Default 0x0 ◀ default ◀ performance NEUTRAL Skin-Temperature-Tracking model coefficient — part of the OEM-tuned thermal model. Don’t hand-edit individual coefficients; adjust the skin-temp limit instead. Leave default. |
| STT_SKIN_TEMPERATURE_LIMIT_APU | Numeric | Min 0x0 · Max 0xFFFF · Default 0x0 ◀ default ◀ performance TUNE Raising the APU skin-temp limit sustains boost longer at higher chassis temperature; default protects surface temp. |
| STT_SKIN_TEMPERATURE_LIMIT_HS2 | Numeric | Min 0x0 · Max 0xFFFF · Default 0x0 ◀ default ◀ performance TUNE Raising the secondary skin-temp limit sustains more power; bounded by safe chassis temps. |
| STT_ERROR_COEFF | Numeric | Min 0x0 · Max 0xFFFF · Default 0x0 ◀ default ◀ performance NEUTRAL Skin-Temperature-Tracking model coefficient — part of the OEM-tuned thermal model. Don’t hand-edit individual coefficients; adjust the skin-temp limit instead. Leave default. |
| STT_ERROR_RATE_COEFF | Numeric | Min 0x0 · Max 0xFFFF · Default 0x0 ◀ default ◀ performance NEUTRAL Skin-Temperature-Tracking model coefficient — part of the OEM-tuned thermal model. Don’t hand-edit individual coefficients; adjust the skin-temp limit instead. Leave default. |
| Fan Control | Selection | Manual [0x1] Auto [0x0] ◀ default ◀ performance TUNE Use a graduated aggressive curve. More airflow = more thermal headroom = higher sustained boost (the documented GTR9 Pro tuning win). |
| Force PWM Control | Selection | Force [0x1] Unforce [0x0] ◀ default ◀ performance TUNE Enable to pin a fixed high PWM for maximum cooling during benchmarks/sustained loads (noisy). |
| Force PWM | Numeric | Min 0x0 · Max 0x64 · Default 0x0 ◀ default ◀ performance TUNE Higher fixed PWM raises airflow and sustained clocks at the cost of noise. |
| Fan Table Control | Selection | Manual [0x1] Auto [0x0] ◀ default ◀ performance TUNE Manual to define an aggressive temp/PWM table for better sustained thermals. |
| Low Temperature | Numeric | Min 0x0 · Max 0xFF · Default 0x0 ◀ default ◀ performance TUNE Fan-curve point. For sustained performance, bias the curve toward more airflow (lower start temp / higher PWM) to gain thermal headroom; trade against noise. |
| Medium Temperature | Numeric | Min 0x0 · Max 0xFF · Default 0x0 ◀ default ◀ performance TUNE Fan-curve point. For sustained performance, bias the curve toward more airflow (lower start temp / higher PWM) to gain thermal headroom; trade against noise. |
| High Temperature | Numeric | Min 0x0 · Max 0xFF · Default 0x0 ◀ default ◀ performance TUNE Fan-curve point. For sustained performance, bias the curve toward more airflow (lower start temp / higher PWM) to gain thermal headroom; trade against noise. |
| Critical Temperature | Numeric | Min 0x0 · Max 0xFF · Default 0x0 ◀ default ◀ performance TUNE Fan-curve point. For sustained performance, bias the curve toward more airflow (lower start temp / higher PWM) to gain thermal headroom; trade against noise. |
| Low Pwm | Numeric | Min 0x0 · Max 0x64 · Default 0x0 ◀ default ◀ performance TUNE Fan-curve point. For sustained performance, bias the curve toward more airflow (lower start temp / higher PWM) to gain thermal headroom; trade against noise. |
| Medium Pwm | Numeric | Min 0x0 · Max 0x65 · Default 0x0 ◀ default ◀ performance TUNE Fan-curve point. For sustained performance, bias the curve toward more airflow (lower start temp / higher PWM) to gain thermal headroom; trade against noise. |
| High Pwm | Numeric | Min 0x0 · Max 0x66 · Default 0x0 ◀ default ◀ performance TUNE Fan-curve point. For sustained performance, bias the curve toward more airflow (lower start temp / higher PWM) to gain thermal headroom; trade against noise. |
| Temperature Hysteresis | Numeric | Min 0x0 · Max 0xFF · Default 0x0 ◀ default ◀ performance TUNE Fan-curve point. For sustained performance, bias the curve toward more airflow (lower start temp / higher PWM) to gain thermal headroom; trade against noise. |
| Pwm Frequency | Selection | Auto [0xF] ◀ default 1 [0x1] 0 [0x0] ◀ performance TUNE Fan-curve point. For sustained performance, bias the curve toward more airflow (lower start temp / higher PWM) to gain thermal headroom; trade against noise. |
| Fan polarity | Selection | Auto [0xF] ◀ default 1 [0x1] 0 [0x0] ◀ performance TUNE Fan-curve point. For sustained performance, bias the curve toward more airflow (lower start temp / higher PWM) to gain thermal headroom; trade against noise. |

### SOC Miscellaneous Control   (Form 0x7007)

Submenus: Pluton Options  ‣  Firmware Anti-rollback (FAR)  ‣  AIM-T Options  ‣  Intrusion Detection  ‣  SecureBio

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| Pluton Attribute Control | Selection | Disabled [0x0] Enabled [0x1] ◀ default ◀ performance NEUTRAL Security/attestation feature — no performance impact. Set per your security policy (defaults are fine for performance). |
| Pluton fTPM Attribute Control | Selection | Disabled [0x0] Enabled [0x1] ◀ default ◀ performance NEUTRAL Security/attestation feature — no performance impact. Set per your security policy (defaults are fine for performance). |
| Trusted Platform Module | Selection | Auto [0xFE] ◀ default Disabled [0xFF] Enable dTPM [0x0] Enable ASP fTPM [0x1] Enable Pluton fTPM [0x2] ◀ performance NEUTRAL Security/attestation feature — no performance impact. Set per your security policy (defaults are fine for performance). |
| Pluton Support | Selection | Auto [0xF] ◀ default Disabled [0x0] Enabled [0x1] ◀ performance NEUTRAL Security/attestation feature — no performance impact. Set per your security policy (defaults are fine for performance). |
| Pluton Security Processor | Selection | Auto [0xF] ◀ default Disabled [0x0] Enabled [0x1] ◀ performance NEUTRAL Security/attestation feature — no performance impact. Set per your security policy (defaults are fine for performance). |
| Microsoft Security Levels | Selection | Customized [0xFF] ◀ default dTPM Level 1 without Pluton Security Processor [0x1] dTPM Level 2 without Pluton Security Processor [0x2] dTPM Level 3 without Pluton Security Processor [0x3] dTPM Level 1 with Pluton Security Processor [0xB] dTPM Level 2 with Pluton Security Processor [0xC] dTPM Level 3 with Pluton Security Processor [0xD] Pluton fTPM Level 1 without Pluton Security Processor [0x15] Pluton fTPM Level 2 without Pluton Security Processor [0x16] Pluton fTPM Level 3 without Pluton Security Processor [0x17] Pluton fTPM Level 1 with Pluton Security Processor [0x1F] Pluton fTPM Level 2 with Pluton Security Processor [0x20] Pluton fTPM Level 3 with Pluton Security Processor [0x21] ASP fTPM Level 0 [0x28] ASP fTPM Level 1 without Pluton Security Processor [0x29] ASP fTPM Level 2 without Pluton Security Processor [0x2A] ASP fTPM Level 1 with Pluton Security Processor [0x33] ASP fTPM Level 2 with Pluton Security Processor [0x34] ◀ performance NEUTRAL Security/attestation feature — no performance impact. Set per your security policy (defaults are fine for performance). |
| Secured-core Auto enablement | Selection | Auto [0xFF] ◀ default Enabled [0x1] Disabled [0x0] ◀ performance NEUTRAL Security/attestation feature — no performance impact. Set per your security policy (defaults are fine for performance). |
| DRTM Support | Selection | Auto [0xFF] ◀ default Enabled [0x1] Disabled [0x0] ◀ performance NEUTRAL Security/attestation feature — no performance impact. Set per your security policy (defaults are fine for performance). |
| SMM Isolation Support | Selection | Auto [0xFF] ◀ default Enabled [0x1] Disabled [0x0] ◀ performance NEUTRAL Security/attestation feature — no performance impact. Set per your security policy (defaults are fine for performance). |
| Pro Part attribute control | Selection | 0 [0x0] ◀ default 1 [0x1] ◀ performance NEUTRAL PMF policy/firmware-binary plumbing — the OEM ships validated policy; no user performance setting here. Leave default. |
| GFX workstation support | Selection | Auto [0xF] ◀ default Enabled [0x1] Disabled [0x0] ◀ performance NEUTRAL PMF policy/firmware-binary plumbing — the OEM ships validated policy; no user performance setting here. Leave default. |
| ABL Console Out Control | Selection | Auto [0x2] ◀ default Enable [0x1] Disable [0x0] ◀ performance NEUTRAL Serial/debug interface — leave Disabled/default. Debug output can slightly slow boot; no runtime performance benefit. |
| ABL Console Out Serial Port | Selection | Auto [0xFF] ◀ default LPC UART [0x0] FCH UART0 [0x1] FCH UART1 [0x2] FCH UART2 [0x3] FCH UART3 [0x4] ◀ performance NEUTRAL Serial/debug interface — leave Disabled/default. Debug output can slightly slow boot; no runtime performance benefit. |
| ABL Console Out Boot Mode Select | Selection | ALL [0xFF] ◀ default S3/S0i3 only [0x2] Normal boot only [0x1] ◀ performance NEUTRAL Serial/debug interface — leave Disabled/default. Debug output can slightly slow boot; no runtime performance benefit. |
| ABL PMU message Control | Selection | Auto [0xFF] ◀ default Maximal debug messages [0x4] Detailed debug message [0x5] Coarse debug message [0xA] Stage completion [0xC8] Assertion message [0xC9] Firmware completion message only [0xFE] ◀ performance NEUTRAL Serial/debug interface — leave Disabled/default. Debug output can slightly slow boot; no runtime performance benefit. |
| PSP RPMC Switch | Selection | Auto [0xFF] ◀ default Disabled [0x0] Enabled [0x1] ◀ performance NEUTRAL Security/attestation feature — no performance impact. Set per your security policy (defaults are fine for performance). |
| ABL Training Mode | Selection | default [0x0] ◀ default serial [0x1] parallel [0x2] ◀ performance NEUTRAL Serial/debug interface — leave Disabled/default. Debug output can slightly slow boot; no runtime performance benefit. |

### Pluton Options   (Form 0x7056)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| Pluton UART | Selection | Disabled [0x0] Enabled [0x1] Auto [0xF] ◀ default ◀ performance NEUTRAL Security/attestation feature — no performance impact. Set per your security policy (defaults are fine for performance). |
| Pluton UART Port | Selection | UART 0 [0x0] UART 1 [0x1] UART 2 [0x2] UART 3 [0x3] UART 4 [0x4] UART 5 [0x5] UART 6 [0x6] UART 7 [0x7] Auto [0xF] ◀ default ◀ performance NEUTRAL Security/attestation feature — no performance impact. Set per your security policy (defaults are fine for performance). |
| Pluton FIPS | Selection | Disabled [0x0] Enabled [0x1] Auto [0xF] ◀ default ◀ performance NEUTRAL Security/attestation feature — no performance impact. Set per your security policy (defaults are fine for performance). |
| Pluton FIPS zeroization | Selection | Disabled [0x0] Enabled [0x1] Auto [0xF] ◀ default ◀ performance NEUTRAL Security/attestation feature — no performance impact. Set per your security policy (defaults are fine for performance). |

### Firmware Anti-rollback (FAR)   (Form 0x7057)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| FAR enforcement state | Selection | Enabled [0x1] Disabled [0x0] ◀ default ◀ performance NEUTRAL Security/attestation feature — no performance impact. Set per your security policy (defaults are fine for performance). |
| SPL value in the CPU fuse | Numeric | Min 0x0 · Max 0xFF · Default 0x0 ◀ default ◀ performance NEUTRAL Security/attestation feature — no performance impact. Set per your security policy (defaults are fine for performance). |
| SPL value in the SPL table | Numeric | Min 0x0 · Max 0xFF · Default 0x0 ◀ default ◀ performance NEUTRAL Security/attestation feature — no performance impact. Set per your security policy (defaults are fine for performance). |
| Initial SPL value for FAR | Numeric | Min 0x0 · Max 0xFF · Default 0x0 ◀ default ◀ performance NEUTRAL Security/attestation feature — no performance impact. Set per your security policy (defaults are fine for performance). |
| SPL update switch | Selection | Auto [0xFF] ◀ default Enabled [0x1] Disabled [0x0] ◀ performance NEUTRAL Security/attestation feature — no performance impact. Set per your security policy (defaults are fine for performance). |

### AIM-T Options   (Form 0x7058)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| AIM-T Support | Selection | AIM-T Disabled [0x0] AIM-T Enabled [0x1] Auto [0xF] ◀ default ◀ performance NEUTRAL PMF policy/firmware-binary plumbing — the OEM ships validated policy; no user performance setting here. Leave default. |
| MPM attribute control | Selection | 0 [0x0] ◀ default 1 [0x1] 2 [0x2] 3 [0x3] 4 [0x4] 5 [0x5] 6 [0x6] 7 [0x7] ◀ performance NEUTRAL PMF policy/firmware-binary plumbing — the OEM ships validated policy; no user performance setting here. Leave default. |
| KVM for Wired Manageability | Selection | Disabled [0x0] Enabled [0x1] Auto [0xF] ◀ default ◀ performance NEUTRAL Out-of-band management (DASH/ASF) — no performance impact; enable only if you use remote management. |
| Wireless Manageability | Selection | Disabled [0x0] Enabled [0x1] Auto [0xF] ◀ default ◀ performance NEUTRAL Out-of-band management (DASH/ASF) — no performance impact; enable only if you use remote management. |

### Intrusion Detection   (Form 0x7059)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| Intrusion Detection Control | Numeric | Min 0x0 · Max 0xFFFF · Default 0x0 ◀ default ◀ performance NEUTRAL Out-of-band management (DASH/ASF) — no performance impact; enable only if you use remote management. |
| Intrusion Detection Enable | Selection | Auto [0xFF] ◀ default Enabled [0x1] Disabled [0x0] ◀ performance NEUTRAL Out-of-band management (DASH/ASF) — no performance impact; enable only if you use remote management. |
| Log Intrusion Event | Selection | Enabled [0x1] Disabled [0x0] ◀ default ◀ performance NEUTRAL Out-of-band management (DASH/ASF) — no performance impact; enable only if you use remote management. |
| Clear TPM | Selection | Enabled [0x1] Disabled [0x0] ◀ default ◀ performance NEUTRAL Security/attestation feature — no performance impact. Set per your security policy (defaults are fine for performance). |
| Power Off System | Selection | Enabled [0x1] Disabled [0x0] ◀ default ◀ performance NEUTRAL ACPI sleep/button policy — no runtime performance impact. S3 sleep is fine; set per preference. |

### SecureBio   (Form 0x705A)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| SecureBio Support | Selection | Auto [0xF] ◀ default Enable [0x1] Disable [0x0] ◀ performance NEUTRAL Security/attestation feature — no performance impact. Set per your security policy (defaults are fine for performance). |
| SecureBio Camera Support | Selection | Disabled [0x0] ◀ default XHCI Camera [0x1] MIPI Camera [0x2] Auto [0xF] ◀ performance NEUTRAL Security/attestation feature — no performance impact. Set per your security policy (defaults are fine for performance). |

# AMD PBS (Platform BIOS Settings)

AMD Platform BIOS Settings — board/OEM-level platform configuration reached via Advanced ‣ AMD PBS. Covers PCIe lane allocation, power-saving (D3) states, USB/Thunderbolt/USB4, graphics/display routing, audio, I2C, thermal, and MP2 sensor fusion.

Form-set GUID: B863B959-0EC6-4033-99C1-8FD89F040222     ·     Forms: 14     ·     Configurable settings: 206

### AMD PBS Option   (Form 0xB)

Submenus: AMD Firmware Version  ‣  PCI Express Configurations  ‣  Power Saving Configurations  ‣  Graphics Configurations  ‣  Display Configurations  ‣  USB/Thunderbolt Configurations  ‣  USB TypeC Security Mode Configurations  ‣  Audio Configurations  ‣  I2C Configurations  ‣  Thermal Configurations  ‣  MP2 Configurations  ‣  EC/PD Configurations  ‣  Debug Configurations

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| Enable SLT check | Selection | Disabled [0x0] Enabled [0x1] ◀ default Force SLT check halt [0x2] ◀ performance NEUTRAL Device power-rail enable / timing — functional, no performance impact. Leave default. |
| Above 4GB MMIO Limit | Selection | 35bit (32GB) [0x23] 36bit (64GB) [0x24] 37bit (128GB) [0x25] 38bit (256GB) [0x26] 39bit (512GB) [0x27] 40bit (1TB) [0x28] ◀ default 41bit (2TB) [0x29] 42bit (4TB) [0x2A] 43bit (8TB) [0x2B] 44bit (16TB) [0x2C] 45bit (32TB) [0x2D] 46bit (64TB) [0x2E] 47bit (128TB) [0x2F] 48bit (256TB) [0x30] ◀ performance KEEP Enable / leave high. Above-4G decoding is required for Resizable BAR and large-BAR GPUs; no downside on UEFI. |
| Wireless LAN Recovery | Selection | Disabled [0x0] ◀ default Enabled [0x1] Dummy reset [0x2] ◀ performance NEUTRAL Peripheral/sensor enable — no compute-performance impact; set per hardware need. |
| Wireless LAN Country Code | Selection | Disabled [0x0] ◀ default Country Code USA [0x1] Country Code China [0x2] ◀ performance NEUTRAL Peripheral/sensor enable — no compute-performance impact; set per hardware need. |
| Bluetooth PLDR | Selection | Disabled [0x0] ◀ default Enabled [0x1] Dummy reset [0x2] ◀ performance NEUTRAL Peripheral/sensor enable — no compute-performance impact; set per hardware need. |
| Wireless Button | Selection | Disabled [0x0] ◀ default HID Based [0x3] ◀ performance NEUTRAL Peripheral/sensor enable — no compute-performance impact; set per hardware need. |
| Processor Aggregator Device | Selection | Disabled [0x0] ◀ default Enabled [0x1] ◀ performance NEUTRAL ACPI processor-aggregator (collaborative power control) — leave default; no throughput impact. |
| Core Count Control | Selection | Disabled [0x0] ◀ default Enabled [0x1] ◀ performance KEEP Leave all cores enabled. Reducing core count lowers multi-thread performance; only down-core for special validation. |
| Core Count SW_SCI_GPE_ID | Numeric | Min 0x0 · Max 0x1F · Default 0x19 ◀ default ◀ performance NEUTRAL Plumbing for core-count signaling — no performance impact. |
| APIC Software Enable | Selection | Disabled [0x0] Enabled [0x1] ◀ default ◀ performance NEUTRAL Leave default; no performance impact. |
| Dynamic P3T limit | Selection | Disabled [0x0] Enable for DC-only case (include fake DC) [0x1] ◀ default Enabled [0x2] ◀ performance TUNE Deep device-idle (D3Cold) power gating. Disabling lowers wake latency for that device (marginal perf); Enabled saves idle power. Default is fine for desktop use. |
| Dynamic LID | Selection | Disabled [0x0] ◀ default Enabled [0x1] ◀ performance TUNE Deep device-idle (D3Cold) power gating. Disabling lowers wake latency for that device (marginal perf); Enabled saves idle power. Default is fine for desktop use. |
| HDMI 3.0G Tx SLEW | Selection | Disabled [0x0] ◀ default Enabled [0x1] ◀ performance NEUTRAL Display/panel/LED configuration — no compute-performance impact. |
| HDMI 3.0G Tx Slew Control Value | Numeric | Min 0x0 · Max 0xFF · Default 0x14 ◀ default ◀ performance NEUTRAL Display/panel/LED configuration — no compute-performance impact. |
| Control Method Button Support | Selection | Legacy Power Button (PNP0C0C) [0x0] ◀ default Generic Button (ACPI0011) [0x1] ◀ performance NEUTRAL ACPI sleep/button policy — no runtime performance impact. S3 sleep is fine; set per preference. |

### AMD Firmware Version   (Form 0xC)

Information fields: AGESA Version, PSP BootLoader Version, PSP SecureOS Version, ABL Version, APCB Version, APOB Version, DF Rib Version, Ucode Patch Version, S3 Image ID, SMU FW Version, ZSC FW Version, MPIO FW Version, MP2 FW Version, XHCI FW Version, IMU FW Version, VBIOS FW Version, GOP Driver Version, EC FW Version, EDKII Version, PD0 FW Version, PD0 Custom, PD0 SKU, PD0 Mini Type, Re-timer, PD1 FW Version, PD1 Custom, PD1 SKU, PD1 Mini Type, AIM-T(MPM) Part1 FW Version, AIM-T(MPM) Part2 FW Version

### PCI Express Configurations   (Form 0xD)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| Pcie Dxio Timing ControlEnable | Selection | Auto [0xF] ◀ default Enabled [0x1] Disabled [0x0] ◀ performance TUNE PCIe link power management. Disabled = full link speed and lowest latency (best performance, higher idle power); Enabled (L1) = power saving with wake latency. Performance: Disabled. |
| PCIE Link Receiver Detection Polling | Numeric | Min 0x0 · Max 0xFFFFFFFF · Default 0x0 ◀ default ◀ performance TUNE PCIe link power management. Disabled = full link speed and lowest latency (best performance, higher idle power); Enabled (L1) = power saving with wake latency. Performance: Disabled. |
| PCIE Link L0 Polling | Numeric | Min 0x0 · Max 0xFFFFFFFF · Default 0x0 ◀ default ◀ performance TUNE PCIe link power management. Disabled = full link speed and lowest latency (best performance, higher idle power); Enabled (L1) = power saving with wake latency. Performance: Disabled. |
| Enable power sequence of specific WWAN | Selection | Disabled [0x0] ◀ default Power Sequence — Fibocom [0x1] Power Sequence — Quectel RM520 [0x2] Power Sequence — Quectel EM160 [0x3] ◀ performance NEUTRAL Device power-rail enable / timing — functional, no performance impact. Leave default. |
| PCIE x4 DT Slot Power Enable | Selection | Disabled [0x0] Enabled [0x1] ◀ default ◀ performance NEUTRAL Device power-rail enable / timing — functional, no performance impact. Leave default. |
| M.2 SSD1 Power Enable | Selection | Disabled [0x0] Enabled [0x1] ◀ default ◀ performance NEUTRAL Device power-rail enable / timing — functional, no performance impact. Leave default. |
| WLAN Power Enable | Selection | Disabled [0x0] x1 [0x1] ◀ default x2 [0x2] ◀ performance NEUTRAL Device power-rail enable / timing — functional, no performance impact. Leave default. |
| WWAN Power Enable | Selection | Disabled [0x0] x1 [0x1] ◀ default x2 [0x2] ◀ performance NEUTRAL Device power-rail enable / timing — functional, no performance impact. Leave default. |
| SD Card Reader Power Enable | Selection | Disabled [0x0] Enabled [0x1] ◀ default ◀ performance NEUTRAL Device power-rail enable / timing — functional, no performance impact. Leave default. |
| Initial SD7 L1 Substates at OS runtime | Selection | Disabled [0x0] ◀ default Enabled [0x1] ◀ performance NEUTRAL SD-card-reader L1 substate handoff — no compute-performance impact; leave default. |
| GbE Power Enable | Selection | Disabled [0x0] Enabled [0x1] ◀ default ◀ performance NEUTRAL Device power-rail enable / timing — functional, no performance impact. Leave default. |
| M.2 SSD0 Power Enable | Selection | Disabled [0x0] Enabled [0x1] ◀ default ◀ performance NEUTRAL Device power-rail enable / timing — functional, no performance impact. Leave default. |
| NVMe RAID mode | Selection | Disabled [0x0] ◀ default Enabled [0x1] ◀ performance NEUTRAL RAID array configuration — performance depends on RAID level and member SSDs, not a single setting. RAID0 maximizes throughput; single-NVMe is simplest. No generic default change. |
| Save Restore DXIO Feature | Selection | Auto [0xF] ◀ default Disabled [0x0] Enabled [0x1] ◀ performance NEUTRAL Firmware boot/training plumbing — leave default; not a user performance setting. |

### Power Saving Configurations   (Form 0xE)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| NVME D3Cold | Selection | Disabled [0x0] ◀ default Enabled [0x1] ◀ performance TUNE Deep device-idle (D3Cold) power gating. Disabling lowers wake latency for that device (marginal perf); Enabled saves idle power. Default is fine for desktop use. |
| M.2 NVME Tpvperl | Numeric | Min 0x32 · Max 0x3E8 · Default 0x32 ◀ default ◀ performance NEUTRAL Firmware boot/training plumbing — leave default; not a user performance setting. |
| M.2 NVME Trst-cfg | Numeric | Min 0x64 · Max 0x3E8 · Default 0x64 ◀ default ◀ performance NEUTRAL Firmware boot/training plumbing — leave default; not a user performance setting. |
| WLAN D3Cold | Selection | Disabled [0x0] ◀ default Enabled [0x1] ◀ performance TUNE Deep device-idle (D3Cold) power gating. Disabling lowers wake latency for that device (marginal perf); Enabled saves idle power. Default is fine for desktop use. |
| LOM D3Cold | Selection | Disabled [0x0] ◀ default Enabled [0x1] ◀ performance TUNE Deep device-idle (D3Cold) power gating. Disabling lowers wake latency for that device (marginal perf); Enabled saves idle power. Default is fine for desktop use. |
| SD CardReader D3Cold | Selection | Disabled [0x0] ◀ default Enabled [0x1] ◀ performance TUNE Deep device-idle (D3Cold) power gating. Disabling lowers wake latency for that device (marginal perf); Enabled saves idle power. Default is fine for desktop use. |
| USB4 D3 Eanble | Selection | Disabled [0x0] Enabled [0x1] ◀ default ◀ performance NEUTRAL USB port/controller enable — functional toggle, no performance impact. Leave default unless disabling unused ports for security/power. |
| Internal PCIe GPP 0 D3 | Selection | Disabled [0x0] ◀ default Enabled [0x4] ◀ performance TUNE Deep device-idle (D3Cold) power gating. Disabling lowers wake latency for that device (marginal perf); Enabled saves idle power. Default is fine for desktop use. |
| SOC GPU D3 | Selection | Disabled [0x0] ◀ default Enabled [0x4] ◀ performance TUNE Deep device-idle (D3Cold) power gating. Disabling lowers wake latency for that device (marginal perf); Enabled saves idle power. Default is fine for desktop use. |
| SOC HD Audio D3 | Selection | Disabled [0x0] Enabled [0x4] ◀ default ◀ performance TUNE Deep device-idle (D3Cold) power gating. Disabling lowers wake latency for that device (marginal perf); Enabled saves idle power. Default is fine for desktop use. |
| SOC USB3.1 D3 | Selection | Disabled [0x0] Enabled [0x4] ◀ default ◀ performance NEUTRAL USB port/controller enable — functional toggle, no performance impact. Leave default unless disabling unused ports for security/power. |
| SOC ACP D3 | Selection | Disabled [0x0] Enabled [0x4] ◀ default ◀ performance TUNE Deep device-idle (D3Cold) power gating. Disabling lowers wake latency for that device (marginal perf); Enabled saves idle power. Default is fine for desktop use. |
| SOC Azalia D3 | Selection | Disabled [0x0] Enabled [0x4] ◀ default ◀ performance TUNE Deep device-idle (D3Cold) power gating. Disabling lowers wake latency for that device (marginal perf); Enabled saves idle power. Default is fine for desktop use. |
| Internal PCIe GPP 2 D3 | Selection | Disabled [0x0] ◀ default Enabled [0x4] ◀ performance TUNE Deep device-idle (D3Cold) power gating. Disabling lowers wake latency for that device (marginal perf); Enabled saves idle power. Default is fine for desktop use. |
| SOC USB3.1 for USB4 D3 | Selection | Disabled [0x0] Enabled [0x4] ◀ default ◀ performance NEUTRAL USB port/controller enable — functional toggle, no performance impact. Leave default unless disabling unused ports for security/power. |
| SOC USB4 D3 | Selection | Disabled [0x0] Enabled [0x4] ◀ default ◀ performance NEUTRAL USB port/controller enable — functional toggle, no performance impact. Leave default unless disabling unused ports for security/power. |
| Internal USB4 PCIe Tunneling D3 | Selection | Disabled [0x0] Enabled [0x4] ◀ default ◀ performance NEUTRAL USB port/controller enable — functional toggle, no performance impact. Leave default unless disabling unused ports for security/power. |
| SOC USB4 PCIe Endpoint D3 | Selection | Disabled [0x0] Enabled [0x4] ◀ default ◀ performance NEUTRAL USB port/controller enable — functional toggle, no performance impact. Leave default unless disabling unused ports for security/power. |
| Keep Wlan Power In S3/S4 state | Selection | Disabled [0x0] ◀ default Enable Wlan Power In S3 And S4 [0x3] ◀ performance NEUTRAL Wake-source/RTC option — no performance impact; set per convenience. |
| Unused GPP Clocks Off | Selection | Disabled [0x0] ◀ default Enabled [0x1] ◀ performance TUNE PCIe link power management. Disabled = full link speed and lowest latency (best performance, higher idle power); Enabled (L1) = power saving with wake latency. Performance: Disabled. |
| Clock PM: CLK_REQ0 | Selection | Disabled [0x0] Enabled [0x1] ◀ default ◀ performance TUNE PCIe link power management. Disabled = full link speed and lowest latency (best performance, higher idle power); Enabled (L1) = power saving with wake latency. Performance: Disabled. |
| Clock PM: CLK_REQ1 | Selection | Disabled [0x0] Enabled [0x1] ◀ default ◀ performance TUNE PCIe link power management. Disabled = full link speed and lowest latency (best performance, higher idle power); Enabled (L1) = power saving with wake latency. Performance: Disabled. |
| Clock PM: CLK_REQ2 | Selection | Disabled [0x0] Enabled [0x1] ◀ default ◀ performance TUNE PCIe link power management. Disabled = full link speed and lowest latency (best performance, higher idle power); Enabled (L1) = power saving with wake latency. Performance: Disabled. |
| Clock PM: CLK_REQ3 | Selection | Disabled [0x0] Enabled [0x1] ◀ default ◀ performance TUNE PCIe link power management. Disabled = full link speed and lowest latency (best performance, higher idle power); Enabled (L1) = power saving with wake latency. Performance: Disabled. |
| Clock PM: CLK_REQ4 | Selection | Disabled [0x0] Enabled [0x1] ◀ default ◀ performance TUNE PCIe link power management. Disabled = full link speed and lowest latency (best performance, higher idle power); Enabled (L1) = power saving with wake latency. Performance: Disabled. |
| Clock PM: CLK_REQ5 | Selection | Disabled [0x0] Enabled [0x1] ◀ default ◀ performance TUNE PCIe link power management. Disabled = full link speed and lowest latency (best performance, higher idle power); Enabled (L1) = power saving with wake latency. Performance: Disabled. |
| Clock PM: CLK_REQ6 | Selection | Disabled [0x0] Enabled [0x1] ◀ default ◀ performance TUNE PCIe link power management. Disabled = full link speed and lowest latency (best performance, higher idle power); Enabled (L1) = power saving with wake latency. Performance: Disabled. |
| PCIe x4 Slot D3 Cold | Selection | Disabled [0x0] ◀ default Renesas XHCI [0x1] Realtek SD Card Reader [0x2] ◀ performance TUNE Deep device-idle (D3Cold) power gating. Disabling lowers wake latency for that device (marginal perf); Enabled saves idle power. Default is fine for desktop use. |

### USB/Thunderbolt Configurations   (Form 0x17)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| PD USB4 Control Enable/Disable | Selection | Disabled [0x0] Enabled [0x1] ◀ default ◀ performance NEUTRAL USB port/controller enable — functional toggle, no performance impact. Leave default unless disabling unused ports for security/power. |
| USB4 Bus Reserved | Numeric | Min 0x0 · Max 0xFF · Default 0x60 ◀ default ◀ performance NEUTRAL USB port/controller enable — functional toggle, no performance impact. Leave default unless disabling unused ports for security/power. |
| USB4 IO Reserved (KB) | Numeric | Min 0x0 · Max 0xFF · Default 0x10 ◀ default ◀ performance NEUTRAL USB port/controller enable — functional toggle, no performance impact. Leave default unless disabling unused ports for security/power. |
| USB4 Non-Prefetch Memory Reserved | Numeric | Min 0x0 · Max 0xFFFF · Default 0x180 ◀ default ◀ performance NEUTRAL USB port/controller enable — functional toggle, no performance impact. Leave default unless disabling unused ports for security/power. |
| USB4 non-Prefetch MMIO align (0 ~ 65534 MB) | Numeric | Min 0x0 · Max 0xFFFF · Default 0x40 ◀ default ◀ performance NEUTRAL USB port/controller enable — functional toggle, no performance impact. Leave default unless disabling unused ports for security/power. |
| USB4 Prefetch Memory Reserved | Selection | 2MB [0x2] 4MB [0x4] 8MB [0x8] 16MB [0x10] 32MB [0x20] 64MB [0x40] 128MB [0x80] 256MB [0x100] 512MB [0x200] 1GB [0x400] 2GB [0x800] 4GB [0x1000] 8GB [0x2000] 16GB [0x4000] 32GB [0x8000] 64GB [0x10000] 128GB [0x20000] ◀ default ◀ performance NEUTRAL USB port/controller enable — functional toggle, no performance impact. Leave default unless disabling unused ports for security/power. |
| USB4 Prefetch MMIO align | Selection | 2MB [0x2] 4MB [0x4] 8MB [0x8] 16MB [0x10] 32MB [0x20] 64MB [0x40] 128MB [0x80] 256MB [0x100] 512MB [0x200] 1GB [0x400] 2GB [0x800] 4GB [0x1000] 8GB [0x2000] 16GB [0x4000] 32GB [0x8000] ◀ default 64GB [0x10000] ◀ performance NEUTRAL USB port/controller enable — functional toggle, no performance impact. Leave default unless disabling unused ports for security/power. |
| USB Camera Enable | Selection | Disabled [0x0] Enabled [0x1] ◀ default ◀ performance NEUTRAL USB port/controller enable — functional toggle, no performance impact. Leave default unless disabling unused ports for security/power. |
| USB Fingerprint or GBE MUX | Selection | Fingerprint [0x0] GBE [0x1] ◀ default ◀ performance NEUTRAL USB port/controller enable — functional toggle, no performance impact. Leave default unless disabling unused ports for security/power. |
| USB Fingerprint Enable | Selection | Disabled [0x0] ◀ default Enabled [0x1] ◀ performance NEUTRAL USB port/controller enable — functional toggle, no performance impact. Leave default unless disabling unused ports for security/power. |
| UCSI Support | Selection | Disabled [0x0] ◀ default Enabled [0x1] ◀ performance NEUTRAL USB port/controller enable — functional toggle, no performance impact. Leave default unless disabling unused ports for security/power. |
| UCSI tunnel location | Selection | UCSI tunnel at EC RAM [0x0] UCSI tunnel at MMIO 0xFEEC2000 [0x1] ◀ default ◀ performance NEUTRAL USB port/controller enable — functional toggle, no performance impact. Leave default unless disabling unused ports for security/power. |
| USBC Port Harware Disable Support | Selection | Disabled [0x0] ◀ default Enabled [0x1] ◀ performance NEUTRAL USB port/controller enable — functional toggle, no performance impact. Leave default unless disabling unused ports for security/power. |
| Reconfig Rebalance Resources by OS | Selection | Disabled [0x0] ◀ default Enabled [0x1] ◀ performance NEUTRAL USB4/Thunderbolt resource reservation — leave default unless a specific TB device needs more PCIe window; no general perf gain. |
| PD Thunderbolt3 Alt Mode | Selection | Enabled [0x0] ◀ default Disabled [0x1] ◀ performance NEUTRAL USB4/Thunderbolt resource reservation — leave default unless a specific TB device needs more PCIe window; no general perf gain. |
| USB4 ACPI _DEP Support | Selection | Disabled [0x0] ◀ default Enabled [0x1] ◀ performance NEUTRAL USB port/controller enable — functional toggle, no performance impact. Leave default unless disabling unused ports for security/power. |
| USB4 LTR Snoop/NonSnoop Scale | Numeric | Min 0x0 · Max 0x7 · Step 0x1 · Default 0x2 ◀ default ◀ performance NEUTRAL USB port/controller enable — functional toggle, no performance impact. Leave default unless disabling unused ports for security/power. |
| USB4 LTR Snoop/NonSnoop Value | Numeric | Min 0x1 · Max 0x3FF · Step 0x1 · Default 0xC8 ◀ default ◀ performance NEUTRAL USB port/controller enable — functional toggle, no performance impact. Leave default unless disabling unused ports for security/power. |
| Modify _UPC/_PLD settings for USB3.2-A vertical port (Requires board rework) | Selection | Disabled [0x0] ◀ default Enabled [0x1] ◀ performance NEUTRAL USB port/controller enable — functional toggle, no performance impact. Leave default unless disabling unused ports for security/power. |

### USB TypeC Security Mode Configurations   (Form 0x18)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| USB Type-C Security | Selection | Auto [0xF] ◀ default Mode 0 [0x0] Mode 1 [0x1] Mode 2 [0x2] ◀ performance NEUTRAL USB port/controller enable — functional toggle, no performance impact. Leave default unless disabling unused ports for security/power. |

### Graphics Configurations   (Form 0xF)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| EVAL Slot Power Enable | Selection | Disabled [0x0] Enabled [0x1] ◀ default ◀ performance NEUTRAL Device power-rail enable / timing — functional, no performance impact. Leave default. |
| EVAL CARD T-Diode Routing Select | Selection | APU SMBUS1 [0x0] EC [0x1] ◀ default ◀ performance NEUTRAL Device power-rail enable / timing — functional, no performance impact. Leave default. |
| Special Display Features | Selection | Disabled [0x0] HybridGraphics [0x4] ◀ default ◀ performance NEUTRAL Panel/display feature toggles — no compute-performance impact. |
| D3Cold Support | Selection | Disabled [0x0] ◀ default Enabled [0x1] Dummy D3Cold [0x2] ◀ performance TUNE Deep device-idle (D3Cold) power gating. Disabling lowers wake latency for that device (marginal perf); Enabled saves idle power. Default is fine for desktop use. |
| Discrete GPU Hotplug Mode | Selection | Basic Mode [0x0] Enhanced Mode [0x1] ◀ default Non-Hotplug Mode [0x2] ◀ performance NEUTRAL External/discrete-GPU or display-mux plumbing — no impact on the iGPU-only default config; leave default. |
| Discrete GPU D3Cold HPD Support | Selection | Disabled [0x0] ◀ default Enabled [0x1] ◀ performance TUNE Deep device-idle (D3Cold) power gating. Disabling lowers wake latency for that device (marginal perf); Enabled saves idle power. Default is fine for desktop use. |
| PME Turn Off Support | Selection | Disabled [0x0] Enabled [0x1] ◀ default ◀ performance TUNE Deep device-idle (D3Cold) power gating. Disabling lowers wake latency for that device (marginal perf); Enabled saves idle power. Default is fine for desktop use. |
| D3Cold Force Gen1 | Selection | Disabled [0x0] ◀ default Enabled [0x1] ◀ performance TUNE Deep device-idle (D3Cold) power gating. Disabling lowers wake latency for that device (marginal perf); Enabled saves idle power. Default is fine for desktop use. |
| SLOTPWR-PWREN Timing(ms) | Numeric | Min 0x0 · Max 0x32 · Step 0x1 · Default 0x1 ◀ default ◀ performance NEUTRAL Device power-rail enable / timing — functional, no performance impact. Leave default. |
| PWREN-RST Timing (ms) | Numeric | Min 0x0 · Max 0x96 · Step 0x1 · Default 0x50 ◀ default ◀ performance NEUTRAL Device power-rail enable / timing — functional, no performance impact. Leave default. |
| PERST-WAKEL23 Timing (ms) | Numeric | Min 0x1 · Max 0x1E · Default 0xA ◀ default ◀ performance NEUTRAL Device power-rail enable / timing — functional, no performance impact. Leave default. |
| DLACT-CFGACC Timing (ms) | Numeric | Min 0x1 · Max 0x64 · Default 0x64 ◀ default ◀ performance NEUTRAL Device power-rail enable / timing — functional, no performance impact. Leave default. |
| NVIDIA DGPU Power Enable | Selection | Disabled [0x0] ◀ default Enabled [0x1] ◀ performance NEUTRAL Device power-rail enable / timing — functional, no performance impact. Leave default. |
| Discrete GPU _DSM Function A | Selection | Disabled [0x0] ◀ default Enabled [0x1] ◀ performance NEUTRAL External/discrete-GPU or display-mux plumbing — no impact on the iGPU-only default config; leave default. |
| Discrete GPU _DSM Function B | Selection | Disabled [0x0] ◀ default Enabled [0x1] ◀ performance NEUTRAL External/discrete-GPU or display-mux plumbing — no impact on the iGPU-only default config; leave default. |
| Non-Eval Discrete GPU Support | Selection | Disabled [0x0] ◀ default Enabled [0x1] ◀ performance NEUTRAL External/discrete-GPU or display-mux plumbing — no impact on the iGPU-only default config; leave default. |
| Discrete GPU HPD Circuitry | Selection | OR Circuitry [0x0] ◀ default Pulse Circuitry [0x1] ◀ performance NEUTRAL External/discrete-GPU or display-mux plumbing — no impact on the iGPU-only default config; leave default. |
| Discrete GPU's USB Port | Selection | Keep Default Setting [0x0] ◀ default Disabled [0x1] ◀ performance NEUTRAL External/discrete-GPU or display-mux plumbing — no impact on the iGPU-only default config; leave default. |
| Discrete GPU's SSID/SVID | Selection | Keep Default Setting [0x0] Program by Vendor [0x1] ◀ default ◀ performance NEUTRAL External/discrete-GPU or display-mux plumbing — no impact on the iGPU-only default config; leave default. |
| Discrete GPU's VGA SSID/SVID | Numeric | Min 0x0 · Max 0xFFFFFFFF · Default 0x0 ◀ default ◀ performance NEUTRAL External/discrete-GPU or display-mux plumbing — no impact on the iGPU-only default config; leave default. |
| Discrete GPU's AUDIO SSID/SVID | Numeric | Min 0x0 · Max 0xFFFFFFFF · Default 0x0 ◀ default ◀ performance NEUTRAL External/discrete-GPU or display-mux plumbing — no impact on the iGPU-only default config; leave default. |
| Discrete GPU BOMACO Support | Selection | Disabled [0x0] ◀ default Enabled [0x1] ◀ performance NEUTRAL Device power-rail enable / timing — functional, no performance impact. Leave default. |
| MACO-PWR Timing (us) | Numeric | Min 0xA · Max 0xFF · Step 0x1 · Default 0xA ◀ default ◀ performance NEUTRAL Device power-rail enable / timing — functional, no performance impact. Leave default. |
| RST-MACO Timing (ms) | Numeric | Min 0x0 · Max 0xA · Step 0x1 · Default 0x0 ◀ default ◀ performance NEUTRAL Device power-rail enable / timing — functional, no performance impact. Leave default. |
| ATCS Function 9 Support | Selection | Disabled [0x0] ◀ default Enabled [0x1] ◀ performance NEUTRAL External/discrete-GPU or display-mux plumbing — no impact on the iGPU-only default config; leave default. |
| AC Maximum Performance Limit | Numeric | Min 0x0 · Max 0x64 · Default 0x0 ◀ default ◀ performance TUNE AC/DC power-profile cap. On this mains-powered mini-PC the AC profile applies — set the AC limits to the highest/Maximum-Performance tier; DC limits are irrelevant. |
| AC Better Performance Limit | Numeric | Min 0x0 · Max 0x64 · Default 0x0 ◀ default ◀ performance TUNE AC/DC power-profile cap. On this mains-powered mini-PC the AC profile applies — set the AC limits to the highest/Maximum-Performance tier; DC limits are irrelevant. |
| AC Better Battery Limit | Numeric | Min 0x0 · Max 0x64 · Default 0x0 ◀ default ◀ performance TUNE AC/DC power-profile cap. On this mains-powered mini-PC the AC profile applies — set the AC limits to the highest/Maximum-Performance tier; DC limits are irrelevant. |
| DC Maximum Performance Limit | Numeric | Min 0x0 · Max 0x64 · Default 0x0 ◀ default ◀ performance TUNE AC/DC power-profile cap. On this mains-powered mini-PC the AC profile applies — set the AC limits to the highest/Maximum-Performance tier; DC limits are irrelevant. |
| DC Better Performance Limit | Numeric | Min 0x0 · Max 0x64 · Default 0x0 ◀ default ◀ performance TUNE AC/DC power-profile cap. On this mains-powered mini-PC the AC profile applies — set the AC limits to the highest/Maximum-Performance tier; DC limits are irrelevant. |
| DC Better Battery Limit | Numeric | Min 0x0 · Max 0x64 · Default 0x0 ◀ default ◀ performance TUNE AC/DC power-profile cap. On this mains-powered mini-PC the AC profile applies — set the AC limits to the highest/Maximum-Performance tier; DC limits are irrelevant. |
| DC Battery Saver Limit | Numeric | Min 0x0 · Max 0x64 · Default 0x0 ◀ default ◀ performance TUNE AC/DC power-profile cap. On this mains-powered mini-PC the AC profile applies — set the AC limits to the highest/Maximum-Performance tier; DC limits are irrelevant. |
| BLINK LED | Selection | Disabled [0x0] Enabled [0x1] ◀ default GPIO 11 Output Low [0x2] GPIO 11 Output High [0x3] ◀ performance NEUTRAL Display/panel/LED configuration — no compute-performance impact. |
| ATIF Notify Command Code | Selection | Notify VGA 0x81 [0x0] ◀ default Notify VGA 0xD0 [0xD0] Notify VGA 0xD1 [0xD1] Notify VGA 0xD2 [0xD2] Notify VGA 0xD3 [0xD3] Notify VGA 0xD4 [0xD4] Notify VGA 0xD5 [0xD5] Notify VGA 0xD6 [0xD6] Notify VGA 0xD7 [0xD7] Notify VGA 0xD8 [0xD8] Notify VGA 0xD9 [0xD9] ◀ performance NEUTRAL External/discrete-GPU or display-mux plumbing — no impact on the iGPU-only default config; leave default. |
| ATIF Function 21 Support | Selection | Disabled [0x0] ◀ default Enabled [0x1] ◀ performance NEUTRAL External/discrete-GPU or display-mux plumbing — no impact on the iGPU-only default config; leave default. |
| External Graphics Port | Selection | Disabled [0x0] ◀ default Enabled [0x1] ◀ performance NEUTRAL External/discrete-GPU or display-mux plumbing — no impact on the iGPU-only default config; leave default. |
| Hide XConnect GUI | Selection | Disabled [0x0] ◀ default Enabled [0x1] ◀ performance NEUTRAL External/discrete-GPU or display-mux plumbing — no impact on the iGPU-only default config; leave default. |
| Run Time PM and D3 | Selection | Disabled [0x0] ◀ default Enabled [0x1] ◀ performance TUNE Deep device-idle (D3Cold) power gating. Disabling lowers wake latency for that device (marginal perf); Enabled saves idle power. Default is fine for desktop use. |
| Support ATIF ATPX | Selection | Disabled [0x0] ◀ default Enabled [0x1] ◀ performance NEUTRAL External/discrete-GPU or display-mux plumbing — no impact on the iGPU-only default config; leave default. |
| ATIF Function 22 Support | Selection | Disabled [0x0] ◀ default Undefined [0x1] Integrated Graphics [0x2] Discrete Graphics [0x3] ◀ performance NEUTRAL External/discrete-GPU or display-mux plumbing — no impact on the iGPU-only default config; leave default. |
| GPU Package Power Limit Value | Numeric | Min 0x0 · Max 0xFFFFFFFF · Default 0x0 ◀ default ◀ performance TUNE Within the shared APU budget, a higher GPU power slice favors graphics-bound work; it competes with CPU power on this single-budget APU. |
| ATIF Function 23 Support | Selection | Disabled [0x0] ◀ default Enabled [0x1] ◀ performance NEUTRAL External/discrete-GPU or display-mux plumbing — no impact on the iGPU-only default config; leave default. |
| Vari-Bright Maximum Performance | Numeric | Min 0x0 · Max 0x4 · Default 0x0 ◀ default ◀ performance NEUTRAL Display/panel/LED configuration — no compute-performance impact. |
| Vari-Bright Better Performance | Numeric | Min 0x0 · Max 0x4 · Default 0x1 ◀ default ◀ performance NEUTRAL Display/panel/LED configuration — no compute-performance impact. |
| Vari-Bright Better Battery | Numeric | Min 0x0 · Max 0x4 · Default 0x3 ◀ default ◀ performance NEUTRAL Display/panel/LED configuration — no compute-performance impact. |
| Vari-Bright Battery Saver | Numeric | Min 0x0 · Max 0x4 · Default 0x3 ◀ default ◀ performance NEUTRAL Display/panel/LED configuration — no compute-performance impact. |
| ATIF Function 24 Support | Selection | Disabled [0x0] Enabled [0x1] ◀ default ◀ performance NEUTRAL External/discrete-GPU or display-mux plumbing — no impact on the iGPU-only default config; leave default. |
| ISP Device number | Numeric | Min 0x0 · Max 0x64 · Step 0x1 · Default 0x0 ◀ default ◀ performance NEUTRAL External/discrete-GPU or display-mux plumbing — no impact on the iGPU-only default config; leave default. |
| Primary Video Adaptor | Selection | Int Graphics (IGD) [0x1] ◀ default Ext Graphics (PEG) [0x2] ◀ performance NEUTRAL Selects boot display device — no throughput impact; set to your preferred output. |
| Smart Mux Acpi Control | Selection | Disabled [0x0] ◀ default Enabled without _DEP [0x1] Enabled [0x3] ◀ performance NEUTRAL External/discrete-GPU or display-mux plumbing — no impact on the iGPU-only default config; leave default. |
| Display Panel Multiplexer | Numeric | Min 0x0 · Max 0x7 · Default 0x0 ◀ default ◀ performance NEUTRAL External/discrete-GPU or display-mux plumbing — no impact on the iGPU-only default config; leave default. |
| Smart Mux _HID Selection | Selection | SMUX1234 [0x0] ◀ default MSFT0007 [0x1] ◀ performance NEUTRAL External/discrete-GPU or display-mux plumbing — no impact on the iGPU-only default config; leave default. |
| Smart Mux MDM Support Level | Selection | No Support [0x0] ◀ default Development Support [0x1] Experimental Support [0x2] Full Support [0x3] ◀ performance NEUTRAL External/discrete-GPU or display-mux plumbing — no impact on the iGPU-only default config; leave default. |
| Smart Mux First Connected GPU | Selection | Integrated Graphics [0x0] ◀ default Discrete Graphics [0x1] ◀ performance NEUTRAL External/discrete-GPU or display-mux plumbing — no impact on the iGPU-only default config; leave default. |
| Smart Mux ACPI Method Location | Selection | Under Mux [0x1] Under dGPU [0x2] ◀ default ◀ performance NEUTRAL External/discrete-GPU or display-mux plumbing — no impact on the iGPU-only default config; leave default. |
| Verify PEI GOP | Selection | Disabled [0x0] ◀ default Enabled [0x1] ◀ performance NEUTRAL External/discrete-GPU or display-mux plumbing — no impact on the iGPU-only default config; leave default. |
| Ultra mode support | Selection | Disabled [0x0] ◀ default Enabled [0x1] ◀ performance NEUTRAL External/discrete-GPU or display-mux plumbing — no impact on the iGPU-only default config; leave default. |

### Display Configurations   (Form 0x10)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| DP0 | Selection | Default [0x0] ◀ default EDP display [0x1] DP display [0x2] HDMI display [0x3] ◀ performance NEUTRAL Display/panel/LED configuration — no compute-performance impact. |
| Adjust DP Caps | Selection | Soc Default [0x0] ◀ default Cap override [0x1] ◀ performance NEUTRAL Display/panel/LED configuration — no compute-performance impact. |
| DP0 Display Caps | Numeric | Min 0x0 · Max 0xFFFFFFFF · Default 0xFFFFFFFF ◀ default ◀ performance NEUTRAL Display/panel/LED configuration — no compute-performance impact. |
| DP1 | Selection | Default [0x0] ◀ default EDP display [0x1] DP display [0x2] HDMI display [0x3] ◀ performance NEUTRAL Display/panel/LED configuration — no compute-performance impact. |
| Adjust DP Caps | Selection | Soc Default [0x0] ◀ default Cap override [0x1] ◀ performance NEUTRAL Display/panel/LED configuration — no compute-performance impact. |
| DP1 Display Caps | Numeric | Min 0x0 · Max 0xFFFFFFFF · Default 0xFFFFFFFF ◀ default ◀ performance NEUTRAL Display/panel/LED configuration — no compute-performance impact. |
| DP2 | Selection | Default [0x0] ◀ default EDP display [0x1] HDMI display [0x3] DP with TypeC display [0x4] DP without TypeC display [0x5] ◀ performance NEUTRAL Display/panel/LED configuration — no compute-performance impact. |
| Adjust DP Caps | Selection | Soc Default [0x0] ◀ default Cap override [0x1] ◀ performance NEUTRAL Display/panel/LED configuration — no compute-performance impact. |
| DP2 Display Caps | Numeric | Min 0x0 · Max 0xFFFFFFFF · Default 0xFFFFFFFF ◀ default ◀ performance NEUTRAL Display/panel/LED configuration — no compute-performance impact. |
| DP3 | Selection | Default [0x0] ◀ default EDP display [0x1] HDMI display [0x3] DP with TypeC display [0x4] DP without TypeC display [0x5] ◀ performance NEUTRAL Display/panel/LED configuration — no compute-performance impact. |
| Adjust DP Caps | Selection | Soc Default [0x0] ◀ default Cap override [0x1] ◀ performance NEUTRAL Display/panel/LED configuration — no compute-performance impact. |
| DP3 Display Caps | Numeric | Min 0x0 · Max 0xFFFFFFFF · Default 0xFFFFFFFF ◀ default ◀ performance NEUTRAL Display/panel/LED configuration — no compute-performance impact. |
| DP4 | Selection | Default [0x0] ◀ default EDP display [0x1] HDMI display [0x3] DP with TypeC display [0x4] DP without TypeC display [0x5] ◀ performance NEUTRAL Display/panel/LED configuration — no compute-performance impact. |
| Adjust DP Caps | Selection | Soc Default [0x0] ◀ default Cap override [0x1] ◀ performance NEUTRAL Display/panel/LED configuration — no compute-performance impact. |
| DP4 Display Caps | Numeric | Min 0x0 · Max 0xFFFFFFFF · Default 0xFFFFFFFF ◀ default ◀ performance NEUTRAL Display/panel/LED configuration — no compute-performance impact. |
| EDP priority | Selection | EDP0 [0x0] ◀ default EDP1 [0x1] ◀ performance NEUTRAL Display/panel/LED configuration — no compute-performance impact. |

### Audio Configurations   (Form 0x11)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| Wake On Voice | Selection | Disabled [0x0] ◀ default Enabled [0x1] ◀ performance NEUTRAL Wake-source/RTC option — no performance impact; set per convenience. |
| ACP Power Gating | Selection | Disabled [0x0] Enabled [0x1] ◀ default ◀ performance KEEP Enabled (Auto). Power-gates the audio co-processor when idle — frees power budget, no downside. |
| Verb Table Select | Selection | ALC245 Crb Default: SVID/SSID: 1022/D670 [0x0] ◀ default ALC245_05112020 : SVID/SSID: 10EC/1208 [0x1] ALC256_06082020 : SVID/SSID: 10EC/1208 [0x2] ALC623 : SVID/SSID: 10EC/1208 [0x3] SN614X : SVID/SSID: 1022/D595 [0x4] CX11970EVK : SVID/SSID: 14F1/0101 [0x5] ◀ performance NEUTRAL Serial/debug interface — leave Disabled/default. Debug output can slightly slow boot; no runtime performance benefit. |
| USB Sideband Audio | Selection | Disabled [0x0] ◀ default Enabled [0x1] ◀ performance NEUTRAL USB port/controller enable — functional toggle, no performance impact. Leave default unless disabling unused ports for security/power. |

### I2C Configurations   (Form 0x12)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| Touch Panel Support | Selection | Under I2C 0 Bus [0x0] Under I2C 1 Bus [0x1] Under I2C 2 Bus [0x2] Under I2C 3 Bus [0x3] ELAN TSP Under I2C 0 Bus [0x10] ELAN TSP Under I2C 1 Bus [0x11] ELAN TSP Under I2C 2 Bus [0x12] ELAN TSP Under I2C 3 Bus [0x13] Under USB 6 Bus [0x4] Disabled [0x5] ◀ default ◀ performance NEUTRAL Peripheral/sensor enable — no compute-performance impact; set per hardware need. |
| Touch Pad Support | Selection | Under I2C 0 Bus [0x0] Under I2C 1 Bus [0x1] Under I2C 2 Bus [0x2] Under I2C 3 Bus [0x3] Disabled [0x4] ◀ default ◀ performance NEUTRAL Peripheral/sensor enable — no compute-performance impact; set per hardware need. |
| Touchpad Slave address | Numeric | Min 0x0 · Max 0xFF · Default 0x2C ◀ default ◀ performance NEUTRAL Peripheral/sensor enable — no compute-performance impact; set per hardware need. |
| Touchpad HID Descriptor address | Numeric | Min 0x0 · Max 0xFF · Default 0x20 ◀ default ◀ performance NEUTRAL Peripheral/sensor enable — no compute-performance impact; set per hardware need. |
| Nfc Support | Selection | Under I2C 0 Bus [0x0] Under I2C 1 Bus [0x1] Under I2C 2 Bus [0x2] Under I2C 3 Bus [0x3] Disabled [0x4] ◀ default ◀ performance NEUTRAL Peripheral/sensor enable — no compute-performance impact; set per hardware need. |
| MITT/WITT Selection | Selection | MITT Only [0x0] WITT Only [0x1] Both disable [0x2] ◀ default ◀ performance NEUTRAL Serial/debug interface — leave Disabled/default. Debug output can slightly slow boot; no runtime performance benefit. |
| I2C MUX | Selection | Disabled [0x0] Enabled [0x1] ◀ default ◀ performance NEUTRAL Peripheral/sensor enable — no compute-performance impact; set per hardware need. |
| SMBUS0 Buffer | Selection | Disabled [0x0] Enabled [0x1] ◀ default ◀ performance NEUTRAL Peripheral/sensor enable — no compute-performance impact; set per hardware need. |
| SMBUS1 Buffer | Selection | Disabled [0x0] Enabled [0x1] ◀ default ◀ performance NEUTRAL Peripheral/sensor enable — no compute-performance impact; set per hardware need. |

### Thermal Configurations   (Form 0x13)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| APU PROCHOT# setting | Selection | Disable APU_PROCHOT# [0x0] Enable APU_PROCHOT# in pure-DC case [0x1] Enable APU_PROCHOT# in pure-AC case [0x2] Enable APU_PROCHOT# in either pure-DC or pure-AC case (not AC+DC) [0x3] ◀ default ◀ performance NEUTRAL PROCHOT pin behavior — leave default (thermal safety). |
| AMD DPTC interface | Selection | Disabled [0x0] ◀ default Enabled [0x1] ◀ performance NEUTRAL Dynamic power/thermal control interface — leave Auto. |
| STT sensor reporting | Selection | Disabled [0x0] ◀ default Report onboard sensors [0x1] Report onboard + eval card sensors [0x2] ◀ performance NEUTRAL Telemetry reporting toggle — no performance impact. |

### MP2 Configurations   (Form 0x14)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| Power Sensors Routing Select | Selection | WALLE lite PDT [0x0] ◀ default WALLE lite PM log [0x1] ◀ performance NEUTRAL Peripheral/sensor enable — no compute-performance impact; set per hardware need. |
| MP2 FW Selection | Selection | MP2_SFH [0x0] ◀ default MP2_WalleLite [0x1] ◀ performance NEUTRAL Peripheral/sensor enable — no compute-performance impact; set per hardware need. |
| Turn off Xtal (S3/S5) | Selection | Disabled [0x0] Enabled [0x1] ◀ default ◀ performance NEUTRAL Wake-source/RTC option — no performance impact; set per convenience. |
| Sensor Fusion User Mode Driver | Selection | Disabled [0x0] Enabled [0x1] ◀ default ◀ performance NEUTRAL Peripheral/sensor enable — no compute-performance impact; set per hardware need. |
| Reserved Memory For MP2 | Selection | Disabled [0xFF] 128 KB [0x0] ◀ default 256 KB [0x1] 512 KB [0x2] 1 MB [0x3] 2 MB [0x4] 4 MB [0x5] 8 MB [0x6] 16 MB [0x7] 32 MB [0x8] 64 MB [0x9] ◀ performance NEUTRAL Peripheral/sensor enable — no compute-performance impact; set per hardware need. |
| MP2 Privilege Mode | Selection | Disabled [0x0] ◀ default Enabled [0x1] ◀ performance NEUTRAL Peripheral/sensor enable — no compute-performance impact; set per hardware need. |
| Accelerator Sensor | Selection | Disabled [0x0] ◀ default Enabled [0x1] ◀ performance NEUTRAL Peripheral/sensor enable — no compute-performance impact; set per hardware need. |
| Magnet Sensor | Selection | Disabled [0x0] ◀ default Enabled [0x1] ◀ performance NEUTRAL Peripheral/sensor enable — no compute-performance impact; set per hardware need. |
| SRA Sensor | Selection | Disabled [0x0] ◀ default Enabled [0x1] ◀ performance NEUTRAL Peripheral/sensor enable — no compute-performance impact; set per hardware need. |
| Light Sensor | Selection | Disabled [0x0] ◀ default Enabled [0x1] ◀ performance NEUTRAL Peripheral/sensor enable — no compute-performance impact; set per hardware need. |
| Proximity Sensor | Selection | Disabled [0x0] ◀ default Enabled [0x1] ◀ performance NEUTRAL Peripheral/sensor enable — no compute-performance impact; set per hardware need. |
| Wake On Human Presence detection WA | Selection | Disabled [0x0] ◀ default Enabled [0x1] ◀ performance NEUTRAL Wake-source/RTC option — no performance impact; set per convenience. |
| Gyroscope Sensor | Selection | Disabled [0x0] ◀ default Enabled [0x1] ◀ performance NEUTRAL Peripheral/sensor enable — no compute-performance impact; set per hardware need. |
| HPD-Lite Sensor | Selection | Disabled [0x0] ◀ default Enabled [0x1] ◀ performance NEUTRAL Peripheral/sensor enable — no compute-performance impact; set per hardware need. |

### EC/PD Configurations   (Form 0x15)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| Charger mode BYPASS | Selection | Disabled [0x0] ◀ default Enabled [0x1] ◀ performance NEUTRAL Display/panel/LED configuration — no compute-performance impact. |
| KBC Support | Selection | Disabled [0x0] Enabled [0x1] ◀ default ◀ performance NEUTRAL Peripheral/sensor enable — no compute-performance impact; set per hardware need. |
| AcDcSwitch | Selection | Disabled [0x0] ◀ default Enabled [0x1] ◀ performance NEUTRAL AC/DC source signaling — no performance impact on a desktop mini-PC. |
| AmdDcTimer | Numeric | Min 0x0 · Max 0xFF · Default 0x0 ◀ default ◀ performance TUNE AC/DC power-profile cap. On this mains-powered mini-PC the AC profile applies — set the AC limits to the highest/Maximum-Performance tier; DC limits are irrelevant. |
| AmdAcTimer | Numeric | Min 0x0 · Max 0xFF · Default 0x0 ◀ default ◀ performance TUNE AC/DC power-profile cap. On this mains-powered mini-PC the AC profile applies — set the AC limits to the highest/Maximum-Performance tier; DC limits are irrelevant. |
| Fake DC Level | Numeric | Min 0x0 · Max 0x64 · Default 0x5E ◀ default ◀ performance TUNE AC/DC power-profile cap. On this mains-powered mini-PC the AC profile applies — set the AC limits to the highest/Maximum-Performance tier; DC limits are irrelevant. |
| VDD adjust for BoardDesign | Selection | Disabled [0x0] ◀ default Enabled [0x1] ◀ performance NEUTRAL Board-design voltage rail — factory-set for the soldered components; leave Auto. Not a performance lever. |
| VDDIO_MEM_S3 voltage | Selection | voltage (1.05V) [0x0] ◀ default voltage (+10mv) [0x1] voltage (+20mv) [0x2] voltage (+30mv) [0x3] voltage (+40mv) [0x4] voltage (+50mv) [0x5] voltage (+60mv) [0x6] voltage (+70mv) [0x7] voltage (+80mv) [0x8] voltage (+90mv) [0x9] voltage (+100mv) [0xA] voltage (-10mv) [0xB] voltage (-20mv) [0xC] voltage (-30mv) [0xD] voltage (-40mv) [0xE] voltage (-50mv) [0xF] voltage (-60mv) [0x10] voltage (-70mv) [0x11] voltage (-80mv) [0x12] voltage (-90mv) [0x13] voltage (-100mv) [0x14] ◀ performance NEUTRAL Board-design voltage rail — factory-set for the soldered components; leave Auto. Not a performance lever. |
| VDD MISC S5 voltage | Selection | voltage (0.75V) [0x0] ◀ default voltage (+10mv) [0x1] voltage (+20mv) [0x2] voltage (+30mv) [0x3] voltage (+40mv) [0x4] voltage (+50mv) [0x5] voltage (+60mv) [0x6] voltage (+70mv) [0x7] voltage (+80mv) [0x8] voltage (+90mv) [0x9] voltage (+100mv) [0xA] voltage (-10mv) [0xB] voltage (-20mv) [0xC] voltage (-30mv) [0xD] voltage (-40mv) [0xE] voltage (-50mv) [0xF] voltage (-60mv) [0x10] voltage (-70mv) [0x11] voltage (-80mv) [0x12] voltage (-90mv) [0x13] voltage (-100mv) [0x14] ◀ performance NEUTRAL Board-design voltage rail — factory-set for the soldered components; leave Auto. Not a performance lever. |
| VDD_MISC voltage | Selection | voltage (0.75V) [0x0] ◀ default voltage (+10mv) [0x1] voltage (+20mv) [0x2] voltage (+30mv) [0x3] voltage (+40mv) [0x4] voltage (+50mv) [0x5] voltage (+60mv) [0x6] voltage (+70mv) [0x7] voltage (+80mv) [0x8] voltage (+90mv) [0x9] voltage (+100mv) [0xA] voltage (-10mv) [0xB] voltage (-20mv) [0xC] voltage (-30mv) [0xD] voltage (-40mv) [0xE] voltage (-50mv) [0xF] voltage (-60mv) [0x10] voltage (-70mv) [0x11] voltage (-80mv) [0x12] voltage (-90mv) [0x13] voltage (-100mv) [0x14] ◀ performance NEUTRAL Board-design voltage rail — factory-set for the soldered components; leave Auto. Not a performance lever. |
| VDD11 voltage | Selection | voltage (1.1V) [0x0] ◀ default voltage (+10mv) [0x1] voltage (+20mv) [0x2] voltage (+30mv) [0x3] voltage (+40mv) [0x4] voltage (+50mv) [0x5] voltage (+60mv) [0x6] voltage (+70mv) [0x7] voltage (+80mv) [0x8] voltage (+90mv) [0x9] voltage (+100mv) [0xA] voltage (-10mv) [0xB] voltage (-20mv) [0xC] voltage (-30mv) [0xD] voltage (-40mv) [0xE] voltage (-50mv) [0xF] voltage (-60mv) [0x10] voltage (-70mv) [0x11] voltage (-80mv) [0x12] voltage (-90mv) [0x13] voltage (-100mv) [0x14] ◀ performance NEUTRAL Board-design voltage rail — factory-set for the soldered components; leave Auto. Not a performance lever. |
| 1V8 ALW voltage | Selection | voltage(1.8V) [0x0] ◀ default voltage (+10mv) [0x1] voltage (+20mv) [0x2] voltage (+30mv) [0x3] voltage (+40mv) [0x4] voltage (+50mv) [0x5] voltage (+60mv) [0x6] voltage (+70mv) [0x7] voltage (+80mv) [0x8] voltage (+90mv) [0x9] voltage (+100mv) [0xA] voltage (-10mv) [0xB] voltage (-20mv) [0xC] voltage (-30mv) [0xD] voltage (-40mv) [0xE] voltage (-50mv) [0xF] voltage (-60mv) [0x10] voltage (-70mv) [0x11] voltage (-80mv) [0x12] voltage (-90mv) [0x13] voltage (-100mv) [0x14] ◀ performance NEUTRAL Board-design voltage rail — factory-set for the soldered components; leave Auto. Not a performance lever. |
| VDD2L voltage | Selection | voltage(0.9V) [0x0] ◀ default voltage (+10mv) [0x1] voltage (+20mv) [0x2] voltage (+30mv) [0x3] voltage (+40mv) [0x4] voltage (+50mv) [0x5] voltage (+60mv) [0x6] voltage (+70mv) [0x7] voltage (+80mv) [0x8] voltage (+90mv) [0x9] voltage (+100mv) [0xA] voltage (-10mv) [0xB] voltage (-20mv) [0xC] voltage (-30mv) [0xD] voltage (-40mv) [0xE] voltage (-50mv) [0xF] voltage (-60mv) [0x10] voltage (-70mv) [0x11] voltage (-80mv) [0x12] voltage (-90mv) [0x13] voltage (-100mv) [0x14] ◀ performance NEUTRAL Board-design voltage rail — factory-set for the soldered components; leave Auto. Not a performance lever. |
| VDDQ voltage | Selection | voltage(0.5V) [0x0] ◀ default voltage (+10mv) [0x1] voltage (+20mv) [0x2] voltage (+30mv) [0x3] voltage (+40mv) [0x4] voltage (+50mv) [0x5] voltage (+60mv) [0x6] voltage (+70mv) [0x7] voltage (+80mv) [0x8] voltage (+90mv) [0x9] voltage (+100mv) [0xA] voltage (-10mv) [0xB] voltage (-20mv) [0xC] voltage (-30mv) [0xD] voltage (-40mv) [0xE] voltage (-50mv) [0xF] voltage (-60mv) [0x10] voltage (-70mv) [0x11] voltage (-80mv) [0x12] voltage (-90mv) [0x13] voltage (-100mv) [0x14] ◀ performance NEUTRAL Board-design voltage rail — factory-set for the soldered components; leave Auto. Not a performance lever. |
| VDD MEM voltage | Selection | voltage(0.78V) [0x0] ◀ default voltage (+10mv) [0x1] voltage (+20mv) [0x2] voltage (+30mv) [0x3] voltage (+40mv) [0x4] voltage (+50mv) [0x5] voltage (+60mv) [0x6] voltage (+70mv) [0x7] voltage (+80mv) [0x8] voltage (+90mv) [0x9] voltage (+100mv) [0xA] voltage (-10mv) [0xB] voltage (-20mv) [0xC] voltage (-30mv) [0xD] voltage (-40mv) [0xE] voltage (-50mv) [0xF] voltage (-60mv) [0x10] voltage (-70mv) [0x11] voltage (-80mv) [0x12] voltage (-90mv) [0x13] voltage (-100mv) [0x14] ◀ performance NEUTRAL Board-design voltage rail — factory-set for the soldered components; leave Auto. Not a performance lever. |
| POST LED on enable | Selection | Disabled [0x0] Enabled [0x1] ◀ default ◀ performance NEUTRAL Display/panel/LED configuration — no compute-performance impact. |
| Smart Mux Support | Selection | Disabled [0x0] ◀ default Hybrid Graphics Mode [0x1] Discrete Mode [0x2] Smart Mux 1.5 [0x3] Smart Mux 2.0 [0x4] ◀ performance NEUTRAL External/discrete-GPU or display-mux plumbing — no impact on the iGPU-only default config; leave default. |
| EC UDC Log Support | Selection | Disabled [0x0] Enabled [0x1] ◀ default ◀ performance NEUTRAL Embedded-controller/debug logging — no performance impact; leave default. |
| Retiemr I2C0/SMBUS1 Enable | Selection | Disabled [0x0] ◀ default Enabled [0x1] ◀ performance NEUTRAL Peripheral/sensor enable — no compute-performance impact; set per hardware need. |
| Retimer FMP driver support | Selection | Disabled [0x0] ◀ default I2c Retimer FMP driver without force power [0x1] I2c Retimer FMP driver with force power [0x2] SBU Retimer FMP driver without force power [0x3] SBU Retimer FMP driver with force power [0x4] ◀ performance NEUTRAL PCIe/USB retimer config — leave default; signal-integrity helper, no user perf setting. |

### Debug Configurations   (Form 0x16)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| Serial Debug Message Under OS | Selection | Disabled [0x0] ◀ default Enabled [0x1] ◀ performance NEUTRAL Serial/debug interface — leave Disabled/default. Debug output can slightly slow boot; no runtime performance benefit. |
| Debug Print In ASL | Selection | Disabled [0x0] ◀ default Enabled [0x1] ◀ performance NEUTRAL Serial/debug interface — leave Disabled/default. Debug output can slightly slow boot; no runtime performance benefit. |
| ISP Camera Module List | Selection | Disabled [0x0] ◀ default Module 1 [0x1] Module 2 [0x2] Module 3 [0x3] Module 4 [0x4] Module 5 [0x5] Module 6 [0x6] Module 7 [0x7] Module 8 [0x8] Module 9 [0x9] Module 10 [0xA] Module 11 [0xB] Module 12 [0xC] Module 13 [0xD] ◀ performance NEUTRAL External/discrete-GPU or display-mux plumbing — no impact on the iGPU-only default config; leave default. |
| UEFI ISP I2c support | Selection | Disabled [0x0] ◀ default Enabled [0x1] ◀ performance NEUTRAL External/discrete-GPU or display-mux plumbing — no impact on the iGPU-only default config; leave default. |
| Debug Data Preserve | Selection | Disabled [0x0] ◀ default Enabled [0x1] ◀ performance NEUTRAL Serial/debug interface — leave Disabled/default. Debug output can slightly slow boot; no runtime performance benefit. |
| Time for Blank Screen (10ms) | Numeric | Min 0x0 · Max 0x3E8 · Step 0x1 · Default 0xC8 ◀ default ◀ performance NEUTRAL Display/panel/LED configuration — no compute-performance impact. |

# AMD Overclocking (AOD)

AMD Overclocking (AOD) — the manual overclocking and memory-timing tree reached via Advanced ‣ AMD Overclocking. Exposes manual CPU/iGPU frequency and voltage, Curve Optimizer, Precision Boost Overdrive (PBO), and the full DDR/Infinity-Fabric timing set.

Form-set GUID: E39AC631-48ED-445E-9042-55B34F405844     ·     Forms: 28     ·     Configurable settings: 153

### AMD Overclocking Option   (Form 0x98)

Submenus: Accept  ‣  Decline

### AMD Overclocking   (Form 0x100)

Submenus: Manual CPU Overclocking  ‣  Manual iGPU Overclocking  ‣  DDR and Infinity Fabric Frequency/Timings  ‣  Precision Boost Overdrive  ‣  VDDP Voltage Control  ‣  SoC/Uncore OC Mode  ‣  SoC Voltage  ‣  LN2 Mode  ‣  VDD Misc  ‣  LCLK Frequency Control  ‣  Onboard Voltage Control

### Manual CPU Overclocking   (Form 0x101)

Submenus: CPU Core Count Control

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| CPU Frequency | Numeric | Min 0x0 · Max 0xFFFFFFFF · Step 0x5 · Default 0x0 ◀ default ◀ performance TUNE Read-only/Auto target. Use PBO + Curve Optimizer rather than a fixed all-core frequency (which disables opportunistic boost). |
| CPU Voltage | Numeric | Min 0x0 · Max 0xAF0 · Default 0x0 ◀ default ◀ performance TUNE Auto. Manual Vcore disables adaptive boost; undervolt via Curve Optimizer instead. |

### CPU Core Count Control   (Form 0x10C)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| SMT Control | Selection | Auto [0x1] ◀ default Disable [0x0] ◀ performance KEEP Enabled (Auto). Simultaneous multithreading gives ~20-30% multi-thread throughput; disable only for niche per-core-latency cases. |
| Down Core Mode | Selection | Enable [0x1] Disable [0x0] ◀ default ◀ performance KEEP Auto (all cores active). Disabling cores reduces throughput. |

### Manual iGPU Overclocking   (Form 0x102)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| GFX Clock Frequency | Numeric | Min 0x0 · Max 0xFFFF · Default 0x0 ◀ default ◀ performance TUNE Auto. Fixing iGPU clock removes dynamic boost; prefer GPU boost-override + GFX curve. |
| GFX Voltage | Numeric | Min 0x0 · Max 0xAF0 · Default 0x0 ◀ default ◀ performance TUNE Auto. Manual iGPU voltage is expert-only; undervolt via GFX Curve Optimizer. |

### DDR and Infinity Fabric Frequency/Timings   (Form 0x103)

Submenus: DDR Options  ‣  Infinity Fabric Frequency and Dividers

### DDR Options   (Form 0x110)

Submenus: DDR Timing Configuration  ‣  DDR Bus Configuration  ‣  DDR Controller Configuration

### DDR Timing Configuration   (Form 0x112)

Submenus: DDR SPD Timing  ‣  DDR Non-SPD Timing

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| Active Memory Timing Settings | Selection | Auto [0xFF] ◀ default Enabled [0x1] ◀ performance NEUTRAL Memory test/training diagnostic — not a runtime performance setting. Leave default (Disabled). |
| Memory Target Speed | Selection | Auto [0xFFFF] ◀ default 2000 MTs [0x7D0] 2400 MTs [0x960] 3200 MTs [0xC80] 3400 MTs [0xD48] 3600 MTs [0xE10] 3800 MTs [0xED8] 4000 MTs [0xFA0] 4200 MTs [0x1068] 4400 MTs [0x1130] 4600 MTs [0x11F8] 4800 MTs [0x12C0] 5000 MTs [0x1388] 5200 MTs [0x1450] 5400 MTs [0x1518] 5600 MTs [0x15E0] 5800 MTs [0x16A8] 6000 MTs [0x1770] 6200 MTs [0x1838] 6400 MTs [0x1900] 6600 MTs [0x19C8] 6800 MTs [0x1A90] 7000 MTs [0x1B58] 7200 MTs [0x1C20] 7400 MTs [0x1CE8] 7600 MTs [0x1DB0] 7800 MTs [0x1E78] 8000 MTs [0x1F40] 8200 MTs [0x2008] 8400 MTs [0x20D0] 8600 MTs [0x2198] 8800 MTs [0x2260] 9000 MTs [0x2328] 9200 MTs [0x23F0] 9400 MTs [0x24B8] 9600 MTs [0x2580] 9800 MTs [0x2648] 10000 MTs [0x2710] 10200 MTs [0x27D8] 10400 MTs [0x28A0] 10600 MTs [0x2968] 10800 MTs [0x2A30] 11000 MTs [0x2AF8] 11200 MTs [0x2BC0] 11400 MTs [0x2C88] 11600 MTs [0x2D50] 11800 MTs [0x2E18] 12000 MTs [0x2EE0] ◀ performance TUNE Keep at the soldered LPDDR5X rated speed (8000 MT/s). This memory is not user-replaceable and already runs at spec. |

### DDR SPD Timing   (Form 0x11B)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| Tcl Ctrl | Selection | Auto [0x0] ◀ default Manual [0x1] ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| Tcl | Numeric | Min 0x16 · Max 0x3E · Step 0x2 · Default 0x16 ◀ default ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| Trcd Ctrl | Selection | Auto [0x0] ◀ default Manual [0x1] ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| Trcd | Numeric | Min 0x8 · Max 0x3E · Default 0x8 ◀ default ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| Trp Ctrl | Selection | Auto [0x0] ◀ default Manual [0x1] ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| Trp | Numeric | Min 0x8 · Max 0x3E · Default 0x8 ◀ default ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| Tras Ctrl | Selection | Auto [0x0] ◀ default Manual [0x1] ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| Tras | Numeric | Min 0x1E · Max 0x7E · Default 0x1E ◀ default ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| Trc Ctrl | Selection | Auto [0x0] ◀ default Manual [0x1] ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| Trc | Numeric | Min 0x20 · Max 0xFF · Default 0x20 ◀ default ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| Twr Ctrl | Selection | Auto [0x0] ◀ default Manual [0x1] ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| Twr | Numeric | Min 0x30 · Max 0x60 · Default 0x30 ◀ default ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| Trfc1 Ctrl | Selection | Auto [0x0] ◀ default Manual [0x1] ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| Trfc1 | Numeric | Min 0x32 · Max 0xFFF · Default 0x32 ◀ default ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| Trfc2 Ctrl | Selection | Auto [0x0] ◀ default Manual [0x1] ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| Trfc2 | Numeric | Min 0x32 · Max 0xFFF · Default 0x32 ◀ default ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| TrfcSb Ctrl | Selection | Auto [0x0] ◀ default Manual [0x1] ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| TrfcSb | Numeric | Min 0x32 · Max 0x7FF · Default 0x32 ◀ default ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| Trtp Ctrl | Selection | Auto [0x0] ◀ default Manual [0x1] ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| Trtp | Numeric | Min 0x5 · Max 0x1F · Default 0x9 ◀ default ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| TrrdL Ctrl | Selection | Auto [0x0] ◀ default Manual [0x1] ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| TrrdL | Numeric | Min 0x4 · Max 0x20 · Default 0x4 ◀ default ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| TrrdS Ctrl | Selection | Auto [0x0] ◀ default Manual [0x1] ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| TrrdS | Numeric | Min 0x4 · Max 0x14 · Default 0x4 ◀ default ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| Tfaw Ctrl | Selection | Auto [0x0] ◀ default Manual [0x1] ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| Tfaw | Numeric | Min 0x14 · Max 0x50 · Default 0x1A ◀ default ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| TwtrL Ctrl | Selection | Auto [0x0] ◀ default Manual [0x1] ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| TwtrL | Numeric | Min 0x8 · Max 0x30 · Default 0x8 ◀ default ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| TwtrS Ctrl | Selection | Auto [0x0] ◀ default Manual [0x1] ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| TwtrS | Numeric | Min 0x2 · Max 0x10 · Default 0x3 ◀ default ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |

### DDR Non-SPD Timing   (Form 0x11C)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| TrdrdScL Ctrl | Selection | Auto [0x0] ◀ default Manual [0x1] ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| TrdrdScL | Numeric | Min 0x1 · Max 0xF · Default 0x1 ◀ default ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| TrdrdSc Ctrl | Selection | Auto [0x0] ◀ default Manual [0x1] ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| TrdrdSc | Numeric | Min 0x1 · Max 0xF · Default 0x1 ◀ default ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| TrdrdSd Ctrl | Selection | Auto [0x0] ◀ default Manual [0x1] ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| TrdrdSd | Numeric | Min 0x1 · Max 0xF · Default 0x3 ◀ default ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| TrdrdDd Ctrl | Selection | Auto [0x0] ◀ default Manual [0x1] ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| TrdrdDd | Numeric | Min 0x1 · Max 0xF · Default 0x3 ◀ default ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| TwrwrScL Ctrl | Selection | Auto [0x0] ◀ default Manual [0x1] ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| TwrwrScL | Numeric | Min 0x1 · Max 0x3F · Default 0x1 ◀ default ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| TwrwrSc Ctrl | Selection | Auto [0x0] ◀ default Manual [0x1] ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| TwrwrSc | Numeric | Min 0x1 · Max 0xF · Default 0x1 ◀ default ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| TwrwrSd Ctrl | Selection | Auto [0x0] ◀ default Manual [0x1] ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| TwrwrSd | Numeric | Min 0x1 · Max 0xF · Default 0x3 ◀ default ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| TwrwrDd Ctrl | Selection | Auto [0x0] ◀ default Manual [0x1] ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| TwrwrDd | Numeric | Min 0x1 · Max 0xF · Default 0x3 ◀ default ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| Twrrd Ctrl | Selection | Auto [0x0] ◀ default Manual [0x1] ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| Twrrd | Numeric | Min 0x1 · Max 0xF · Default 0x5 ◀ default ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| Trdwr Ctrl | Selection | Auto [0x0] ◀ default Manual [0x1] ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |
| Trdwr | Numeric | Min 0x1 · Max 0x1F · Default 0x5 ◀ default ◀ performance TUNE DRAM sub-timing. On soldered LPDDR5X these are factory-trained to spec; tightening risks instability with little headroom. Leave Auto unless you are an experienced memory overclocker validating each change. |

### DDR Bus Configuration   (Form 0x113)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| Processor CA drive strengths | Selection | Auto [0xFF] ◀ default 120.0 Ohm [0x78] 60.0 Ohm [0x3C] 40.0 Ohm [0x28] 30.0 Ohm [0x1E] ◀ performance NEUTRAL Memory signal-integrity (drive strength / ODT) — auto-trained for the soldered LPDDR5X. Manual changes are expert-only and risk training failure; leave Auto. |
| Processor DQ drive strengths | Selection | Auto [0xFF] ◀ default High Impedance [0x0] 240 ohm [0x2] 120 ohm [0x4] 80 ohm [0x6] 60 ohm [0xC] 48 ohm [0xE] 40 ohm [0x1C] 34.3 ohm [0x1E] ◀ performance NEUTRAL Memory signal-integrity (drive strength / ODT) — auto-trained for the soldered LPDDR5X. Manual changes are expert-only and risk training failure; leave Auto. |
| Processor ODT impedance | Selection | Auto [0xFF] ◀ default High Impedance [0x0] 480 ohm [0x1] 240 ohm [0x2] 160 ohm [0x3] 120 ohm [0x4] 96 ohm [0x5] 80 ohm [0x6] 68.6 ohm [0x7] 60 ohm [0xC] 53.3 ohm [0xD] 48 ohm [0xE] 43.6 ohm [0xF] 40 ohm [0x1C] 36.9 ohm [0x1D] 34.3 ohm [0x1E] 32 ohm [0x1F] 30 ohm [0x3C] 28.2 ohm [0x3D] 26.7 ohm [0x3E] 25.3 ohm [0x3F] ◀ performance NEUTRAL Memory signal-integrity (drive strength / ODT) — auto-trained for the soldered LPDDR5X. Manual changes are expert-only and risk training failure; leave Auto. |
| Dram DQ drive strengths | Selection | Auto [0xFF] ◀ default 48 ohm [0x2] 40 ohm [0x1] 34 ohm [0x0] ◀ performance NEUTRAL Memory signal-integrity (drive strength / ODT) — auto-trained for the soldered LPDDR5X. Manual changes are expert-only and risk training failure; leave Auto. |
| Dram ODT impedance RTT_NOM_WR | Selection | Auto [0xFF] ◀ default RTT_OFF [0x0] RZQ (240) [0x1] RZQ/2 (120) [0x2] RZQ/3 (80) [0x3] RZQ/4 (60) [0x4] RZQ/5 (48) [0x5] RZQ/6 (40) [0x6] RZQ/7 (34) [0x7] ◀ performance NEUTRAL Memory signal-integrity (drive strength / ODT) — auto-trained for the soldered LPDDR5X. Manual changes are expert-only and risk training failure; leave Auto. |
| Dram ODT impedance RTT_NOM_RD | Selection | Auto [0xFF] ◀ default RTT_OFF [0x0] RZQ (240) [0x1] RZQ/2 (120) [0x2] RZQ/3 (80) [0x3] RZQ/4 (60) [0x4] RZQ/5 (48) [0x5] RZQ/6 (40) [0x6] RZQ/7 (34) [0x7] ◀ performance NEUTRAL Memory signal-integrity (drive strength / ODT) — auto-trained for the soldered LPDDR5X. Manual changes are expert-only and risk training failure; leave Auto. |
| Dram ODT impedance RTT_WR | Selection | Auto [0xFF] ◀ default RTT_OFF [0x0] RZQ (240) [0x1] RZQ/2 (120) [0x2] RZQ/3 (80) [0x3] RZQ/4 (60) [0x4] RZQ/5 (48) [0x5] RZQ/6 (40) [0x6] RZQ/7 (34) [0x7] ◀ performance NEUTRAL Memory signal-integrity (drive strength / ODT) — auto-trained for the soldered LPDDR5X. Manual changes are expert-only and risk training failure; leave Auto. |
| Dram ODT impedance RTT_PARK | Selection | Auto [0xFF] ◀ default RTT_OFF [0x0] RZQ (240) [0x1] RZQ/2 (120) [0x2] RZQ/3 (80) [0x3] RZQ/4 (60) [0x4] RZQ/5 (48) [0x5] RZQ/6 (40) [0x6] RZQ/7 (34) [0x7] ◀ performance NEUTRAL Memory signal-integrity (drive strength / ODT) — auto-trained for the soldered LPDDR5X. Manual changes are expert-only and risk training failure; leave Auto. |
| Dram ODT impedance DQS_RTT_PARK | Selection | Auto [0xFF] ◀ default RTT_OFF [0x0] RZQ (240) [0x1] RZQ/2 (120) [0x2] RZQ/3 (80) [0x3] RZQ/4 (60) [0x4] RZQ/5 (48) [0x5] RZQ/6 (40) [0x6] RZQ/7 (34) [0x7] ◀ performance NEUTRAL Memory signal-integrity (drive strength / ODT) — auto-trained for the soldered LPDDR5X. Manual changes are expert-only and risk training failure; leave Auto. |

### DDR Controller Configuration   (Form 0x114)

Submenus: DDR Power Options

### DDR Power Options   (Form 0x11D)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| Power Down Enable | Selection | Disabled [0x0] Enabled [0x1] Auto [0xFF] ◀ default ◀ performance TUNE Disabled can trim memory latency slightly under load; Enabled (Auto) saves idle power. Default is fine for most. |

### Infinity Fabric Frequency and Dividers   (Form 0x111)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| Infinity Fabric Frequency and Dividers | Selection | Auto [0xFFFF] ◀ default 333 MHz [0x14D] 400 MHz [0x190] 533 MHz [0x215] 667 MHz [0x29B] 800 MHz [0x320] 933 MHz [0x3A5] 1050 MHz [0x41A] 1066 MHz [0x42A] 1067 MHz [0x42B] 1200 MHz [0x4B0] 1333 MHz [0x535] 1367 MHz [0x557] 1400 MHz [0x578] 1433 MHz [0x599] 1467 MHz [0x5BB] 1500 MHz [0x5DC] 1533 MHz [0x5FD] 1567 MHz [0x61F] 1600 MHz [0x640] 1633 MHz [0x661] 1667 MHz [0x683] 1700 MHz [0x6A4] 1733 MHz [0x6C5] 1767 MHz [0x6E7] 1800 MHz [0x708] 1833 MHz [0x729] 1867 MHz [0x74B] 1900 MHz [0x76C] 1933 MHz [0x78D] 1967 MHz [0x7AF] 2000 MHz [0x7D0] 2033 MHz [0x7F1] 2067 MHz [0x813] 2100 MHz [0x834] 2133 MHz [0x855] 2167 MHz [0x877] 2200 MHz [0x898] 2233 MHz [0x8B9] 2267 MHz [0x8DB] 2300 MHz [0x8FC] 2333 MHz [0x91D] 2367 MHz [0x93F] 2400 MHz [0x960] 2433 MHz [0x981] 2467 MHz [0x9A3] 2500 MHz [0x9C4] 2550 MHz [0x9F6] 2600 MHz [0xA28] 2650 MHz [0xA5A] 2700 MHz [0xA8C] 2750 MHz [0xABE] 2800 MHz [0xAF0] 2850 MHz [0xB22] 2900 MHz [0xB54] 2950 MHz [0xB86] 3000 MHz [0xBB8] ◀ performance TUNE Auto. On Strix Halo the fabric/UCLK ratio is pre-balanced to the soldered LPDDR5X; manual changes rarely help and can destabilize. |
| UCLK DIV1 MODE | Selection | Auto [0xFF] ◀ default UCLK=MEMCLK [0x1] UCLK=MEMCLK/2 [0x0] ◀ performance TUNE Auto (UCLK=MEMCLK, i.e. 1:1, where firmware allows it). 1:1 minimizes memory latency; leave the firmware default for soldered RAM. |

### Precision Boost Overdrive   (Form 0x104)

Submenus: GFX Curve Optimizer  ‣  Curve Optimizer  ‣  Curve Shaper

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| Precision Boost Overdrive | Selection | Auto [0xFF] ◀ default Disable [0x0] Enabled [0x1] Advanced [0x2] ◀ performance CHANGE Advanced (or Enabled). Raises PPT/TDC/EDC ceilings so boost sustains longer — the main free-performance lever. Pair with adequate cooling. |
| PBO Limits | Selection | Auto [0xFF] ◀ default Disable [0x0] Motherboard [0x1] Manual [0x2] ◀ performance CHANGE Motherboard (not Auto) to let the chip use the full board power budget, or Manual to set explicit PPT/TDC/EDC. Auto is conservative. |
| PPT Limit [mW] | Numeric | Min 0x0 · Max 0xFFFFFFFF · Default 0x0 ◀ default ◀ performance TUNE Raise toward the platform max (this APU sustains up to ~140 W on the GTR9 Pro) for more all-core boost, within thermal limits. |
| TDC Limit [mA] | Numeric | Min 0x0 · Max 0xFFFFFFFF · Default 0x0 ◀ default ◀ performance TUNE Increase alongside PPT for sustained-load current headroom; keep within VRM/thermal capability. |
| EDC Limit [mA] | Numeric | Min 0x0 · Max 0xFFFFFFFF · Default 0x0 ◀ default ◀ performance TUNE Increase for peak/burst current headroom; raising EDC mainly helps short bursty boost. |
| CCX (or second rail) TDC Limit [mA] | Numeric | Min 0x0 · Max 0xFFFFFFFF · Default 0x0 ◀ default ◀ performance TUNE Secondary-rail TDC — raise in step with the main TDC limit when pushing all-core. |
| CCX (or second rail) EDC Limit [mA] | Numeric | Min 0x0 · Max 0xFFFFFFFF · Default 0x0 ◀ default ◀ performance TUNE Secondary-rail EDC — raise in step with the main EDC limit for burst headroom. |
| Precision Boost Overdrive Scalar Ctrl | Selection | Auto [0xFF] ◀ default Manual [0x1] ◀ performance TUNE Manual to expose the Scalar. Higher scalar extends boost residency (more heat); start conservative. |
| Precision Boost Overdrive Scalar | Selection | 1X [0x64] ◀ default 2X [0xC8] 3X [0x12C] 4X [0x190] 5X [0x1F4] 6X [0x258] 7X [0x2BC] 8X [0x320] 9X [0x384] 10X [0x3E8] ◀ performance TUNE 1X-2X for daily; up to 10X sustains higher voltage/boost longer at the cost of heat and longevity. 2-4X is a sane gaming target. |
| CPU Boost Clock Override | Selection | Disabled [0x0] ◀ default Enabled (Positive) [0x1] Enabled (Negative) [0x2] ◀ performance TUNE Enabled (Positive) to allow up to +200 MHz above rated boost when thermals permit. |
| Max CPU Boost Clock Override(+) | Numeric | Min 0x19 · Max 0xC8 · Step 0x19 · Default 0x19 ◀ default ◀ performance TUNE Add up to +200 MHz; effective only with thermal/power headroom (PBO + cooling). |
| Max CPU Boost Clock Override(-) | Numeric | Min 0x19 · Max 0x3E8 · Step 0x19 · Default 0x19 ◀ default ◀ performance NEUTRAL Negative override lowers the boost ceiling — leave 0 unless capping for thermals/quiet. |
| GPU Boost Clock Override | Selection | Enabled [0x1] Disabled [0x0] ◀ default ◀ performance TUNE Enabled (Positive) to lift the iGPU boost ceiling for graphics-bound gains, thermals permitting. |
| Max GPU Boost Clock Override | Selection | 0 MHz [0x0] ◀ default 25 MHz [0x19] 50 MHz [0x32] 75 MHz [0x4B] 100 MHz [0x64] 125 MHz [0x7D] 150 MHz [0x96] 175 MHz [0xAF] 200 MHz [0xC8] ◀ performance TUNE Add a modest iGPU boost offset and test; the 8060S is bandwidth/thermal-limited, so gains taper. |
| Platform Thermal Throttle Ctrl | Selection | Manual [0x1] Auto [0x0] ◀ default ◀ performance TUNE Manual to raise the throttle limit for more sustained clocks (only with strong cooling). |
| Platform Thermal Throttle Limit | Numeric | Min 0x0 · Max 0xFF · Default 0x0 ◀ default ◀ performance TUNE Raising the skin/platform throttle point sustains boost longer but increases temperatures and fan noise. |

### GFX Curve Optimizer   (Form 0x11E)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| GFX Curve Optimizer | Selection | Disable [0x0] ◀ default GFX Curve Optimizer [0x1] ◀ performance TUNE Enable + Negative to undervolt the iGPU for more sustained graphics clock within the shared power budget. |
| GFX Curve Optimizer Sign | Selection | Positive [0x0] ◀ default Negative [0x1] ◀ performance TUNE Negative undervolts the iGPU V/F curve for cooler, higher-sustained clocks. |
| GFX Curve Optimizer Magnitude | Numeric | Min 0x0 · Max 0x32 · Default 0x0 ◀ default ◀ performance TUNE Start small (e.g. -10) and validate; iGPU stability is workload-sensitive. |

### Curve Optimizer   (Form 0x11F)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| Curve Optimizer | Selection | Disable [0x0] ◀ default All Cores [0x1] Per Core [0x2] ◀ performance CHANGE Enable, All-Core, Negative. Undervolting the V/F curve lets the chip hit higher effective clocks cooler — the highest-value PBO sub-setting. |
| All Core Curve Optimizer Sign | Selection | Positive [0x0] ◀ default Negative [0x1] ◀ performance CHANGE Negative. A negative offset undervolts for higher sustained boost and lower temps. |
| All Core Curve Optimizer Magnitude | Numeric | Min 0x0 · Max 0x1E · Default 0x0 ◀ default ◀ performance TUNE Start around -15 to -20 and stress-test; stable mobile Zen 5 silicon often reaches -25 to -30. Validate for stability. |

### Curve Shaper   (Form 0x126)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| Min Frequency — Low Temperature | Selection | Auto [0xFF] ◀ default Enable [0x1] Disable [0x0] ◀ performance TUNE Fan-curve point. For sustained performance, bias the curve toward more airflow (lower start temp / higher PWM) to gain thermal headroom; trade against noise. |
| Min Frequency — Low Temperature Sign | Selection | Positive [0x0] ◀ default Negative [0x1] ◀ performance TUNE Fan-curve point. For sustained performance, bias the curve toward more airflow (lower start temp / higher PWM) to gain thermal headroom; trade against noise. |
| Min Frequency — Low Temperature Magnitude | Numeric | Min 0x0 · Max 0x1E · Default 0x0 ◀ default ◀ performance TUNE Fan-curve point. For sustained performance, bias the curve toward more airflow (lower start temp / higher PWM) to gain thermal headroom; trade against noise. |
| Min Frequency — Med Temperature | Selection | Auto [0xFF] ◀ default Enable [0x1] Disable [0x0] ◀ performance TUNE iGPU DPM freq-vs-temp point. Raising the per-temperature floor/ceiling sustains higher graphics clocks within thermal limits; expert tuning, validate stability. |
| Min Frequency — Med Temperature Sign | Selection | Positive [0x0] ◀ default Negative [0x1] ◀ performance TUNE iGPU DPM freq-vs-temp point. Raising the per-temperature floor/ceiling sustains higher graphics clocks within thermal limits; expert tuning, validate stability. |
| Min Frequency — Med Temperature Magnitude | Numeric | Min 0x0 · Max 0x1E · Default 0x0 ◀ default ◀ performance TUNE iGPU DPM freq-vs-temp point. Raising the per-temperature floor/ceiling sustains higher graphics clocks within thermal limits; expert tuning, validate stability. |
| Min Frequency — High Temperature | Selection | Auto [0xFF] ◀ default Enable [0x1] Disable [0x0] ◀ performance TUNE Fan-curve point. For sustained performance, bias the curve toward more airflow (lower start temp / higher PWM) to gain thermal headroom; trade against noise. |
| Min Frequency — High Temperature Sign | Selection | Positive [0x0] ◀ default Negative [0x1] ◀ performance TUNE Fan-curve point. For sustained performance, bias the curve toward more airflow (lower start temp / higher PWM) to gain thermal headroom; trade against noise. |
| Min Frequency — High Temperature Magnitude | Numeric | Min 0x0 · Max 0x1E · Default 0x0 ◀ default ◀ performance TUNE Fan-curve point. For sustained performance, bias the curve toward more airflow (lower start temp / higher PWM) to gain thermal headroom; trade against noise. |
| Low Frequency — Low Temperature | Selection | Auto [0xFF] ◀ default Enable [0x1] Disable [0x0] ◀ performance TUNE Fan-curve point. For sustained performance, bias the curve toward more airflow (lower start temp / higher PWM) to gain thermal headroom; trade against noise. |
| Low Frequency — Low Temperature Sign | Selection | Positive [0x0] ◀ default Negative [0x1] ◀ performance TUNE Fan-curve point. For sustained performance, bias the curve toward more airflow (lower start temp / higher PWM) to gain thermal headroom; trade against noise. |
| Low Frequency — Low Temperature Magnitude | Numeric | Min 0x0 · Max 0x1E · Default 0x0 ◀ default ◀ performance TUNE Fan-curve point. For sustained performance, bias the curve toward more airflow (lower start temp / higher PWM) to gain thermal headroom; trade against noise. |
| Low Frequency — Med Temperature | Selection | Auto [0xFF] ◀ default Enable [0x1] Disable [0x0] ◀ performance TUNE iGPU DPM freq-vs-temp point. Raising the per-temperature floor/ceiling sustains higher graphics clocks within thermal limits; expert tuning, validate stability. |
| Low Frequency — Med Temperature Sign | Selection | Positive [0x0] ◀ default Negative [0x1] ◀ performance TUNE iGPU DPM freq-vs-temp point. Raising the per-temperature floor/ceiling sustains higher graphics clocks within thermal limits; expert tuning, validate stability. |
| Low Frequency — Med Temperature Magnitude | Numeric | Min 0x0 · Max 0x1E · Default 0x0 ◀ default ◀ performance TUNE iGPU DPM freq-vs-temp point. Raising the per-temperature floor/ceiling sustains higher graphics clocks within thermal limits; expert tuning, validate stability. |
| Low Frequency — High Temperature | Selection | Auto [0xFF] ◀ default Enable [0x1] Disable [0x0] ◀ performance TUNE Fan-curve point. For sustained performance, bias the curve toward more airflow (lower start temp / higher PWM) to gain thermal headroom; trade against noise. |
| Low Frequency — High Temperature Sign | Selection | Positive [0x0] ◀ default Negative [0x1] ◀ performance TUNE Fan-curve point. For sustained performance, bias the curve toward more airflow (lower start temp / higher PWM) to gain thermal headroom; trade against noise. |
| Low Frequency — High Temperature Magnitude | Numeric | Min 0x0 · Max 0x1E · Default 0x0 ◀ default ◀ performance TUNE Fan-curve point. For sustained performance, bias the curve toward more airflow (lower start temp / higher PWM) to gain thermal headroom; trade against noise. |
| Med Frequency — Low Temperature | Selection | Auto [0xFF] ◀ default Enable [0x1] Disable [0x0] ◀ performance TUNE Fan-curve point. For sustained performance, bias the curve toward more airflow (lower start temp / higher PWM) to gain thermal headroom; trade against noise. |
| Med Frequency — Low Temperature Sign | Selection | Positive [0x0] ◀ default Negative [0x1] ◀ performance TUNE Fan-curve point. For sustained performance, bias the curve toward more airflow (lower start temp / higher PWM) to gain thermal headroom; trade against noise. |
| Med Frequency — Low Temperature Magnitude | Numeric | Min 0x0 · Max 0x1E · Default 0x0 ◀ default ◀ performance TUNE Fan-curve point. For sustained performance, bias the curve toward more airflow (lower start temp / higher PWM) to gain thermal headroom; trade against noise. |
| Med Frequency — Med Temperature | Selection | Auto [0xFF] ◀ default Enable [0x1] Disable [0x0] ◀ performance TUNE iGPU DPM freq-vs-temp point. Raising the per-temperature floor/ceiling sustains higher graphics clocks within thermal limits; expert tuning, validate stability. |
| Med Frequency — Med Temperature Sign | Selection | Positive [0x0] ◀ default Negative [0x1] ◀ performance TUNE iGPU DPM freq-vs-temp point. Raising the per-temperature floor/ceiling sustains higher graphics clocks within thermal limits; expert tuning, validate stability. |
| Med Frequency — Med Temperature Magnitude | Numeric | Min 0x0 · Max 0x1E · Default 0x0 ◀ default ◀ performance TUNE iGPU DPM freq-vs-temp point. Raising the per-temperature floor/ceiling sustains higher graphics clocks within thermal limits; expert tuning, validate stability. |
| Med Frequency — High Temperature | Selection | Auto [0xFF] ◀ default Enable [0x1] Disable [0x0] ◀ performance TUNE Fan-curve point. For sustained performance, bias the curve toward more airflow (lower start temp / higher PWM) to gain thermal headroom; trade against noise. |
| Med Frequency — High Temperature Sign | Selection | Positive [0x0] ◀ default Negative [0x1] ◀ performance TUNE Fan-curve point. For sustained performance, bias the curve toward more airflow (lower start temp / higher PWM) to gain thermal headroom; trade against noise. |
| Med Frequency — High Temperature Magnitude | Numeric | Min 0x0 · Max 0x1E · Default 0x0 ◀ default ◀ performance TUNE Fan-curve point. For sustained performance, bias the curve toward more airflow (lower start temp / higher PWM) to gain thermal headroom; trade against noise. |
| High Frequency — Low Temperature | Selection | Auto [0xFF] ◀ default Enable [0x1] Disable [0x0] ◀ performance TUNE Fan-curve point. For sustained performance, bias the curve toward more airflow (lower start temp / higher PWM) to gain thermal headroom; trade against noise. |
| High Frequency — Low Temperature Sign | Selection | Positive [0x0] ◀ default Negative [0x1] ◀ performance TUNE Fan-curve point. For sustained performance, bias the curve toward more airflow (lower start temp / higher PWM) to gain thermal headroom; trade against noise. |
| High Frequency — Low Temperature Magnitude | Numeric | Min 0x0 · Max 0x1E · Default 0x0 ◀ default ◀ performance TUNE Fan-curve point. For sustained performance, bias the curve toward more airflow (lower start temp / higher PWM) to gain thermal headroom; trade against noise. |
| High Frequency — Med Temperature | Selection | Auto [0xFF] ◀ default Enable [0x1] Disable [0x0] ◀ performance TUNE iGPU DPM freq-vs-temp point. Raising the per-temperature floor/ceiling sustains higher graphics clocks within thermal limits; expert tuning, validate stability. |
| High Frequency — Med Temperature Sign | Selection | Positive [0x0] ◀ default Negative [0x1] ◀ performance TUNE iGPU DPM freq-vs-temp point. Raising the per-temperature floor/ceiling sustains higher graphics clocks within thermal limits; expert tuning, validate stability. |
| High Frequency — Med Temperature Magnitude | Numeric | Min 0x0 · Max 0x1E · Default 0x0 ◀ default ◀ performance TUNE iGPU DPM freq-vs-temp point. Raising the per-temperature floor/ceiling sustains higher graphics clocks within thermal limits; expert tuning, validate stability. |
| High Frequency — High Temperature | Selection | Auto [0xFF] ◀ default Enable [0x1] Disable [0x0] ◀ performance TUNE Fan-curve point. For sustained performance, bias the curve toward more airflow (lower start temp / higher PWM) to gain thermal headroom; trade against noise. |
| High Frequency — High Temperature Sign | Selection | Positive [0x0] ◀ default Negative [0x1] ◀ performance TUNE Fan-curve point. For sustained performance, bias the curve toward more airflow (lower start temp / higher PWM) to gain thermal headroom; trade against noise. |
| High Frequency — High Temperature Magnitude | Numeric | Min 0x0 · Max 0x1E · Default 0x0 ◀ default ◀ performance TUNE Fan-curve point. For sustained performance, bias the curve toward more airflow (lower start temp / higher PWM) to gain thermal headroom; trade against noise. |
| Max Frequency — Low Temperature | Selection | Auto [0xFF] ◀ default Enable [0x1] Disable [0x0] ◀ performance TUNE Fan-curve point. For sustained performance, bias the curve toward more airflow (lower start temp / higher PWM) to gain thermal headroom; trade against noise. |
| Max Frequency — Low Temperature Sign | Selection | Positive [0x0] ◀ default Negative [0x1] ◀ performance TUNE Fan-curve point. For sustained performance, bias the curve toward more airflow (lower start temp / higher PWM) to gain thermal headroom; trade against noise. |
| Max Frequency — Low Temperature Magnitude | Numeric | Min 0x0 · Max 0x1E · Default 0x0 ◀ default ◀ performance TUNE Fan-curve point. For sustained performance, bias the curve toward more airflow (lower start temp / higher PWM) to gain thermal headroom; trade against noise. |
| Max Frequency — Med Temperature | Selection | Auto [0xFF] ◀ default Enable [0x1] Disable [0x0] ◀ performance TUNE iGPU DPM freq-vs-temp point. Raising the per-temperature floor/ceiling sustains higher graphics clocks within thermal limits; expert tuning, validate stability. |
| Max Frequency — Med Temperature Sign | Selection | Positive [0x0] ◀ default Negative [0x1] ◀ performance TUNE iGPU DPM freq-vs-temp point. Raising the per-temperature floor/ceiling sustains higher graphics clocks within thermal limits; expert tuning, validate stability. |
| Max Frequency — Med Temperature Magnitude | Numeric | Min 0x0 · Max 0x1E · Default 0x0 ◀ default ◀ performance TUNE iGPU DPM freq-vs-temp point. Raising the per-temperature floor/ceiling sustains higher graphics clocks within thermal limits; expert tuning, validate stability. |
| Max Frequency — High Temperature | Selection | Auto [0xFF] ◀ default Enable [0x1] Disable [0x0] ◀ performance TUNE Fan-curve point. For sustained performance, bias the curve toward more airflow (lower start temp / higher PWM) to gain thermal headroom; trade against noise. |
| Max Frequency — High Temperature Sign | Selection | Positive [0x0] ◀ default Negative [0x1] ◀ performance TUNE Fan-curve point. For sustained performance, bias the curve toward more airflow (lower start temp / higher PWM) to gain thermal headroom; trade against noise. |
| Max Frequency — High Temperature Magnitude | Numeric | Min 0x0 · Max 0x1E · Default 0x0 ◀ default ◀ performance TUNE Fan-curve point. For sustained performance, bias the curve toward more airflow (lower start temp / higher PWM) to gain thermal headroom; trade against noise. |

### VDDP Voltage Control   (Form 0x105)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| VDDP Voltage Control | Selection | Auto [0xFF] ◀ default Manual [0x1] ◀ performance NEUTRAL Memory PHY rail — expert memory-OC only; leave Auto on soldered RAM. |
| VDDP Voltage Adjust | Numeric | Min 0x0 · Max 0x7D0 · Default 0x0 ◀ default ◀ performance NEUTRAL Expert memory-PHY tuning — leave Auto; no benefit on fixed soldered memory. |

### SoC/Uncore OC Mode   (Form 0x106)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| SoC/Uncore OC Mode | Selection | Auto [0xFF] ◀ default Enabled [0x1] Disabled [0x0] ◀ performance TUNE Auto. Forcing max SoC/uncore raises power and heat for little gain on this APU; leave default. |

### SoC Voltage   (Form 0x107)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| SoC Voltage | Numeric | Min 0x0 · Max 0xAF0 · Default 0x0 ◀ default ◀ performance NEUTRAL Expert SoC rail tuning — leave Auto; mis-set values risk instability. |

### LN2 Mode   (Form 0x108)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| LN2 Mode | Selection | Auto [0xFF] ◀ default Disabled [0x0] Enabled [0x1] ◀ performance NEUTRAL Extreme sub-zero overclocking aid — keep Disabled for normal use. |

### VDD Misc   (Form 0x109)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| VDD Misc Control | Selection | Auto [0x0] ◀ default Manual [0x1] ◀ performance NEUTRAL Auxiliary rail control — leave Auto. |
| VDD Misc Voltage | Numeric | Min 0x1F4 · Max 0x15E0 · Default 0x320 ◀ default ◀ performance NEUTRAL Auxiliary rail — leave Auto; not a performance lever. |

### LCLK Frequency Control   (Form 0x10A)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| LCLK Frequency Control | Selection | Auto [0x0] ◀ default Manual [0x1] ◀ performance TUNE Auto. LCLK (NBIO/PCIe fabric clock) — Manual max can lower PCIe/IO latency but raises power; default is balanced. |
| Minimun Frequency | Numeric | Min 0x96 · Max 0x9C4 · Default 0x96 ◀ default ◀ performance TUNE LCLK floor. Raising the minimum keeps the NBIO/PCIe fabric clock high for lower IO latency at the cost of idle power; default is balanced. |
| Maximum Frequency | Numeric | Min 0x96 · Max 0x9C4 · Default 0x9C4 ◀ default ◀ performance TUNE LCLK ceiling. Leaving at max lets the IO fabric reach full speed under load; default Auto already allows this. |

### Onboard Voltage Control   (Form 0x10B)

Submenus: VDDIO Voltage Control  ‣  VPP Voltage Control  ‣  VDD_MEM Voltage Control

### VDDIO Voltage Control   (Form 0x123)

Information fields: Current DIMM VDD Voltage, Current DIMM VDDQ Voltage, Current APU VDDIO Voltage

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| VDDIO Ctrl | Selection | Auto [0x0] ◀ default Manual [0x1] Separate [0x2] ◀ performance NEUTRAL Memory IO voltage — leave Auto on soldered RAM. |
| DIMM VDD Adjust | Numeric | Min 0x320 · Max 0x596 · Step 0xA · Default 0x44C ◀ default ◀ performance NEUTRAL Soldered memory — not adjustable in practice; leave Auto. |
| DIMM VDDQ Adjust | Numeric | Min 0x320 · Max 0x596 · Default 0x44C ◀ default ◀ performance NEUTRAL Soldered memory — leave Auto. |

### VPP Voltage Control   (Form 0x124)

Information fields: Current VPP Voltage

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| VPP Ctrl | Selection | Auto [0x0] ◀ default Manual [0x1] ◀ performance NEUTRAL Memory VPP rail — leave Auto. |
| VPP Adjust | Numeric | Min 0x5DC · Max 0x852 · Step 0xA · Default 0x708 ◀ default ◀ performance NEUTRAL Memory VPP rail — leave Auto. |

### VDD_MEM Voltage Control   (Form 0x125)

Information fields: Current VDD_MEM Voltage

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| VDD_MEM Control | Selection | Auto [0x0] ◀ default Manual [0x1] ◀ performance NEUTRAL Memory rail control — leave Auto on soldered RAM. |

# AMD PMF (Platform Management Framework)

AMD Platform Management Framework (PMF) — APMF/APTS/ATST function-table configuration that drives dynamic thermal and power policy (Smart PC / sensor-driven power steering), reached via Advanced ‣ AMD PMF Feature.

Form-set GUID: 6A1C2AEA-F71E-43D7-B17F-F551B89257DB     ·     Forms: 28     ·     Configurable settings: 139

### AMD PMF Feature   (Form 0x64)

Submenus: APMF Multiple Binaries Settings  ‣  APTS Control Method Settings  ‣  ATST Function  ‣  APMF Function 1 Settings  ‣  APMF Function 4 Settings  ‣  APMF Function 7 Settings  ‣  APMF Function 9 Settings  ‣  APMF Function 11 Settings  ‣  APMF Function 12 Settings  ‣  APMF Function 14 Settings  ‣  APMF Function 16 Settings

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| PMF Device Support | Selection | Disabled [0x0] ◀ default Enabled [0x1] ◀ performance NEUTRAL PMF policy/firmware-binary plumbing — the OEM ships validated policy; no user performance setting here. Leave default. |

### APMF Multiple Binaries Settings   (Form 0x6D)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| APMF Multiple Binaries Support | Selection | Disabled [0x0] ◀ default Enabled [0x1] ◀ performance NEUTRAL PMF policy/firmware-binary plumbing — the OEM ships validated policy; no user performance setting here. Leave default. |
| PMF Policy Binaries Index | Selection | PMF Binary 0 [0x0] ◀ default PMF Binary 1 [0x1] PMF Binary 2 [0x2] PMF Binary 3 [0x3] PMF Binary 4 [0x4] PMF Binary 5 [0x5] PMF Binary 6 [0x6] PMF Binary 7 [0x7] PMF Binary 8 [0x8] PMF Binary 9 [0x9] ◀ performance NEUTRAL PMF policy/firmware-binary plumbing — the OEM ships validated policy; no user performance setting here. Leave default. |
| PMF Binary 0 Size | Selection | 0 KB [0x0] ◀ default 30 KB [0x1] 60 KB [0x2] 90 KB [0x3] 120 KB [0x4] 150 KB [0x5] 180 KB [0x6] 210 KB [0x7] 240 KB [0x8] 270 KB [0x9] 300 KB [0xA] ◀ performance NEUTRAL PMF policy/firmware-binary plumbing — the OEM ships validated policy; no user performance setting here. Leave default. |
| PMF Binary 1 Size | Selection | 0 KB [0x0] ◀ default 30 KB [0x1] 60 KB [0x2] 90 KB [0x3] 120 KB [0x4] 150 KB [0x5] 180 KB [0x6] 210 KB [0x7] 240 KB [0x8] 270 KB [0x9] 300 KB [0xA] ◀ performance NEUTRAL PMF policy/firmware-binary plumbing — the OEM ships validated policy; no user performance setting here. Leave default. |
| PMF Binary 2 Size | Selection | 0 KB [0x0] ◀ default 30 KB [0x1] 60 KB [0x2] 90 KB [0x3] 120 KB [0x4] 150 KB [0x5] 180 KB [0x6] 210 KB [0x7] 240 KB [0x8] 270 KB [0x9] 300 KB [0xA] ◀ performance NEUTRAL PMF policy/firmware-binary plumbing — the OEM ships validated policy; no user performance setting here. Leave default. |
| PMF Binary 3 Size | Selection | 0 KB [0x0] ◀ default 30 KB [0x1] 60 KB [0x2] 90 KB [0x3] 120 KB [0x4] 150 KB [0x5] 180 KB [0x6] 210 KB [0x7] 240 KB [0x8] 270 KB [0x9] 300 KB [0xA] ◀ performance NEUTRAL PMF policy/firmware-binary plumbing — the OEM ships validated policy; no user performance setting here. Leave default. |
| PMF Binary 4 Size | Selection | 0 KB [0x0] ◀ default 30 KB [0x1] 60 KB [0x2] 90 KB [0x3] 120 KB [0x4] 150 KB [0x5] 180 KB [0x6] 210 KB [0x7] 240 KB [0x8] 270 KB [0x9] 300 KB [0xA] ◀ performance NEUTRAL PMF policy/firmware-binary plumbing — the OEM ships validated policy; no user performance setting here. Leave default. |
| PMF Binary 5 Size | Selection | 0 KB [0x0] ◀ default 30 KB [0x1] 60 KB [0x2] 90 KB [0x3] 120 KB [0x4] 150 KB [0x5] 180 KB [0x6] 210 KB [0x7] 240 KB [0x8] 270 KB [0x9] 300 KB [0xA] ◀ performance NEUTRAL PMF policy/firmware-binary plumbing — the OEM ships validated policy; no user performance setting here. Leave default. |
| PMF Binary 6 Size | Selection | 0 KB [0x0] ◀ default 30 KB [0x1] 60 KB [0x2] 90 KB [0x3] 120 KB [0x4] 150 KB [0x5] 180 KB [0x6] 210 KB [0x7] 240 KB [0x8] 270 KB [0x9] 300 KB [0xA] ◀ performance NEUTRAL PMF policy/firmware-binary plumbing — the OEM ships validated policy; no user performance setting here. Leave default. |
| PMF Binary 7 Size | Selection | 0 KB [0x0] ◀ default 30 KB [0x1] 60 KB [0x2] 90 KB [0x3] 120 KB [0x4] 150 KB [0x5] 180 KB [0x6] 210 KB [0x7] 240 KB [0x8] 270 KB [0x9] 300 KB [0xA] ◀ performance NEUTRAL PMF policy/firmware-binary plumbing — the OEM ships validated policy; no user performance setting here. Leave default. |
| PMF Binary 8 Size | Selection | 0 KB [0x0] ◀ default 30 KB [0x1] 60 KB [0x2] 90 KB [0x3] 120 KB [0x4] 150 KB [0x5] 180 KB [0x6] 210 KB [0x7] 240 KB [0x8] 270 KB [0x9] 300 KB [0xA] ◀ performance NEUTRAL PMF policy/firmware-binary plumbing — the OEM ships validated policy; no user performance setting here. Leave default. |
| PMF Binary 9 Size | Selection | 0 KB [0x0] ◀ default 30 KB [0x1] 60 KB [0x2] 90 KB [0x3] 120 KB [0x4] 150 KB [0x5] 180 KB [0x6] 210 KB [0x7] 240 KB [0x8] 270 KB [0x9] 300 KB [0xA] ◀ performance NEUTRAL PMF policy/firmware-binary plumbing — the OEM ships validated policy; no user performance setting here. Leave default. |

### APTS Control Method Settings   (Form 0x6E)

Submenus: APTS State Index 0  ‣  APTS State Index 1  ‣  APTS State Index 2  ‣  APTS State Index 3  ‣  APTS State Index 4  ‣  APTS State Index 5  ‣  APTS State Index 6  ‣  APTS State Index 7  ‣  APTS State Index 8  ‣  APTS State Index 9  ‣  APTS State Index 10  ‣  APTS State Index 11  ‣  APTS State Index 12  ‣  APTS State Index 13  ‣  APTS State Index 14  ‣  APTS State Index 15

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| Performance and Thermal State Control Method | Selection | Disabled [0x0] ◀ default Enabled [0x1] ◀ performance NEUTRAL PMF policy/firmware-binary plumbing — the OEM ships validated policy; no user performance setting here. Leave default. |

### ATST Function   (Form 0x7F)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| ATST Function 3 | Selection | Disabled [0x0] ◀ default Enabled [0x1] ◀ performance NEUTRAL External/discrete-GPU or display-mux plumbing — no impact on the iGPU-only default config; leave default. |

### APMF Function 1 Settings   (Form 0x65)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| Heartbeat Time Interval | Numeric | Min 0x0 · Max 0xFFFFFFFF · Step 0x1 · Default 0x3C ◀ default ◀ performance NEUTRAL Watchdog timer — reliability feature, no performance impact. Leave default. |
| PMF PPT Time Constant Control | Selection | Disabled [0x0] ◀ default Enabled [0x1] ◀ performance NEUTRAL PMF policy/firmware-binary plumbing — the OEM ships validated policy; no user performance setting here. Leave default. |
| PMF PPT Time Constant | Numeric | Min 0x64 · Max 0x7530 · Step 0x1 · Default 0x64 ◀ default ◀ performance NEUTRAL PMF policy/firmware-binary plumbing — the OEM ships validated policy; no user performance setting here. Leave default. |

### APMF Function 4 Settings   (Form 0x66)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| AMPF Function 4 | Selection | Disabled [0x0] ◀ default Enabled [0x1] ◀ performance NEUTRAL PMF policy/firmware-binary plumbing — the OEM ships validated policy; no user performance setting here. Leave default. |

### APMF Function 7 Settings   (Form 0x67)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| AMPF Function 7 | Selection | Disabled [0x0] ◀ default Enabled [0x1] ◀ performance NEUTRAL PMF policy/firmware-binary plumbing — the OEM ships validated policy; no user performance setting here. Leave default. |

### APMF Function 9 Settings   (Form 0x68)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| AMPF Function 9 | Selection | Disabled [0x0] ◀ default Enabled [0x1] ◀ performance NEUTRAL PMF policy/firmware-binary plumbing — the OEM ships validated policy; no user performance setting here. Leave default. |
| APTS State Index of AC Best Performance | Numeric | Min 0x0 · Max 0xF · Step 0x1 · Default 0x0 ◀ default ◀ performance NEUTRAL PMF policy/firmware-binary plumbing — the OEM ships validated policy; no user performance setting here. Leave default. |
| APTS State Index of AC Best Balanced | Numeric | Min 0x0 · Max 0xF · Step 0x1 · Default 0x1 ◀ default ◀ performance NEUTRAL PMF policy/firmware-binary plumbing — the OEM ships validated policy; no user performance setting here. Leave default. |
| APTS State Index of AC Best Power Efficiency | Numeric | Min 0x0 · Max 0xF · Step 0x1 · Default 0x2 ◀ default ◀ performance NEUTRAL PMF policy/firmware-binary plumbing — the OEM ships validated policy; no user performance setting here. Leave default. |
| APTS State Index of AC Energy Saver | Numeric | Min 0x0 · Max 0xF · Step 0x1 · Default 0x3 ◀ default ◀ performance NEUTRAL PMF policy/firmware-binary plumbing — the OEM ships validated policy; no user performance setting here. Leave default. |
| APTS State Index of DC Best Performance | Numeric | Min 0x0 · Max 0xF · Step 0x1 · Default 0x4 ◀ default ◀ performance NEUTRAL PMF policy/firmware-binary plumbing — the OEM ships validated policy; no user performance setting here. Leave default. |
| APTS State Index of DC Balanced | Numeric | Min 0x0 · Max 0xF · Step 0x1 · Default 0x5 ◀ default ◀ performance NEUTRAL PMF policy/firmware-binary plumbing — the OEM ships validated policy; no user performance setting here. Leave default. |
| APTS State Index of DC Best Power Efficiency | Numeric | Min 0x0 · Max 0xF · Step 0x1 · Default 0x6 ◀ default ◀ performance NEUTRAL PMF policy/firmware-binary plumbing — the OEM ships validated policy; no user performance setting here. Leave default. |
| APTS State Index of DC Battery Saver | Numeric | Min 0x0 · Max 0xF · Step 0x1 · Default 0x7 ◀ default ◀ performance TUNE AC/DC power-profile cap. On this mains-powered mini-PC the AC profile applies — set the AC limits to the highest/Maximum-Performance tier; DC limits are irrelevant. |

### APMF Function 11 Settings   (Form 0x69)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| AMPF Function 11 | Selection | Disabled [0x0] ◀ default Enabled [0x1] ◀ performance NEUTRAL PMF policy/firmware-binary plumbing — the OEM ships validated policy; no user performance setting here. Leave default. |
| Flags | Numeric | Min 0x0 · Max 0xFFFF · Step 0x1 · Default 0x0 ◀ default ◀ performance NEUTRAL Informational / non-configurable field — no performance action. |
| APTS State Index of ASM Performance | Numeric | Min 0x0 · Max 0xF · Step 0x1 · Default 0x0 ◀ default ◀ performance NEUTRAL PMF policy/firmware-binary plumbing — the OEM ships validated policy; no user performance setting here. Leave default. |
| APTS State Index of ASM Balanced | Numeric | Min 0x0 · Max 0xF · Step 0x1 · Default 0x1 ◀ default ◀ performance NEUTRAL PMF policy/firmware-binary plumbing — the OEM ships validated policy; no user performance setting here. Leave default. |
| APTS State Index of ASM Virtually Silent | Numeric | Min 0x0 · Max 0xF · Step 0x1 · Default 0x2 ◀ default ◀ performance NEUTRAL PMF policy/firmware-binary plumbing — the OEM ships validated policy; no user performance setting here. Leave default. |
| APTS State Index of ASM Non-Interactive | Numeric | Min 0x0 · Max 0xF · Step 0x1 · Default 0x3 ◀ default ◀ performance NEUTRAL PMF policy/firmware-binary plumbing — the OEM ships validated policy; no user performance setting here. Leave default. |

### APMF Function 12 Settings   (Form 0x6A)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| AMPF Function 12 | Selection | Disabled [0x0] ◀ default Enabled [0x1] ◀ performance NEUTRAL PMF policy/firmware-binary plumbing — the OEM ships validated policy; no user performance setting here. Leave default. |
| Flags | Numeric | Min 0x0 · Max 0xFFFF · Step 0x1 · Default 0x0 ◀ default ◀ performance NEUTRAL Informational / non-configurable field — no performance action. |
| APTS State Index of ASM Performance | Numeric | Min 0x0 · Max 0xF · Step 0x1 · Default 0x4 ◀ default ◀ performance NEUTRAL PMF policy/firmware-binary plumbing — the OEM ships validated policy; no user performance setting here. Leave default. |
| APTS State Index of ASM Balanced | Numeric | Min 0x0 · Max 0xF · Step 0x1 · Default 0x5 ◀ default ◀ performance NEUTRAL PMF policy/firmware-binary plumbing — the OEM ships validated policy; no user performance setting here. Leave default. |
| APTS State Index of ASM Virtually Silent | Numeric | Min 0x0 · Max 0xF · Step 0x1 · Default 0x6 ◀ default ◀ performance NEUTRAL PMF policy/firmware-binary plumbing — the OEM ships validated policy; no user performance setting here. Leave default. |
| APTS State Index of ASM Non-Interactive | Numeric | Min 0x0 · Max 0xF · Step 0x1 · Default 0x7 ◀ default ◀ performance NEUTRAL PMF policy/firmware-binary plumbing — the OEM ships validated policy; no user performance setting here. Leave default. |

### APMF Function 14 Settings   (Form 0x6B)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| AMPF Function 14 | Selection | Disabled [0x0] ◀ default Enabled [0x1] ◀ performance NEUTRAL PMF policy/firmware-binary plumbing — the OEM ships validated policy; no user performance setting here. Leave default. |

### APMF Function 16 Settings   (Form 0x6C)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| AMPF Function 16 | Selection | Disabled [0x0] ◀ default Enabled [0x1] ◀ performance NEUTRAL PMF policy/firmware-binary plumbing — the OEM ships validated policy; no user performance setting here. Leave default. |

### APTS State Index 0   (Form 0x6F)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| Fan Table Index | Numeric | Min 0x0 · Max 0xFFFFFFFF · Step 0x1 · Default 0x0 ◀ default ◀ performance TUNE Fan-curve point. For sustained performance, bias the curve toward more airflow (lower start temp / higher PWM) to gain thermal headroom; trade against noise. |
| PPT Limit for PMF power throttler | Numeric | Min 0x0 · Max 0xFFFFFFFF · Step 0x1 · Default 0x0 ◀ default ◀ performance NEUTRAL PMF policy/firmware-binary plumbing — the OEM ships validated policy; no user performance setting here. Leave default. |
| PPT Limit for PMF APU Only power throttler | Numeric | Min 0x0 · Max 0xFFFFFFFF · Step 0x1 · Default 0x0 ◀ default ◀ performance NEUTRAL PMF policy/firmware-binary plumbing — the OEM ships validated policy; no user performance setting here. Leave default. |
| STT Min Power Limit in milliwatts | Numeric | Min 0x0 · Max 0xFFFFFFFF · Step 0x1 · Default 0x0 ◀ default ◀ performance TUNE STT thermal/power limit. Raising the skin-temp or min-power limit lets the APU sustain more power (more performance, hotter chassis); default protects surface temperature and safety. |
| STT Skin Temp Limit APU in Celsius | Numeric | Min 0x0 · Max 0xFF · Step 0x1 · Default 0x0 ◀ default ◀ performance TUNE STT thermal/power limit. Raising the skin-temp or min-power limit lets the APU sustain more power (more performance, hotter chassis); default protects surface temperature and safety. |
| STT Skin Temp Limit HS2 in Celsius | Numeric | Min 0x0 · Max 0xFF · Step 0x1 · Default 0x0 ◀ default ◀ performance TUNE STT thermal/power limit. Raising the skin-temp or min-power limit lets the APU sustain more power (more performance, hotter chassis); default protects surface temperature and safety. |

### APTS State Index 1   (Form 0x70)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| Fan Table Index | Numeric | Min 0x0 · Max 0xFFFFFFFF · Step 0x1 · Default 0x0 ◀ default ◀ performance TUNE Fan-curve point. For sustained performance, bias the curve toward more airflow (lower start temp / higher PWM) to gain thermal headroom; trade against noise. |
| PPT Limit for PMF power throttler | Numeric | Min 0x0 · Max 0xFFFFFFFF · Step 0x1 · Default 0x0 ◀ default ◀ performance NEUTRAL PMF policy/firmware-binary plumbing — the OEM ships validated policy; no user performance setting here. Leave default. |
| PPT Limit for PMF APU Only power throttler | Numeric | Min 0x0 · Max 0xFFFFFFFF · Step 0x1 · Default 0x0 ◀ default ◀ performance NEUTRAL PMF policy/firmware-binary plumbing — the OEM ships validated policy; no user performance setting here. Leave default. |
| STT Min Power Limit in milliwatts | Numeric | Min 0x0 · Max 0xFFFFFFFF · Step 0x1 · Default 0x0 ◀ default ◀ performance TUNE STT thermal/power limit. Raising the skin-temp or min-power limit lets the APU sustain more power (more performance, hotter chassis); default protects surface temperature and safety. |
| STT Skin Temp Limit APU in Celsius | Numeric | Min 0x0 · Max 0xFF · Step 0x1 · Default 0x0 ◀ default ◀ performance TUNE STT thermal/power limit. Raising the skin-temp or min-power limit lets the APU sustain more power (more performance, hotter chassis); default protects surface temperature and safety. |
| STT Skin Temp Limit HS2 in Celsius | Numeric | Min 0x0 · Max 0xFF · Step 0x1 · Default 0x0 ◀ default ◀ performance TUNE STT thermal/power limit. Raising the skin-temp or min-power limit lets the APU sustain more power (more performance, hotter chassis); default protects surface temperature and safety. |

### APTS State Index 2   (Form 0x71)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| Fan Table Index | Numeric | Min 0x0 · Max 0xFFFFFFFF · Step 0x1 · Default 0x0 ◀ default ◀ performance TUNE Fan-curve point. For sustained performance, bias the curve toward more airflow (lower start temp / higher PWM) to gain thermal headroom; trade against noise. |
| PPT Limit for PMF power throttler | Numeric | Min 0x0 · Max 0xFFFFFFFF · Step 0x1 · Default 0x0 ◀ default ◀ performance NEUTRAL PMF policy/firmware-binary plumbing — the OEM ships validated policy; no user performance setting here. Leave default. |
| PPT Limit for PMF APU Only power throttler | Numeric | Min 0x0 · Max 0xFFFFFFFF · Step 0x1 · Default 0x0 ◀ default ◀ performance NEUTRAL PMF policy/firmware-binary plumbing — the OEM ships validated policy; no user performance setting here. Leave default. |
| STT Min Power Limit in milliwatts | Numeric | Min 0x0 · Max 0xFFFFFFFF · Step 0x1 · Default 0x0 ◀ default ◀ performance TUNE STT thermal/power limit. Raising the skin-temp or min-power limit lets the APU sustain more power (more performance, hotter chassis); default protects surface temperature and safety. |
| STT Skin Temp Limit APU in Celsius | Numeric | Min 0x0 · Max 0xFF · Step 0x1 · Default 0x0 ◀ default ◀ performance TUNE STT thermal/power limit. Raising the skin-temp or min-power limit lets the APU sustain more power (more performance, hotter chassis); default protects surface temperature and safety. |
| STT Skin Temp Limit HS2 in Celsius | Numeric | Min 0x0 · Max 0xFF · Step 0x1 · Default 0x0 ◀ default ◀ performance TUNE STT thermal/power limit. Raising the skin-temp or min-power limit lets the APU sustain more power (more performance, hotter chassis); default protects surface temperature and safety. |

### APTS State Index 3   (Form 0x72)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| Fan Table Index | Numeric | Min 0x0 · Max 0xFFFFFFFF · Step 0x1 · Default 0x0 ◀ default ◀ performance TUNE Fan-curve point. For sustained performance, bias the curve toward more airflow (lower start temp / higher PWM) to gain thermal headroom; trade against noise. |
| PPT Limit for PMF power throttler | Numeric | Min 0x0 · Max 0xFFFFFFFF · Step 0x1 · Default 0x0 ◀ default ◀ performance NEUTRAL PMF policy/firmware-binary plumbing — the OEM ships validated policy; no user performance setting here. Leave default. |
| PPT Limit for PMF APU Only power throttler | Numeric | Min 0x0 · Max 0xFFFFFFFF · Step 0x1 · Default 0x0 ◀ default ◀ performance NEUTRAL PMF policy/firmware-binary plumbing — the OEM ships validated policy; no user performance setting here. Leave default. |
| STT Min Power Limit in milliwatts | Numeric | Min 0x0 · Max 0xFFFFFFFF · Step 0x1 · Default 0x0 ◀ default ◀ performance TUNE STT thermal/power limit. Raising the skin-temp or min-power limit lets the APU sustain more power (more performance, hotter chassis); default protects surface temperature and safety. |
| STT Skin Temp Limit APU in Celsius | Numeric | Min 0x0 · Max 0xFF · Step 0x1 · Default 0x0 ◀ default ◀ performance TUNE STT thermal/power limit. Raising the skin-temp or min-power limit lets the APU sustain more power (more performance, hotter chassis); default protects surface temperature and safety. |
| STT Skin Temp Limit HS2 in Celsius | Numeric | Min 0x0 · Max 0xFF · Step 0x1 · Default 0x0 ◀ default ◀ performance TUNE STT thermal/power limit. Raising the skin-temp or min-power limit lets the APU sustain more power (more performance, hotter chassis); default protects surface temperature and safety. |

### APTS State Index 4   (Form 0x73)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| Fan Table Index | Numeric | Min 0x0 · Max 0xFFFFFFFF · Step 0x1 · Default 0x0 ◀ default ◀ performance TUNE Fan-curve point. For sustained performance, bias the curve toward more airflow (lower start temp / higher PWM) to gain thermal headroom; trade against noise. |
| PPT Limit for PMF power throttler | Numeric | Min 0x0 · Max 0xFFFFFFFF · Step 0x1 · Default 0x0 ◀ default ◀ performance NEUTRAL PMF policy/firmware-binary plumbing — the OEM ships validated policy; no user performance setting here. Leave default. |
| PPT Limit for PMF APU Only power throttler | Numeric | Min 0x0 · Max 0xFFFFFFFF · Step 0x1 · Default 0x0 ◀ default ◀ performance NEUTRAL PMF policy/firmware-binary plumbing — the OEM ships validated policy; no user performance setting here. Leave default. |
| STT Min Power Limit in milliwatts | Numeric | Min 0x0 · Max 0xFFFFFFFF · Step 0x1 · Default 0x0 ◀ default ◀ performance TUNE STT thermal/power limit. Raising the skin-temp or min-power limit lets the APU sustain more power (more performance, hotter chassis); default protects surface temperature and safety. |
| STT Skin Temp Limit APU in Celsius | Numeric | Min 0x0 · Max 0xFF · Step 0x1 · Default 0x0 ◀ default ◀ performance TUNE STT thermal/power limit. Raising the skin-temp or min-power limit lets the APU sustain more power (more performance, hotter chassis); default protects surface temperature and safety. |
| STT Skin Temp Limit HS2 in Celsius | Numeric | Min 0x0 · Max 0xFF · Step 0x1 · Default 0x0 ◀ default ◀ performance TUNE STT thermal/power limit. Raising the skin-temp or min-power limit lets the APU sustain more power (more performance, hotter chassis); default protects surface temperature and safety. |

### APTS State Index 5   (Form 0x74)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| Fan Table Index | Numeric | Min 0x0 · Max 0xFFFFFFFF · Step 0x1 · Default 0x0 ◀ default ◀ performance TUNE Fan-curve point. For sustained performance, bias the curve toward more airflow (lower start temp / higher PWM) to gain thermal headroom; trade against noise. |
| PPT Limit for PMF power throttler | Numeric | Min 0x0 · Max 0xFFFFFFFF · Step 0x1 · Default 0x0 ◀ default ◀ performance NEUTRAL PMF policy/firmware-binary plumbing — the OEM ships validated policy; no user performance setting here. Leave default. |
| PPT Limit for PMF APU Only power throttler | Numeric | Min 0x0 · Max 0xFFFFFFFF · Step 0x1 · Default 0x0 ◀ default ◀ performance NEUTRAL PMF policy/firmware-binary plumbing — the OEM ships validated policy; no user performance setting here. Leave default. |
| STT Min Power Limit in milliwatts | Numeric | Min 0x0 · Max 0xFFFFFFFF · Step 0x1 · Default 0x0 ◀ default ◀ performance TUNE STT thermal/power limit. Raising the skin-temp or min-power limit lets the APU sustain more power (more performance, hotter chassis); default protects surface temperature and safety. |
| STT Skin Temp Limit APU in Celsius | Numeric | Min 0x0 · Max 0xFF · Step 0x1 · Default 0x0 ◀ default ◀ performance TUNE STT thermal/power limit. Raising the skin-temp or min-power limit lets the APU sustain more power (more performance, hotter chassis); default protects surface temperature and safety. |
| STT Skin Temp Limit HS2 in Celsius | Numeric | Min 0x0 · Max 0xFF · Step 0x1 · Default 0x0 ◀ default ◀ performance TUNE STT thermal/power limit. Raising the skin-temp or min-power limit lets the APU sustain more power (more performance, hotter chassis); default protects surface temperature and safety. |

### APTS State Index 6   (Form 0x75)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| Fan Table Index | Numeric | Min 0x0 · Max 0xFFFFFFFF · Step 0x1 · Default 0x0 ◀ default ◀ performance TUNE Fan-curve point. For sustained performance, bias the curve toward more airflow (lower start temp / higher PWM) to gain thermal headroom; trade against noise. |
| PPT Limit for PMF power throttler | Numeric | Min 0x0 · Max 0xFFFFFFFF · Step 0x1 · Default 0x0 ◀ default ◀ performance NEUTRAL PMF policy/firmware-binary plumbing — the OEM ships validated policy; no user performance setting here. Leave default. |
| PPT Limit for PMF APU Only power throttler | Numeric | Min 0x0 · Max 0xFFFFFFFF · Step 0x1 · Default 0x0 ◀ default ◀ performance NEUTRAL PMF policy/firmware-binary plumbing — the OEM ships validated policy; no user performance setting here. Leave default. |
| STT Min Power Limit in milliwatts | Numeric | Min 0x0 · Max 0xFFFFFFFF · Step 0x1 · Default 0x0 ◀ default ◀ performance TUNE STT thermal/power limit. Raising the skin-temp or min-power limit lets the APU sustain more power (more performance, hotter chassis); default protects surface temperature and safety. |
| STT Skin Temp Limit APU in Celsius | Numeric | Min 0x0 · Max 0xFF · Step 0x1 · Default 0x0 ◀ default ◀ performance TUNE STT thermal/power limit. Raising the skin-temp or min-power limit lets the APU sustain more power (more performance, hotter chassis); default protects surface temperature and safety. |
| STT Skin Temp Limit HS2 in Celsius | Numeric | Min 0x0 · Max 0xFF · Step 0x1 · Default 0x0 ◀ default ◀ performance TUNE STT thermal/power limit. Raising the skin-temp or min-power limit lets the APU sustain more power (more performance, hotter chassis); default protects surface temperature and safety. |

### APTS State Index 7   (Form 0x76)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| Fan Table Index | Numeric | Min 0x0 · Max 0xFFFFFFFF · Step 0x1 · Default 0x0 ◀ default ◀ performance TUNE Fan-curve point. For sustained performance, bias the curve toward more airflow (lower start temp / higher PWM) to gain thermal headroom; trade against noise. |
| PPT Limit for PMF power throttler | Numeric | Min 0x0 · Max 0xFFFFFFFF · Step 0x1 · Default 0x0 ◀ default ◀ performance NEUTRAL PMF policy/firmware-binary plumbing — the OEM ships validated policy; no user performance setting here. Leave default. |
| PPT Limit for PMF APU Only power throttler | Numeric | Min 0x0 · Max 0xFFFFFFFF · Step 0x1 · Default 0x0 ◀ default ◀ performance NEUTRAL PMF policy/firmware-binary plumbing — the OEM ships validated policy; no user performance setting here. Leave default. |
| STT Min Power Limit in milliwatts | Numeric | Min 0x0 · Max 0xFFFFFFFF · Step 0x1 · Default 0x0 ◀ default ◀ performance TUNE STT thermal/power limit. Raising the skin-temp or min-power limit lets the APU sustain more power (more performance, hotter chassis); default protects surface temperature and safety. |
| STT Skin Temp Limit APU in Celsius | Numeric | Min 0x0 · Max 0xFF · Step 0x1 · Default 0x0 ◀ default ◀ performance TUNE STT thermal/power limit. Raising the skin-temp or min-power limit lets the APU sustain more power (more performance, hotter chassis); default protects surface temperature and safety. |
| STT Skin Temp Limit HS2 in Celsius | Numeric | Min 0x0 · Max 0xFF · Step 0x1 · Default 0x0 ◀ default ◀ performance TUNE STT thermal/power limit. Raising the skin-temp or min-power limit lets the APU sustain more power (more performance, hotter chassis); default protects surface temperature and safety. |

### APTS State Index 8   (Form 0x77)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| Fan Table Index | Numeric | Min 0x0 · Max 0xFFFFFFFF · Step 0x1 · Default 0x0 ◀ default ◀ performance TUNE Fan-curve point. For sustained performance, bias the curve toward more airflow (lower start temp / higher PWM) to gain thermal headroom; trade against noise. |
| PPT Limit for PMF power throttler | Numeric | Min 0x0 · Max 0xFFFFFFFF · Step 0x1 · Default 0x0 ◀ default ◀ performance NEUTRAL PMF policy/firmware-binary plumbing — the OEM ships validated policy; no user performance setting here. Leave default. |
| PPT Limit for PMF APU Only power throttler | Numeric | Min 0x0 · Max 0xFFFFFFFF · Step 0x1 · Default 0x0 ◀ default ◀ performance NEUTRAL PMF policy/firmware-binary plumbing — the OEM ships validated policy; no user performance setting here. Leave default. |
| STT Min Power Limit in milliwatts | Numeric | Min 0x0 · Max 0xFFFFFFFF · Step 0x1 · Default 0x0 ◀ default ◀ performance TUNE STT thermal/power limit. Raising the skin-temp or min-power limit lets the APU sustain more power (more performance, hotter chassis); default protects surface temperature and safety. |
| STT Skin Temp Limit APU in Celsius | Numeric | Min 0x0 · Max 0xFF · Step 0x1 · Default 0x0 ◀ default ◀ performance TUNE STT thermal/power limit. Raising the skin-temp or min-power limit lets the APU sustain more power (more performance, hotter chassis); default protects surface temperature and safety. |
| STT Skin Temp Limit HS2 in Celsius | Numeric | Min 0x0 · Max 0xFF · Step 0x1 · Default 0x0 ◀ default ◀ performance TUNE STT thermal/power limit. Raising the skin-temp or min-power limit lets the APU sustain more power (more performance, hotter chassis); default protects surface temperature and safety. |

### APTS State Index 9   (Form 0x78)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| Fan Table Index | Numeric | Min 0x0 · Max 0xFFFFFFFF · Step 0x1 · Default 0x0 ◀ default ◀ performance TUNE Fan-curve point. For sustained performance, bias the curve toward more airflow (lower start temp / higher PWM) to gain thermal headroom; trade against noise. |
| PPT Limit for PMF power throttler | Numeric | Min 0x0 · Max 0xFFFFFFFF · Step 0x1 · Default 0x0 ◀ default ◀ performance NEUTRAL PMF policy/firmware-binary plumbing — the OEM ships validated policy; no user performance setting here. Leave default. |
| PPT Limit for PMF APU Only power throttler | Numeric | Min 0x0 · Max 0xFFFFFFFF · Step 0x1 · Default 0x0 ◀ default ◀ performance NEUTRAL PMF policy/firmware-binary plumbing — the OEM ships validated policy; no user performance setting here. Leave default. |
| STT Min Power Limit in milliwatts | Numeric | Min 0x0 · Max 0xFFFFFFFF · Step 0x1 · Default 0x0 ◀ default ◀ performance TUNE STT thermal/power limit. Raising the skin-temp or min-power limit lets the APU sustain more power (more performance, hotter chassis); default protects surface temperature and safety. |
| STT Skin Temp Limit APU in Celsius | Numeric | Min 0x0 · Max 0xFF · Step 0x1 · Default 0x0 ◀ default ◀ performance TUNE STT thermal/power limit. Raising the skin-temp or min-power limit lets the APU sustain more power (more performance, hotter chassis); default protects surface temperature and safety. |
| STT Skin Temp Limit HS2 in Celsius | Numeric | Min 0x0 · Max 0xFF · Step 0x1 · Default 0x0 ◀ default ◀ performance TUNE STT thermal/power limit. Raising the skin-temp or min-power limit lets the APU sustain more power (more performance, hotter chassis); default protects surface temperature and safety. |

### APTS State Index 10   (Form 0x79)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| Fan Table Index | Numeric | Min 0x0 · Max 0xFFFFFFFF · Step 0x1 · Default 0x0 ◀ default ◀ performance TUNE Fan-curve point. For sustained performance, bias the curve toward more airflow (lower start temp / higher PWM) to gain thermal headroom; trade against noise. |
| PPT Limit for PMF power throttler | Numeric | Min 0x0 · Max 0xFFFFFFFF · Step 0x1 · Default 0x0 ◀ default ◀ performance NEUTRAL PMF policy/firmware-binary plumbing — the OEM ships validated policy; no user performance setting here. Leave default. |
| PPT Limit for PMF APU Only power throttler | Numeric | Min 0x0 · Max 0xFFFFFFFF · Step 0x1 · Default 0x0 ◀ default ◀ performance NEUTRAL PMF policy/firmware-binary plumbing — the OEM ships validated policy; no user performance setting here. Leave default. |
| STT Min Power Limit in milliwatts | Numeric | Min 0x0 · Max 0xFFFFFFFF · Step 0x1 · Default 0x0 ◀ default ◀ performance TUNE STT thermal/power limit. Raising the skin-temp or min-power limit lets the APU sustain more power (more performance, hotter chassis); default protects surface temperature and safety. |
| STT Skin Temp Limit APU in Celsius | Numeric | Min 0x0 · Max 0xFF · Step 0x1 · Default 0x0 ◀ default ◀ performance TUNE STT thermal/power limit. Raising the skin-temp or min-power limit lets the APU sustain more power (more performance, hotter chassis); default protects surface temperature and safety. |
| STT Skin Temp Limit HS2 in Celsius | Numeric | Min 0x0 · Max 0xFF · Step 0x1 · Default 0x0 ◀ default ◀ performance TUNE STT thermal/power limit. Raising the skin-temp or min-power limit lets the APU sustain more power (more performance, hotter chassis); default protects surface temperature and safety. |

### APTS State Index 11   (Form 0x7A)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| Fan Table Index | Numeric | Min 0x0 · Max 0xFFFFFFFF · Step 0x1 · Default 0x0 ◀ default ◀ performance TUNE Fan-curve point. For sustained performance, bias the curve toward more airflow (lower start temp / higher PWM) to gain thermal headroom; trade against noise. |
| PPT Limit for PMF power throttler | Numeric | Min 0x0 · Max 0xFFFFFFFF · Step 0x1 · Default 0x0 ◀ default ◀ performance NEUTRAL PMF policy/firmware-binary plumbing — the OEM ships validated policy; no user performance setting here. Leave default. |
| PPT Limit for PMF APU Only power throttler | Numeric | Min 0x0 · Max 0xFFFFFFFF · Step 0x1 · Default 0x0 ◀ default ◀ performance NEUTRAL PMF policy/firmware-binary plumbing — the OEM ships validated policy; no user performance setting here. Leave default. |
| STT Min Power Limit in milliwatts | Numeric | Min 0x0 · Max 0xFFFFFFFF · Step 0x1 · Default 0x0 ◀ default ◀ performance TUNE STT thermal/power limit. Raising the skin-temp or min-power limit lets the APU sustain more power (more performance, hotter chassis); default protects surface temperature and safety. |
| STT Skin Temp Limit APU in Celsius | Numeric | Min 0x0 · Max 0xFF · Step 0x1 · Default 0x0 ◀ default ◀ performance TUNE STT thermal/power limit. Raising the skin-temp or min-power limit lets the APU sustain more power (more performance, hotter chassis); default protects surface temperature and safety. |
| STT Skin Temp Limit HS2 in Celsius | Numeric | Min 0x0 · Max 0xFF · Step 0x1 · Default 0x0 ◀ default ◀ performance TUNE STT thermal/power limit. Raising the skin-temp or min-power limit lets the APU sustain more power (more performance, hotter chassis); default protects surface temperature and safety. |

### APTS State Index 12   (Form 0x7B)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| Fan Table Index | Numeric | Min 0x0 · Max 0xFFFFFFFF · Step 0x1 · Default 0x0 ◀ default ◀ performance TUNE Fan-curve point. For sustained performance, bias the curve toward more airflow (lower start temp / higher PWM) to gain thermal headroom; trade against noise. |
| PPT Limit for PMF power throttler | Numeric | Min 0x0 · Max 0xFFFFFFFF · Step 0x1 · Default 0x0 ◀ default ◀ performance NEUTRAL PMF policy/firmware-binary plumbing — the OEM ships validated policy; no user performance setting here. Leave default. |
| PPT Limit for PMF APU Only power throttler | Numeric | Min 0x0 · Max 0xFFFFFFFF · Step 0x1 · Default 0x0 ◀ default ◀ performance NEUTRAL PMF policy/firmware-binary plumbing — the OEM ships validated policy; no user performance setting here. Leave default. |
| STT Min Power Limit in milliwatts | Numeric | Min 0x0 · Max 0xFFFFFFFF · Step 0x1 · Default 0x0 ◀ default ◀ performance TUNE STT thermal/power limit. Raising the skin-temp or min-power limit lets the APU sustain more power (more performance, hotter chassis); default protects surface temperature and safety. |
| STT Skin Temp Limit APU in Celsius | Numeric | Min 0x0 · Max 0xFF · Step 0x1 · Default 0x0 ◀ default ◀ performance TUNE STT thermal/power limit. Raising the skin-temp or min-power limit lets the APU sustain more power (more performance, hotter chassis); default protects surface temperature and safety. |
| STT Skin Temp Limit HS2 in Celsius | Numeric | Min 0x0 · Max 0xFF · Step 0x1 · Default 0x0 ◀ default ◀ performance TUNE STT thermal/power limit. Raising the skin-temp or min-power limit lets the APU sustain more power (more performance, hotter chassis); default protects surface temperature and safety. |

### APTS State Index 13   (Form 0x7C)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| Fan Table Index | Numeric | Min 0x0 · Max 0xFFFFFFFF · Step 0x1 · Default 0x0 ◀ default ◀ performance TUNE Fan-curve point. For sustained performance, bias the curve toward more airflow (lower start temp / higher PWM) to gain thermal headroom; trade against noise. |
| PPT Limit for PMF power throttler | Numeric | Min 0x0 · Max 0xFFFFFFFF · Step 0x1 · Default 0x0 ◀ default ◀ performance NEUTRAL PMF policy/firmware-binary plumbing — the OEM ships validated policy; no user performance setting here. Leave default. |
| PPT Limit for PMF APU Only power throttler | Numeric | Min 0x0 · Max 0xFFFFFFFF · Step 0x1 · Default 0x0 ◀ default ◀ performance NEUTRAL PMF policy/firmware-binary plumbing — the OEM ships validated policy; no user performance setting here. Leave default. |
| STT Min Power Limit in milliwatts | Numeric | Min 0x0 · Max 0xFFFFFFFF · Step 0x1 · Default 0x0 ◀ default ◀ performance TUNE STT thermal/power limit. Raising the skin-temp or min-power limit lets the APU sustain more power (more performance, hotter chassis); default protects surface temperature and safety. |
| STT Skin Temp Limit APU in Celsius | Numeric | Min 0x0 · Max 0xFF · Step 0x1 · Default 0x0 ◀ default ◀ performance TUNE STT thermal/power limit. Raising the skin-temp or min-power limit lets the APU sustain more power (more performance, hotter chassis); default protects surface temperature and safety. |
| STT Skin Temp Limit HS2 in Celsius | Numeric | Min 0x0 · Max 0xFF · Step 0x1 · Default 0x0 ◀ default ◀ performance TUNE STT thermal/power limit. Raising the skin-temp or min-power limit lets the APU sustain more power (more performance, hotter chassis); default protects surface temperature and safety. |

### APTS State Index 14   (Form 0x7D)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| Fan Table Index | Numeric | Min 0x0 · Max 0xFFFFFFFF · Step 0x1 · Default 0x0 ◀ default ◀ performance TUNE Fan-curve point. For sustained performance, bias the curve toward more airflow (lower start temp / higher PWM) to gain thermal headroom; trade against noise. |
| PPT Limit for PMF power throttler | Numeric | Min 0x0 · Max 0xFFFFFFFF · Step 0x1 · Default 0x0 ◀ default ◀ performance NEUTRAL PMF policy/firmware-binary plumbing — the OEM ships validated policy; no user performance setting here. Leave default. |
| PPT Limit for PMF APU Only power throttler | Numeric | Min 0x0 · Max 0xFFFFFFFF · Step 0x1 · Default 0x0 ◀ default ◀ performance NEUTRAL PMF policy/firmware-binary plumbing — the OEM ships validated policy; no user performance setting here. Leave default. |
| STT Min Power Limit in milliwatts | Numeric | Min 0x0 · Max 0xFFFFFFFF · Step 0x1 · Default 0x0 ◀ default ◀ performance TUNE STT thermal/power limit. Raising the skin-temp or min-power limit lets the APU sustain more power (more performance, hotter chassis); default protects surface temperature and safety. |
| STT Skin Temp Limit APU in Celsius | Numeric | Min 0x0 · Max 0xFF · Step 0x1 · Default 0x0 ◀ default ◀ performance TUNE STT thermal/power limit. Raising the skin-temp or min-power limit lets the APU sustain more power (more performance, hotter chassis); default protects surface temperature and safety. |
| STT Skin Temp Limit HS2 in Celsius | Numeric | Min 0x0 · Max 0xFF · Step 0x1 · Default 0x0 ◀ default ◀ performance TUNE STT thermal/power limit. Raising the skin-temp or min-power limit lets the APU sustain more power (more performance, hotter chassis); default protects surface temperature and safety. |

### APTS State Index 15   (Form 0x7E)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| Fan Table Index | Numeric | Min 0x0 · Max 0xFFFFFFFF · Step 0x1 · Default 0x0 ◀ default ◀ performance TUNE Fan-curve point. For sustained performance, bias the curve toward more airflow (lower start temp / higher PWM) to gain thermal headroom; trade against noise. |
| PPT Limit for PMF power throttler | Numeric | Min 0x0 · Max 0xFFFFFFFF · Step 0x1 · Default 0x0 ◀ default ◀ performance NEUTRAL PMF policy/firmware-binary plumbing — the OEM ships validated policy; no user performance setting here. Leave default. |
| PPT Limit for PMF APU Only power throttler | Numeric | Min 0x0 · Max 0xFFFFFFFF · Step 0x1 · Default 0x0 ◀ default ◀ performance NEUTRAL PMF policy/firmware-binary plumbing — the OEM ships validated policy; no user performance setting here. Leave default. |
| STT Min Power Limit in milliwatts | Numeric | Min 0x0 · Max 0xFFFFFFFF · Step 0x1 · Default 0x0 ◀ default ◀ performance TUNE STT thermal/power limit. Raising the skin-temp or min-power limit lets the APU sustain more power (more performance, hotter chassis); default protects surface temperature and safety. |
| STT Skin Temp Limit APU in Celsius | Numeric | Min 0x0 · Max 0xFF · Step 0x1 · Default 0x0 ◀ default ◀ performance TUNE STT thermal/power limit. Raising the skin-temp or min-power limit lets the APU sustain more power (more performance, hotter chassis); default protects surface temperature and safety. |
| STT Skin Temp Limit HS2 in Celsius | Numeric | Min 0x0 · Max 0xFF · Step 0x1 · Default 0x0 ◀ default ◀ performance TUNE STT thermal/power limit. Raising the skin-temp or min-power limit lets the APU sustain more power (more performance, hotter chassis); default protects surface temperature and safety. |

# DASH / ASF Management

DASH / ASF Management — the Dash Function submenu reached via Advanced ‣ Dash Function. Configures the DASH/ASF out-of-band management agent and identifies the LAN controller (bus / device / function) it binds to.

Form-set GUID: 64C1CC5F-ED4C-4042-9455-D3859B2F78D8     ·     Forms: 1     ·     Configurable settings: 8

### Dash Function   (Form 0x34)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| Dash Function Support | Selection | Disabled [0x0] ◀ default Enabled [0x1] ◀ performance NEUTRAL Out-of-band management (DASH/ASF) — no performance impact; enable only if you use remote management. |
| Disable Multi-Function | Selection | Disabled [0x0] Enabled [0x1] ◀ default ◀ performance NEUTRAL Out-of-band management (DASH/ASF) — no performance impact; enable only if you use remote management. |
| Bus number Of LAN Controller | Numeric | Min 0x0 · Max 0xFF · Default 0x0 ◀ default ◀ performance NEUTRAL Out-of-band management (DASH/ASF) — no performance impact; enable only if you use remote management. |
| Dev number Of LAN Controller | Numeric | Min 0x0 · Max 0x1F · Default 0x0 ◀ default ◀ performance NEUTRAL Out-of-band management (DASH/ASF) — no performance impact; enable only if you use remote management. |
| Function number Of LAN Controller | Numeric | Min 0x0 · Max 0x7 · Default 0x0 ◀ default ◀ performance NEUTRAL Out-of-band management (DASH/ASF) — no performance impact; enable only if you use remote management. |
| Asf Message Function Support | Selection | Disabled [0x0] ◀ default Enabled [0x1] ◀ performance NEUTRAL Out-of-band management (DASH/ASF) — no performance impact; enable only if you use remote management. |
| Asf BIOS WatchDog Message Time Out Value | Numeric | Min 0x0 · Max 0xFFFF · Default 0x0 ◀ default ◀ performance NEUTRAL Out-of-band management (DASH/ASF) — no performance impact; enable only if you use remote management. |
| Asf OS WatchDog Message Time Out Value | Numeric | Min 0x0 · Max 0xFFFF · Default 0x0 ◀ default ◀ performance NEUTRAL Out-of-band management (DASH/ASF) — no performance impact; enable only if you use remote management. |

# RAIDXpert2 Configuration Utility

RAIDXpert2 Configuration Utility — the AMD storage RAID manager surfaced under Advanced when SATA/NVMe RAID mode is enabled. Provides controller management, array create/manage/delete, physical-disk management, and hot-spare assignment. Many entries are populated dynamically from attached drives and arrays, so option lists shown here are the static templates rather than live device names.

Form-set GUID: AA5D54A2-59C8-458B-80D8-E41944AD1D99     ·     Forms: 29     ·     Configurable settings: 37

### Configuration Options   (Form 0x1)

Submenus: Controller Management  ‣  Array Management  ‣  Physical Disk Management

### Controller Management   (Form 0x100)

Submenus: View Controller Information  ‣  Rescan Disks

### View Controller Information   (Form 0x110)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| Select Controller: | Selection | 32 entries, populated at runtime from detected devices Default selection: index 0x1 ◀ performance NEUTRAL RAID array configuration — performance depends on RAID level and member SSDs, not a single setting. RAID0 maximizes throughput; single-NVMe is simplest. No generic default change. |

### Change Controller Properties   (Form 0x120)

Submenus: Apply Changes

### Convert to Legacy Disk   (Form 0x130)

Submenus: OK  ‣  Cancel  ‣  Cancel

### Convert to Legacy Disk   (Form 0x131)

Submenus: Yes  ‣  Yes  ‣  Yes  ‣  Yes  ‣  No

### Initialize Disk(s)   (Form 0x140)

Submenus: OK  ‣  Cancel  ‣  Cancel

### Initialize Disk(s)   (Form 0x141)

Submenus: Yes  ‣  Yes  ‣  Yes  ‣  Yes  ‣  No

### Rescan Disks   (Form 0x150)

Submenus: OK

### Update Controller Firmware   (Form 0x160)

Submenus: Select Firmware File  ‣  Proceed with Firmware Download  ‣  Cancel Firmware Download  ‣  Activate Firmware  ‣  Cancel Firmware Activation

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| Firmware file extension | Selection | *.* [0x0] ◀ default *.fluf [0x1] *.fw [0x2] *.bin [0x3] *.raw [0x4] ◀ performance NEUTRAL RAID array configuration — performance depends on RAID level and member SSDs, not a single setting. RAID0 maximizes throughput; single-NVMe is simplest. No generic default change. |

### Array Management   (Form 0x200)

Submenus: Create Array  ‣  Manage Array Properties  ‣  Delete Array

### Create Array   (Form 0x210)

Submenus: Create Array  ‣  Create Array  ‣  Select Physical Disks  ‣  Select Physical Disks  ‣  Create Array  ‣  Create Array

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| Select RAID Level: | Selection | Volume [0x8] ◀ default RAIDABLE [0xD] RAID 0 [0x1] RAID 1 [0x2] RAID 5 [0x3] RAID 6 [0xB] RAID 10 [0x4] ◀ performance NEUTRAL RAID array configuration — performance depends on RAID level and member SSDs, not a single setting. RAID0 maximizes throughput; single-NVMe is simplest. No generic default change. |
| Array Size: | Numeric | Min 0x0 · Max 0xFFFFFFFFFFFFFFFF ◀ performance NEUTRAL RAID array configuration — performance depends on RAID level and member SSDs, not a single setting. RAID0 maximizes throughput; single-NVMe is simplest. No generic default change. |
| Array Size Unit: | Selection | MB (MegaBytes) [0x2] ◀ performance NEUTRAL RAID array configuration — performance depends on RAID level and member SSDs, not a single setting. RAID0 maximizes throughput; single-NVMe is simplest. No generic default change. |
| Select CacheTagSize: | Selection | (populated at runtime) [0x0] ◀ default 64KB [0x1] 128KB [0x2] 256KB [0x3] ◀ performance NEUTRAL RAID array configuration — performance depends on RAID level and member SSDs, not a single setting. RAID0 maximizes throughput; single-NVMe is simplest. No generic default change. |
| Select CacheTagSize: | Selection | 64KB [0x1] ◀ default 128KB [0x2] 256KB [0x3] ◀ performance NEUTRAL RAID array configuration — performance depends on RAID level and member SSDs, not a single setting. RAID0 maximizes throughput; single-NVMe is simplest. No generic default change. |
| Read Cache Policy: | Selection | Read Cache [0x2] ◀ default No Cache [0x1] ◀ performance NEUTRAL RAID array configuration — performance depends on RAID level and member SSDs, not a single setting. RAID0 maximizes throughput; single-NVMe is simplest. No generic default change. |
| Write Cache Policy: | Selection | Write Back Cache [0x4] ◀ default No Cache [0x3] ◀ performance NEUTRAL RAID array configuration — performance depends on RAID level and member SSDs, not a single setting. RAID0 maximizes throughput; single-NVMe is simplest. No generic default change. |

### Select Physical Disks   (Form 0x212)

Submenus: Apply Changes  ‣  Apply Changes  ‣  Apply Changes  ‣  Apply Changes

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| Select Media Type: | Selection | HDD [0x1] ◀ default SSD [0x2] BOTH [0x3] ◀ performance NEUTRAL RAID array configuration — performance depends on RAID level and member SSDs, not a single setting. RAID0 maximizes throughput; single-NVMe is simplest. No generic default change. |

### Select Physical Disks   (Form 0x211)

Submenus: Apply Changes  ‣  Apply Changes

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| Select Media Type: | Selection | HDD [0x1] ◀ default SSD [0x2] BOTH [0x3] ◀ performance NEUTRAL RAID array configuration — performance depends on RAID level and member SSDs, not a single setting. RAID0 maximizes throughput; single-NVMe is simplest. No generic default change. |

### Select Physical Disks   (Form 0x213)

Submenus: OK  ‣  OK

### Select Physical Disks   (Form 0x214)

Submenus: Yes  ‣  No  ‣  No

### Create Array   (Form 0x215)

Submenus: YES  ‣  NO

### Manage Array Properties   (Form 0x220)

Submenus: View Associated Physical Disks  ‣  Manage Dedicated Hot Spares

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| Select Array: | Selection | VIRT 0 0123456789012345678901234567890123456789012345678901234567890 [0x1] ◀ default VIRT 1 0123456789012345678901234567890123456789012345678901234567890 [0x2] VIRT 2 0123456789012345678901234567890123456789012345678901234567890 [0x3] VIRT 3 0123456789012345678901234567890123456789012345678901234567890 [0x4] VIRT 4 0123456789012345678901234567890123456789012345678901234567890 [0x5] VIRT 5 0123456789012345678901234567890123456789012345678901234567890 [0x6] VIRT 6 0123456789012345678901234567890123456789012345678901234567890 [0x7] VIRT 7 0123456789012345678901234567890123456789012345678901234567890 [0x8] VIRT 8 0123456789012345678901234567890123456789012345678901234567890 [0x9] VIRT 9 0123456789012345678901234567890123456789012345678901234567890 [0xA] VIRT 10 0123456789012345678901234567890123456789012345678901234567890 [0xB] VIRT 11 0123456789012345678901234567890123456789012345678901234567890 [0xC] VIRT 12 0123456789012345678901234567890123456789012345678901234567890 [0xD] VIRT 13 0123456789012345678901234567890123456789012345678901234567890 [0xE] VIRT 14 0123456789012345678901234567890123456789012345678901234567890 [0xF] VIRT 15 0123456789012345678901234567890123456789012345678901234567890 [0x10] VIRT 16 0123456789012345678901234567890123456789012345678901234567890 [0x11] VIRT 17 0123456789012345678901234567890123456789012345678901234567890 [0x12] VIRT 18 0123456789012345678901234567890123456789012345678901234567890 [0x13] VIRT 19 0123456789012345678901234567890123456789012345678901234567890 [0x14] VIRT 20 0123456789012345678901234567890123456789012345678901234567890 [0x15] VIRT 21 0123456789012345678901234567890123456789012345678901234567890 [0x16] VIRT 22 0123456789012345678901234567890123456789012345678901234567890 [0x17] VIRT 23 0123456789012345678901234567890123456789012345678901234567890 [0x18] VIRT 24 0123456789012345678901234567890123456789012345678901234567890 [0x19] VIRT 25 0123456789012345678901234567890123456789012345678901234567890 [0x1A] VIRT 26 0123456789012345678901234567890123456789012345678901234567890 [0x1B] VIRT 27 0123456789012345678901234567890123456789012345678901234567890 [0x1C] VIRT 28 0123456789012345678901234567890123456789012345678901234567890 [0x1D] VIRT 29 0123456789012345678901234567890123456789012345678901234567890 [0x1E] VIRT 30 0123456789012345678901234567890123456789012345678901234567890 [0x1F] VIRT 31 0123456789012345678901234567890123456789012345678901234567890 [0x20] ◀ performance NEUTRAL RAID array configuration — performance depends on RAID level and member SSDs, not a single setting. RAID0 maximizes throughput; single-NVMe is simplest. No generic default change. |

### View Associated Physical Disks   (Form 0x222)

Submenus: View Physical Disk Properties

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| Array ID: | Text | Text input · length 0x1–0x40 ◀ performance NEUTRAL RAID array configuration — performance depends on RAID level and member SSDs, not a single setting. RAID0 maximizes throughput; single-NVMe is simplest. No generic default change. |
| RAID Level: | Text | Text input · length 0x1–0x40 ◀ performance NEUTRAL RAID array configuration — performance depends on RAID level and member SSDs, not a single setting. RAID0 maximizes throughput; single-NVMe is simplest. No generic default change. |
| Array Status: | Text | Text input · length 0x1–0x40 ◀ performance NEUTRAL RAID array configuration — performance depends on RAID level and member SSDs, not a single setting. RAID0 maximizes throughput; single-NVMe is simplest. No generic default change. |
| Array Capacity: | Text | Text input · length 0x1–0x40 ◀ performance NEUTRAL RAID array configuration — performance depends on RAID level and member SSDs, not a single setting. RAID0 maximizes throughput; single-NVMe is simplest. No generic default change. |
| Cache Tag Size: | Text | Text input · length 0x1–0x40 ◀ performance NEUTRAL RAID array configuration — performance depends on RAID level and member SSDs, not a single setting. RAID0 maximizes throughput; single-NVMe is simplest. No generic default change. |
| Hidden: | Text | Text input · length 0x1–0x40 ◀ performance NEUTRAL Informational / non-configurable field — no performance action. |
| Read Cache Policy: | Text | Text input · length 0x1–0x40 ◀ performance NEUTRAL RAID array configuration — performance depends on RAID level and member SSDs, not a single setting. RAID0 maximizes throughput; single-NVMe is simplest. No generic default change. |
| Write Cache Policy: | Text | Text input · length 0x1–0x40 ◀ performance NEUTRAL RAID array configuration — performance depends on RAID level and member SSDs, not a single setting. RAID0 maximizes throughput; single-NVMe is simplest. No generic default change. |
| Array ID: | Text | Text input · length 0x1–0x40 ◀ performance NEUTRAL RAID array configuration — performance depends on RAID level and member SSDs, not a single setting. RAID0 maximizes throughput; single-NVMe is simplest. No generic default change. |
| RAID Level: | Text | Text input · length 0x1–0x40 ◀ performance NEUTRAL RAID array configuration — performance depends on RAID level and member SSDs, not a single setting. RAID0 maximizes throughput; single-NVMe is simplest. No generic default change. |
| Array Status: | Text | Text input · length 0x1–0x40 ◀ performance NEUTRAL RAID array configuration — performance depends on RAID level and member SSDs, not a single setting. RAID0 maximizes throughput; single-NVMe is simplest. No generic default change. |
| Array Capacity: | Text | Text input · length 0x1–0x40 ◀ performance NEUTRAL RAID array configuration — performance depends on RAID level and member SSDs, not a single setting. RAID0 maximizes throughput; single-NVMe is simplest. No generic default change. |
| Cache Tag Size: | Text | Text input · length 0x1–0x40 ◀ performance NEUTRAL RAID array configuration — performance depends on RAID level and member SSDs, not a single setting. RAID0 maximizes throughput; single-NVMe is simplest. No generic default change. |
| Hidden: | Text | Text input · length 0x1–0x40 ◀ performance NEUTRAL Informational / non-configurable field — no performance action. |
| Read Cache Policy: | Text | Text input · length 0x1–0x40 ◀ performance NEUTRAL RAID array configuration — performance depends on RAID level and member SSDs, not a single setting. RAID0 maximizes throughput; single-NVMe is simplest. No generic default change. |
| Write Cache Policy: | Text | Text input · length 0x1–0x40 ◀ performance NEUTRAL RAID array configuration — performance depends on RAID level and member SSDs, not a single setting. RAID0 maximizes throughput; single-NVMe is simplest. No generic default change. |

### Manage Dedicated Hot Spares   (Form 0x224)

Submenus: Add Hot Spare Disk

### Manage Dedicated Hot Spares   (Form 0x225)

Submenus: YES  ‣  NO

### Delete Array   (Form 0x230)

Submenus: Delete Array(s)

### Warning   (Form 0x231)

Submenus: YES  ‣  NO

### View Array Properties.   (Form 0x240)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| Select Array: | Selection | VIRT 0 0123456789012345678901234567890123456789012345678901234567890 [0x1] ◀ default VIRT 1 0123456789012345678901234567890123456789012345678901234567890 [0x2] VIRT 2 0123456789012345678901234567890123456789012345678901234567890 [0x3] VIRT 3 0123456789012345678901234567890123456789012345678901234567890 [0x4] VIRT 4 0123456789012345678901234567890123456789012345678901234567890 [0x5] VIRT 5 0123456789012345678901234567890123456789012345678901234567890 [0x6] VIRT 6 0123456789012345678901234567890123456789012345678901234567890 [0x7] VIRT 7 0123456789012345678901234567890123456789012345678901234567890 [0x8] VIRT 8 0123456789012345678901234567890123456789012345678901234567890 [0x9] VIRT 9 0123456789012345678901234567890123456789012345678901234567890 [0xA] VIRT 10 0123456789012345678901234567890123456789012345678901234567890 [0xB] VIRT 11 0123456789012345678901234567890123456789012345678901234567890 [0xC] VIRT 12 0123456789012345678901234567890123456789012345678901234567890 [0xD] VIRT 13 0123456789012345678901234567890123456789012345678901234567890 [0xE] VIRT 14 0123456789012345678901234567890123456789012345678901234567890 [0xF] VIRT 15 0123456789012345678901234567890123456789012345678901234567890 [0x10] VIRT 16 0123456789012345678901234567890123456789012345678901234567890 [0x11] VIRT 17 0123456789012345678901234567890123456789012345678901234567890 [0x12] VIRT 18 0123456789012345678901234567890123456789012345678901234567890 [0x13] VIRT 19 0123456789012345678901234567890123456789012345678901234567890 [0x14] VIRT 20 0123456789012345678901234567890123456789012345678901234567890 [0x15] VIRT 21 0123456789012345678901234567890123456789012345678901234567890 [0x16] VIRT 22 0123456789012345678901234567890123456789012345678901234567890 [0x17] VIRT 23 0123456789012345678901234567890123456789012345678901234567890 [0x18] VIRT 24 0123456789012345678901234567890123456789012345678901234567890 [0x19] VIRT 25 0123456789012345678901234567890123456789012345678901234567890 [0x1A] VIRT 26 0123456789012345678901234567890123456789012345678901234567890 [0x1B] VIRT 27 0123456789012345678901234567890123456789012345678901234567890 [0x1C] VIRT 28 0123456789012345678901234567890123456789012345678901234567890 [0x1D] VIRT 29 0123456789012345678901234567890123456789012345678901234567890 [0x1E] VIRT 30 0123456789012345678901234567890123456789012345678901234567890 [0x1F] VIRT 31 0123456789012345678901234567890123456789012345678901234567890 [0x20] ◀ performance NEUTRAL RAID array configuration — performance depends on RAID level and member SSDs, not a single setting. RAID0 maximizes throughput; single-NVMe is simplest. No generic default change. |
| Array ID: | Text | Text input · length 0x1–0x40 ◀ performance NEUTRAL RAID array configuration — performance depends on RAID level and member SSDs, not a single setting. RAID0 maximizes throughput; single-NVMe is simplest. No generic default change. |
| RAID Level: | Text | Text input · length 0x1–0x40 ◀ performance NEUTRAL RAID array configuration — performance depends on RAID level and member SSDs, not a single setting. RAID0 maximizes throughput; single-NVMe is simplest. No generic default change. |
| Array Status: | Text | Text input · length 0x1–0x40 ◀ performance NEUTRAL RAID array configuration — performance depends on RAID level and member SSDs, not a single setting. RAID0 maximizes throughput; single-NVMe is simplest. No generic default change. |
| Array Capacity: | Text | Text input · length 0x1–0x40 ◀ performance NEUTRAL RAID array configuration — performance depends on RAID level and member SSDs, not a single setting. RAID0 maximizes throughput; single-NVMe is simplest. No generic default change. |
| Read Cache Policy: | Text | Text input · length 0x1–0x40 ◀ performance NEUTRAL RAID array configuration — performance depends on RAID level and member SSDs, not a single setting. RAID0 maximizes throughput; single-NVMe is simplest. No generic default change. |
| Write Cache Policy: | Text | Text input · length 0x1–0x40 ◀ performance NEUTRAL RAID array configuration — performance depends on RAID level and member SSDs, not a single setting. RAID0 maximizes throughput; single-NVMe is simplest. No generic default change. |

### Physical Disk Management   (Form 0x300)

Submenus: View Physical Disk Properties  ‣  Select Physical Disk Operations  ‣  View Global Hot Spares

### View Physical Disk Properties   (Form 0x310)

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| Select Physical Disk: | Selection | Physical Disk 0: XX: SATA, Size, Ready [0x1] ◀ default Physical Disk 1: XX: SATA, Size, Ready [0x2] Physical Disk 2: XX: SATA, Size, Ready [0x3] Physical Disk 3: XX: SATA, Size, Ready [0x4] Physical Disk 4: XX: SATA, Size, Ready [0x5] Physical Disk 5: XX: SATA, Size, Ready [0x6] Physical Disk 6: XX: SATA, Size, Ready [0x7] Physical Disk 7: XX: SATA, Size, Ready [0x8] Physical Disk 8: XX: SATA, Size, Ready [0x9] Physical Disk 9: XX: SATA, Size, Ready [0xA] Physical Disk 10: XX: SATA, Size, Ready [0xB] Physical Disk 11: XX: SATA, Size, Ready [0xC] Physical Disk 12: XX: SATA, Size, Ready [0xD] Physical Disk 13: XX: SATA, Size, Ready [0xE] Physical Disk 14: XX: SATA, Size, Ready [0xF] Physical Disk 15: XX: SATA, Size, Ready [0x10] Physical Disk 16: XX: SATA, Size, Ready [0x11] Physical Disk 17: XX: SATA, Size, Ready [0x12] Physical Disk 18: XX: SATA, Size, Ready [0x13] Physical Disk 19: XX: SATA, Size, Ready [0x14] Physical Disk 20: XX: SATA, Size, Ready [0x15] Physical Disk 21: XX: SATA, Size, Ready [0x16] Physical Disk 22: XX: SATA, Size, Ready [0x17] Physical Disk 23: XX: SATA, Size, Ready [0x18] Physical Disk 24: XX: SATA, Size, Ready [0x19] Physical Disk 25: XX: SATA, Size, Ready [0x1A] Physical Disk 26: XX: SATA, Size, Ready [0x1B] Physical Disk 27: XX: SATA, Size, Ready [0x1C] Physical Disk 28: XX: SATA, Size, Ready [0x1D] Physical Disk 29: XX: SATA, Size, Ready [0x1E] Physical Disk 30: XX: SATA, Size, Ready [0x1F] Physical Disk 31: XX: SATA, Size, Ready [0x20] ◀ performance NEUTRAL RAID array configuration — performance depends on RAID level and member SSDs, not a single setting. RAID0 maximizes throughput; single-NVMe is simplest. No generic default change. |

### Select Physical Disk Operations   (Form 0x320)

Submenus: Initialize Disk  ‣  Assign Global Hot Spare  ‣  Assign Global Hot Spare  ‣  Unassign Hot Spare

| Setting | Type | Options / Values / Range |
| --- | --- | --- |
| Select Physical Disk: | Selection | Physical Disk 0: XX: SATA, Size, Ready [0x1] ◀ default Physical Disk 1: XX: SATA, Size, Ready [0x2] Physical Disk 2: XX: SATA, Size, Ready [0x3] Physical Disk 3: XX: SATA, Size, Ready [0x4] Physical Disk 4: XX: SATA, Size, Ready [0x5] Physical Disk 5: XX: SATA, Size, Ready [0x6] Physical Disk 6: XX: SATA, Size, Ready [0x7] Physical Disk 7: XX: SATA, Size, Ready [0x8] Physical Disk 8: XX: SATA, Size, Ready [0x9] Physical Disk 9: XX: SATA, Size, Ready [0xA] Physical Disk 10: XX: SATA, Size, Ready [0xB] Physical Disk 11: XX: SATA, Size, Ready [0xC] Physical Disk 12: XX: SATA, Size, Ready [0xD] Physical Disk 13: XX: SATA, Size, Ready [0xE] Physical Disk 14: XX: SATA, Size, Ready [0xF] Physical Disk 15: XX: SATA, Size, Ready [0x10] Physical Disk 16: XX: SATA, Size, Ready [0x11] Physical Disk 17: XX: SATA, Size, Ready [0x12] Physical Disk 18: XX: SATA, Size, Ready [0x13] Physical Disk 19: XX: SATA, Size, Ready [0x14] Physical Disk 20: XX: SATA, Size, Ready [0x15] Physical Disk 21: XX: SATA, Size, Ready [0x16] Physical Disk 22: XX: SATA, Size, Ready [0x17] Physical Disk 23: XX: SATA, Size, Ready [0x18] Physical Disk 24: XX: SATA, Size, Ready [0x19] Physical Disk 25: XX: SATA, Size, Ready [0x1A] Physical Disk 26: XX: SATA, Size, Ready [0x1B] Physical Disk 27: XX: SATA, Size, Ready [0x1C] Physical Disk 28: XX: SATA, Size, Ready [0x1D] Physical Disk 29: XX: SATA, Size, Ready [0x1E] Physical Disk 30: XX: SATA, Size, Ready [0x1F] Physical Disk 31: XX: SATA, Size, Ready [0x20] ◀ performance NEUTRAL RAID array configuration — performance depends on RAID level and member SSDs, not a single setting. RAID0 maximizes throughput; single-NVMe is simplest. No generic default change. |

### Select Physical Disk Operations   (Form 0x321)

Submenus: Yes  ‣  No

### Select Physical Disk Operations   (Form 0x322)

Submenus: Yes  ‣  No

# Appendix A — Dynamic Driver Forms (Excluded)

The following HII forms exist in the firmware but are provided by generic UEFI network and boot drivers rather than the board BIOS. They are surfaced only when the relevant controller or driver binds at runtime (for example, when a NIC is detected under Network Stack), and their contents are standardized by the UEFI driver model rather than specific to this platform. They are listed here for completeness and are not enumerated option-by-option.

| Form / Page | Purpose | Provider Module |
| --- | --- | --- |
| Network Stack Configuration | IPv4 / IPv6 network device policy | NetworkStackSetupScreen |
| IPv4 Network Configuration | Static IP, netmask, gateway, DNS (per NIC) | Ip4Dxe |
| IPv6 Network Configuration | IPv6 interface policy and addressing (per NIC) | Ip6Dxe |
| VLAN Configuration | VLAN ID and priority (per NIC) | VlanConfigDxe |
| HTTP Boot Configuration | UEFI HTTP/HTTPS boot URI and parameters | HttpBootDxe |
| TLS Auth Configuration | Server CA / client certificate enrollment | TlsAuthConfigDxe |
| PXE / Network Boot | CRB PXE UEFI boot driver options | CrbPxeUefiDriver |